| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to Femto ABS initialization(15.4.5) |  |
| Date Submitted | 2009-11-16 |  |
| Source(s) | Kanchei (Ken) Loa, Chun-Yen Hsu Institute for Information Industry | loa@iii.org.tw |
| Re: | IEEE 802.16 Working Group Letter Ballot #30a, on P802.16m/D2 |  |
| Abstract | Proposed change to Femto ABS initialization |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to Femto ABS initialization (15.4.5)

Kanchei (Ken) Loa, Chun-Yen Hsu

Institute for Information Industry

# 1. Instruction

This contribution is proposed on the basis of C802.16m-09/1834r1.

# 2. Text Proposal for the 802.16m/ D2

------------------------------------------------------------Start of the Text--------------------------------------------------------

15.4.5 Femto ABS initialization

The Femto ABS shall perform Femto ABS initialization procedures to register itself to the network and to configure itself.

15.4.5.1 Femto ABS attachment to the Macro ABS and Core network

The Femto ABS shall perform Femto ABS initialization procedures to register itself to the network. For a Femto ABS that uses air interface connection with the overlapped Macro ABS for exchanging control messages, the Femto ABS shall perform ABS initialization procedures to register itself to the network and overlaid Macro ABS and obtain direct wireless link configuration information

The Femto ABS initialization procedures can be divided into the following pahses:

- Scan for DL channel and establish synchronization with the Macro ABS

  - Perform the overlaid Macro ABS selection(may be omitted if the overlaid Macro ABS was pre-configured)

- Obtain UL parameters

- Perform ranging

- Negotiate pre-authentication capabilities

- Authorize Femto ABS and perform key exchange

- Perform registration

- Set up connections with Macro ABS(basic and management service flows only)

- Obtain  initial neighbor list

- Perform neighbor measurement and report

- Obtain and configure Femto ABS air interfaces operation parameters

Phase a),ai),c),g) and i) are performed over the air interface with Macro ABS. Other phases are performed over the backhaul connection.

The Femto ABS initialization procedure can be divided into the following phases:

- Performs authentication with the network controller

- Perform registration to the network

- Perform scanning for DL channel

- Establish synchronization with the macro ABS if needed

- Obtain initial neighbor list

- Perform neighbor measurement and report

- Obtain configuration parameters

15.4.5.1 Femto ABS attachment to the Macro ABS

For a Femto ABS that uses air interface connection with the overlaid Macro ABS for exchanging control messages, the Femto ABS shall perform additional initialization procedure after the initialization procedure to register itself to overlaid Macro ABS and obtain direct wireless link configuration information

- Scan for DL channel and establish synchronization with the Macro ABS

h1) Perform the overlaid Macro ABS selection(may be omitted if the overlaid Macro ABS was pre-configured)

- Obtain UL parameters

- Perform ranging

- Negotiate pre-authentication capabilities

- Authorize Femto ABS and perform key exchange

- Capability exchange and perform registration

- Set up connections with Macro ABS(management service flows only)

- Obtain direct wireless link configuration parameters

Phases h), h1) and j) are performed over the air-interface with Macro ABS. Other phases are performed over the backhaul connection.

15.4.5.2 Femto ABS de-attachement detachment to the Macro ABS and Core Network

The Femto ABS de-registration procedure is performed through the backhaul network.

In the case of power down of Femto ABS, the Femto ABS sends out-of-service information to the subordinated MSs to prevent MSs entry or reentry from other cells. Before powering down of changing to the initialization state, the Femto ABS may request the subordinated MSs to perform handover to neighbor cell if required. In case of power down or reset, the Femto ABS performs de-registration procedure through the backhaul network to detach from the Core Network. If the Femto ABS maintains a connection with the overlaid Macro ABS over the air-interface it shall perform de-registration procedure to detach from the overlaid Macro ABS through the backhaul network (or air-interface). In case of backhaul link down, the Femto ABS does not perform de-registration procedure through the backhaul network. When the backhaul link of Femto ABS is down or the connection with the service provider network is lost for a configurable pre-defined time, the Femto ABS shall disable air interface and may store static user information. The user information may help AMS trying to call back to the Femto ABS, to perform optimized network reentry when the backhaul link is restored.

-------------------------------------------------------------End of the Text--------------------------------------------------------