| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to Femto ABS initialization(15.4.5) |  |
| Date Submitted | 2009-11-17 |  |
| Source(s) | Kanchei (Ken) Loa, Chun-Yen Hsu Institute for Information Industry Linghang Fan NEC | loa@iii.org.tw Linghang.Fan@eu.nec.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30a, on P802.16m/D2 |  |
| Abstract | Proposed change to Femto ABS initialization |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to Femto ABS initialization (15.4.5)

Kanchei (Ken) Loa, Chun-Yen Hsu

Institute for Information Industry

Linghang Fan

NEC

# 1. Instruction

This contribution is proposed on the basis of C802.16m-09/1834r1.

# 2. Text Proposal for the 802.16m/ D2

------------------------------------------------------------Start of the Text--------------------------------------------------------

15.4.5 Femto ABS initialization

The Femto ABS shall perform Femto ABS initialization procedures to register itself to the network and to configure itself through the backhaul connection.

15.4.5.1 Femto ABS attachment to the Macro ABS and Core network

The Femto ABS shall perform Femto ABS initialization procedures to register itself to the network. For a Femto ABS that uses air interface connection with the overlapped Macro ABS for exchanging control messages, the Femto ABS shall perform ABS initialization procedures to register itself to the network and overlaid Macro ABS and obtain direct wireless link configuration information

The Femto ABS initialization procedures can be divided into the following pahses:

- Scan for DL channel and establish synchronization with the Macro ABS

  - Perform the overlaid Macro ABS selection(may be omitted if the overlaid Macro ABS was pre-configured)

- Obtain UL parameters

- Perform ranging

- Negotiate pre-authentication capabilities

- Authorize Femto ABS and perform key exchange

- Perform registration

- Set up connections with Macro ABS(basic and management service flows only)

- Obtain  initial neighbor list

- Perform neighbor measurement and report

- Obtain and configure Femto ABS air interfaces operation parameters

Phase a),ai),c),g) and i) are performed over the air interface with Macro ABS. Other phases are performed over the backhaul connection.

15.4.5.1 Femto ABS attachment to the Macro ABS

For a Femto ABS that uses air interface connection with the overlaid Macro ABS for exchanging control messages, the Femto ABS shall perform the following additional initialization procedure during the Femto ABS initialization procedure.

- Scan for DL channel and establish synchronization with the Macro ABS

- Perform ranging

The details of Femto ABS initialization procedure including obtaining and configuring Femto ABS air interfaces operation parameters through the backhaul connection is out of scope of this specification.

-------------------------------------------------------------End of the Text--------------------------------------------------------