| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on predefined BR index mapping (16.2.3) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Daniel Cohn Alvarion Jack Jang Samsung Kelvin Chou Mediatek Xiangying Yang Intel | daniel.cohn@alvarion.com jack.jang@samsung.com kelvin.chou@mediatek.com xiangying.yang@intel.com |
| Re: | Letter Ballot #30b on the Draft Amendment (IEEE P802.16m/D3) |  |
| Abstract | D3 does not specify how MS should interpret different BR index mappings sent by BS in different DSx messages |  |
| Purpose | To be discussed and adopted by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on predefined BR index mapping (16.2.3)

Daniel Cohn, Alvarion

Jack Jang, Samsung

Kelvin Chou, Mediatek

Xiangying Yang, Intel

Problem Statement

In the 3-step bandwidth request procedures as defined in IEEE 802.16m/D3, the AMS can send a BR preamble sequence and a quick access message, where the quick access message is comprised of the AMS ST-ID and a predefined BR index. The predefined BR index is mapped to the requested BR size and the scheduling type of the requesting service flow, using a mapping table sent by the ABS in DSA/DSC-REQ messages.

The standard does not specify how the BR index should be mapped in a case where the ABS sends different BR index mappings for the same BR index in different DSA/DSC-REQ messages. In this case the ABS will not be able to unambiguously determine the BR size and QoS level requested by the AMS when this BR index is sent in a quick access message.

Proposed Solution

The standard should specify that when the BS sends a DSA/DSC-REQ message with BR index mapping and this mapping is confirmed by the MS in DSA/DSC-RSP message, BR index mappings in this message will override previous mapping for the same BR index.

Suggested Standard Modifications

Add in section 16.2.3.41.1, page 111, line 8:

After a successful DSA/DSC transaction, predefined BR index mappings included in the AAI_DSA-REQ or AAI_DSA-RSP messages will override previously redefined BR index mappings for the same BR indices.