| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal to change the Feedback Allocation A-MAP IE and Feedback Polling A-MAP IE so that ABS can specify whether Midamble can be used for CQI measurement or not |  |
| Date Submitted | 2009-12 |  |
| Source(s) | Li Guo, Hanqing Lou, Genyuan Wang, Yanxin Na, Hang Jin, Herbert Ruck Cisco Systems | liguo2@cisco.com |
| Re: | P802.16m/D3 LB30b |  |
| Abstract | The contribution proposes text to describe the usage of pilots for downlink measurements and burst demodulation. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 4) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal to change the Feedback Allocation A-MAP IE and Feedback Polling A-MAP IE so that ABS can specify whether Midamble can be used for CQI measurement or not

Li Guo, Hanqing Lou, Genyuan Wang, Yanxin Na, Hang Jin, Herbert Ruck

Cisco Systems

- Introduction

The contribution proposes the text to describe the usage of midamble/pilot for CQI calculation. It provides the mechanism so that ABS can inform the AMS that the AMS should use midamble or pilot for CQI measurement.

Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and struck through and existing AWD text is shown in black.

- References

[1] P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D3, Dec 2009.

- Text proposal for inclusion in the 802.16m AWD

< Make change in Table 821 as marked below >

-------------------------------  Start of text proposal #1 --------------------------------------------

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| Feedback-Allocation-MAP_IE() { |  |  |
| A-MAP IE Type | 4 | Feedback Allocation A-MAP IE = 0b0100 |
| Channel Index | 6 | Feedback channel index within the UL fast feedback control resource region (Dependent on LFB,FPi defined in 16.3.8.3.3.2) |
| Short-term feedback period (p) | 3 | Feedback is transmitted on the FBCH every 2p frames |
| Long-term feedback Period (q) | 2 | Long-term feedback is transmitted on the FBCH every 2q short-term feedback opportunities. If q = 0b00, long-term feedback is not used. |
| Frame offset | 3 | The AMS starts reporting at the frame of which the number has the same 3 LSB as the specified frame offset. If the current frame is specified, the AMS starts reporting in eight frames. |
| Subframe index | 3 | Indicates the UL AAI subframe index in the UL portion of the frame |
| Allocation Duration(d) | 3 | An FBCH is transmitted on the FBCH channels indexed by Channel Index for 8×2d frames. If d = 0b000, the FBCH is deallocated. If d = 0b111, the AMS should report until the ABS command for the AMS to stop. |
| ACK allocation flag | 1 | Indicates if one ACK channel is allocated to acknowledge the successful detection of this IE. |
| if ( ACK allocation flag == 0b1 ) { |  |  |
| HFA | 3 | HARQ feedback channel allocation for confirmation of Feedback Channel De-allocation |
| } |  |  |
| MFM | 3 | MIMO Feedback Mode as defined in Table 839 |
| MaxMt | Variable 1 or 2 | Variable number of bits - depends on number of transmit antennas Nt If Nt =2: (Any MFM) 0b0: 1 0b1: 2 If Nt =4: (Any MFM) 0b00: 1 0b01: 2 0b10: 3 0b11: 4 If Nt =8 (SU-MIMO, MFM 0,1,2,3,4) 0b00: 1 0b01: 2 0b10: 4 0b11: 8 If Nt =8: (MU-MIMO, MFM 5,6,7) 0b00: 1 0b01: 2 0b10: 3 0b11: 4 |
| if (MFM = 2,3,5,6) { |  |  |
| Feedback Format | 2 |  |
| } |  |  |
| if (MFM = 0,1,4,7) { |  |  |
| FPI | 2 | Frequency partition indication: ABS indicate AMS to send wideband CQI and STC rate of the frequency partition and reuse factor in the future: 0b00: Frequency partition index 0 0b01: Frequency partition index 1 0b10: Frequency partition index 2 0b11: Frequency partition index 3 |
| } |  |  |
| if (MFM = 0,1 and q!= 0b00{ |  |  |
| Long term FPI | 2 | Frequency partition indication: ABS indicate AMS to send wideband CQI and STC rate for the second frequency partition using long term feedback: 0b00: Frequency partition index 0 0b01: Frequency partition index 1 0b10: Frequency partition index 2 0b11: Frequency partition index 3 |
| } |  |  |
| if (MFM == 3,4,6,7) { |  | CL SU and MU MIMO |
| CM | 2 | Codebook Feedback Mode and Codebook Coordination Enable 0b00: base mode with CCE disabled 0b01: transformation mode with CCE disabled 0b10: differential mode with CCE disabled 0b11: base mode with CCE enabled |
| CS | 1 | Codebook subset if Nt= 4, otherwise CS shall be ignored. 0b0: report PMI from the base codebook or transformed base codebook 0b1: report PMI from the codebook subset or transformed codebook subset |
| } |  |  |
| if(MFM==0,1,2,5) { |  |  |
| Measurement Method Indication | 1 | 0b0: Use the midamble for CQI measurements 0b1: Use pilots in OL region with MaxMt streams associated with the burst allocation for CQI measurements |
| } |  |  |
| Padding | Variable | Padding to reach byte boundary |
| } |  |  |

-------------------------------  End of text proposal #1 --------------------------------------------

< Make change in Table 827 as marked below >

-------------------------------  Start of text proposal #2 --------------------------------------------

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| Feedback_Polling_A-MAP_IE(){ |  |  |
| A-MAP IE Type | 4 | Feedback Polling IE |
| Allocation Duration (d) | 3 | The allocation is valid for 2(d-1) superframes starting from the superframe defined by allocation relevance. If d == 0b000, the pre-scheduled feedback transmission is released. If d == 0b111, the pre-scheduled feedback transmission shall be valid until the ABS commands to release it. |
| if (d ==0b000){ |  | Feedback de-allocation |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| HFA | 3 | HARQ feedback channel allocation for Feedback Channel De-allocation confirmation |
| } else if (d != 0b000){ |  |  |
| Dedicated UL allocation | ` | 0b0: No dedicated UL resource is allocated. ABS shall provide UL allocation for the MIMO feedback IE transmission through UL A-MAP IE at each designated transmitting frame defined by this IE. 0b1: Dedicated UL resource is included in this IE |
| if (Dedicated UL allocation ==0b1){ |  |  |
| ISizeOffset | 5 | Offset used to compute burst size index |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| MEF | 1 | MIMO encoder format for uplink feedback transmission Non-adaptive precoding shall be used at the AMS. 0b0: SFBC 0b1: VE with Mt= 2 if Nt > 1, or VE with Mt = 1 if Nt = 1 Nt is the number of transmit antennas at the AMS. |
| } |  |  |
| Period (p) | 3 | Transmit feedback every 4p frame. The first report shall start at the next frame. |
| MIMO_feedback_IE_type | 1 | 0b0: feedback for single-BS MIMO operation 0b1: feedback for multi-BS MIMO operation |
| if (MIMO_feedback_IE_type == 0b0){ |  | Single-BS MIMO feedback request |
| Transmit_Correlation_Matrix | 1 | 0b0: feedback of the transmit correlation matrix is indicated by CM 0b1: feedback of the quantized ABS transmit correlation matrix only Transmit correlation matrix shall be feedback if CM = 0b1 |
| if (Transmit_Correlation_Matrix == 0b0){ |  | ABS requests AMS to feedback CQI And CSI for a specific MFM. |
| MaxMt | Variable 1 or 2 | Variable number of bits - depends on number of transmit antennas Nt If Nt =2: (Any MFM) 0b0: 1 0b1: 2 If Nt =4: (Any MFM) 0b00: 1 0b01: 2 0b10: 3 0b11: 4 If Nt =8 (SU-MIMO, MFM 0,1,2,3,4) 0b00: 1 0b01: 2 0b10: 4 0b11: 8 If Nt =8: (MU-MIMO, MFM 5,6,7) 0b00: 1 0b01: 2 0b10: 3 0b11: 4 |
| MFM | 3 | MIMO Feedback Mode for which the AMS shall transmit feedback |
| if (MFM = 2,3,5,6) { |  |  |
| Num_best_subbands | 2 | 0b00: report all subbands 0b01: 1 best subband 0b10: min{6, YSB} best subbands 0b11: min{12, YSB} best subbands 1< Num_best_subbands <= YSB |
| } |  |  |
| if (MFM = 0,1,4,7) { |  | Feedback of CQI and CSI for CL SU and MU MIMO |
| CM | [TBD] | [TBD] |
| CS | 1 | 0b0: report PMI from the base codebook 0b1: report PMI from the codebook subset |
| } |  |  |
| if(MFM==0,1,2,5) { |  |  |
| Measurement Method Indication | 1 | 0b0: Use the midamble for CQI measurements 0b1: Use pilots in OL region with MaxMt streams associated with the burst allocation for CQI measurements |
| } |  |  |
| } |  |  |
| } |  |  |
| else{ |  | Multi-BS MIMO feedback request |
| TRU | 2 | Target RU indicating which RUs or which type of RU to work on for feedback 0b00: Latest best subbands reported for single BS MIMO 0b01: Whole bandwidth 0b10: FFR partition 0 0b11: boosted FFR partition |
| ICT | 2 | 0b00: PMI restriction for single-BS precoding; 0b01: PMI recommendation for single-BS precoding; 0b10: CL-MD for multi-BS precoding; 0b11: Co-MIMO for multi-BS precoding; |
| CS | 1 | 0b0: report PMI from the base codebook 0b1: report PMI from the codebook subset |
| N_multiBS_reports | 3 | N_multiBS_reports indicates the number of reports. |
| if (ICT = 0b11) { |  |  |
| MaxUser | 2 | Maximum number of users supported in Co- MIMO in the same resource. 0b00: 2 users 0b01: 3 users 0b10: 4 users 0b11: reserved |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| Padding | Variable | Padding to reach byte boundary |
| } |  |  |

-------------------------------  End of text proposal #2 --------------------------------------------