| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal to add a mechanism so that some pilot streams in downlink MIMO are silent to allow for interference information observation |  |
| Date Submitted | 2009-12 |  |
| Source(s) | Hang Jin, Li Guo, Hanqing Lou, Genyuan Wang, Yanxin Na, Herbert Ruck Cisco Systems | hangjin@cisco.com |
| Re: | P802.16m/D3 LB30b |  |
| Abstract | The contribution proposes text to describe the usage of pilots for downlink measurements and burst demodulation. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 4) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal to add a mechanism so that some pilot streams in downlink MIMO are silent to allow for interference information observation

Hang Jin, Li Guo, Hanqing Lou, Genyuan Wang, Yanxin Na, Herbert Ruck

Cisco Systems

- Introduction

The contribution proposes the text to add one mechanism so that the AMS can obtain the information of interference associated with each burst allocation in downlink. This interference information would be very useful for interference suppression. In this mechanism, the ABS does not transmit anything in some pilot stream symbols. Then the AMS can obtain the information of interference through those ‘silent’ pilot streams in downlink.

Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and struck through and existing AWD text is shown in black.

- References

[1] P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D3, Dec 2009.

- Text proposal for inclusion in the 802.16m AWD

<Make change in Table 812 as marked below>

-------------------------------  Start of text proposal #1 --------------------------------------------

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| DL_Basic_Assignment_A-MAP_IE() { |  |  |
| A-MAP IE Type | 4 | DL Basic Assignment A-MAP IE |
| ISizeOffset | 5 | Offset used to compute burst size index |
| MEF | 2 | MIMO encoder format 0b00: SFBC 0b01: Vertical encoding 0b10: Horizontal encoding 0b11: CDR |
| if (MEF == 0b01){ |  | Parameters for vertical encoding |
| Mt | 3 | Number of streams in transmission Mt <= Nt Nt : Number of transmit antennas at the ABS 0b000: 1 stream 0b001: 2 streams 0b010: 3 streams 0b011: 4 streams 0b100: 5 streams 0b101: 6 streams 0b110: 7 streams 0b111: 8 streams |
| MsilentPilostStream | 3 | Number of ‘silent’ pilot streams. MsilentPilostStream <Mt Mt: Number of streams in transmission 0b000: 0 ‘silent’ stream 0b001: 1 ‘silent’ streams 0b010: 2 ‘silent’ streams 0b011: 3 ‘silent’ streams 0b100: 4 ‘silent’ streams 0b101: 5 ‘silent’ streams 0b110: 6 ‘silent’ streams 0b111: 7 ‘silent’ streams |
| Reserved | 1 | Reserved bit |
| } else if(MEF == 0b10) { |  | Parameters for horizontal encoding |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size. |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 DL AAI subframes for FDD or all DL AAI subframes for TDD |
| HFA | 3 | HARQ Feedback Allocation |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | 4 | HARQ channel identifier |
| SPID | 2 | HARQ subpacket identifier for HARQ IR 0b00: 0 0b01: 1 0b10: 2 0b11: 3 |
| CRV | 1 | Constellation Rearrangement Version 0b0: 0 0b1: 1 |
| Reserved | 2 | Reserved bits |
| } |  |  |

-------------------------------  End of text proposal #1 --------------------------------------------

<Insert the following paragraph on page 397 on line 53 (16.3.6.5.2.4.1) >

-------------------------------  Start of text proposal #2 --------------------------------------------

MsilentPilostStream: specify the number of ‘silent’ pilot streams. Its value is between 0 and Mt-1, where Mt is number of MIMO streams. If MsilentPilostStream  >0,  ABS transmits pilots on pilot streams 0 to Mt- MsilentPilostStream-1and does not transmit pilot streams Mt- MsilentPilostStream to Mt-1. The AMS could use the silent symbols on the pilot streams Mt- MsilentPilostStream to Mt-1 to collect the information of interference. When the pilot streams are silent, their associated data streams will not be transmitted.

-------------------------------  End of text proposal #2 --------------------------------------------