| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal to add a mechanism so that some pilot streams in uplink MIMO are silent to allow for interference information observation |  |
| Date Submitted | 2009-12 |  |
| Source(s) | Hang Jin, Li Guo, Hanqing Lou, Genyuan Wang, Yanxin Na, Herbert Ruck Cisco Systems | hangjin@cisco.com |
| Re: | P802.16m/D3 LB30b |  |
| Abstract | The contribution proposes text to describe the usage of pilots for downlink measurements and burst demodulation. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 4) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal to add one mechanism so that some pilot streams in uplink MIMO are silent to allow for interference information observation

Hang Jin, Li Guo, Hanqing Lou, Genyuan Wang, Yanxin Na, Herbert Ruck

Cisco Systems

- Introduction

The contribution proposes the text to add one mechanism so that the ABS can obtain the information of uplink interference associated with each burst allocation. This interference information would be very useful for interference suppression. In this mechanism, the AMS does not transmit anything in some pilot stream symbols. Then the ABS can obtain the information of interference through those ‘silent’ pilot streams.

Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and struck through and existing AWD text is shown in black.

- References

[1] P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D3, Dec 2009.

- Text proposal for inclusion in the 802.16m AWD

< Make change in Table 813 as marked below >

-------------------------------  Start of text proposal #1 --------------------------------------------

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL_Basic_Assignment_A-MAP_IE() { |  |  |
| A-MAP IE Type | 4 | UL Basic Assignment A-MAP IE |
| ISizeOffset | 5 | Offset used to compute burst size index |
| CSM-PMI Indicator | 1 | Flag to indicate if both CSM and PMI are signaled 0b0: SU-MIMO with or without PMI indication , or CSM without PMI indication 0b1: CSM with PMI indication |
| if(CSM-PMI Indicator == 0b0){ |  |  |
| Mt | 2 | Number of streams in transmission, up to 4 streams per AMS supported Nt : Number of transmit antennas at the AMS 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| SU-PMI Indicator | 1 | Flag to indicate if both SU-MIMO and PMI are signaled 0b0: SU MIMO without PMI indication, or CSM without PMI indication 0b1: SU MIMO with PMI indication |
| If(SU-PMI Indicator == 0b0){ |  |  |
| CSM | 1 | Flag to indicate CSM 0b0: SU-MIMO 0b1: CSM |
| if(CSM == 0b0){ |  |  |
| MsilentPilostStream | 2 | Number of ‘silent’ pilot streams. MsilentPilostStream <Mt Mt: Number of streams in transmission 0b00: 0 ‘silent’ stream 0b01: 1 ‘silent’ streams 0b10: 2 ‘silent’ streams 0b11: 3 ‘silent’ streams |
| MEF | 1 | MIMO encoder format 0b0: SFBC 0b1: Vertical encoding Non-adaptive precoding shall be used at the AMS with SFBC. Mt shall be ignored with SFBC |
| if(MEF == 0b0){ |  |  |
| Reserved | 5 | Reserved bits |
| else if(MEF == 0b1){ |  |  |
| PF | 1 | Precoding flag for SU-MIMO when PMI is not signaled 0b0: Non-adaptive precoding 0b1: Adaptive precoding using the precoder of rank Mt of the AMS's choice |
| Reserved | 4 | Reserved bits |
| } |  |  |
| } else if(CSM == 0b1){ |  | Parameters for CSM |
| MsilentPilostStream | 2 | Number of ‘silent’ pilot streams. MsilentPilostStream <Mt Mt: Number of streams in transmission 0b00: 0 ‘silent’ stream 0b01: 1 ‘silent’ streams 0b10: 2 ‘silent’ streams 0b11: 3 ‘silent’ streams |
| PF | 1 | Precoding flag for CSM when PMI is not signaled 0b0: Non-adaptive precoding 0b1: Adaptive precoding using the precoder of rank Mt of the AMS's choice |
| TNS | 2 | Total number of streams in the LRU for CSM 0b00: reserved 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| SI | 2 | First pilot index for CSM with TNS = 2 streams: 0b00, 0b01 First pilot index for CSM with TNS = 3,4: 0b00, 0b01, 0b10, 0b11 |
| Reserved | 1 | Reserved bit |
| } |  |  |
| }else if (SU-PMI Indicator == 0b1){ |  | Parameters for SU-MIMO with PMI (adaptive precoding) |
| PMI | 6 | 4 bits PMI for Nt = 2, first 2 MSB set to 0 6 bits PMI for Nt = 4 |
| Reserved | 1 | Reserved bit |
| } |  |  |
| }else if(CSM-PMI Indicator == 0b1){ |  | Parameters for CSM with PMI (adaptive precoding) |
| CSM Format | 4 | For non CSM, modes, TNS = Mt. For CSM modes the following combinations apply for upto 4 streams 0b0000: TNS = 2, Mt=1, SI =1 0b0001: TNS = 2, Mt =1, SI =2 0b0010: TNS = 3, Mt =1, SI =1 0b0011: TNS = 3, Mt =1, SI =2 0b0100: TNS = 3, Mt =1, SI =3 0b0101: TNS = 3, Mt =2, SI =1 0b0110: TNS = 3, Mt =2, SI =2 0b0111: TNS = 4, Mt =1, SI =1 0b1000: TNS = 4, Mt =1, SI =2 0b1001: TNS = 4, Mt =1, SI =3 0b1010: TNS = 4, Mt =1, SI =4 0b1011: TNS = 4, Mt =2, SI =1 0b1100: TNS = 4, Mt =2, SI =2 0b1101: TNS = 4, Mt =2, SI =3 0b1110: TNS = 4, Mt =3, SI =1 0b1111: TNS = 4, Mt =3, SI =2 |
| PMI | 6 | 4 bits PMI for Nt = 2, first 2 MSB set to 0 6 bits PMI for Nt = 4 |
| } |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size If all bits are set to 1, the AMS shall not transmit the corresponding HARQ subpacket for the ACID in this IE. The retransmission number is incremented by one for the skipped retransmission. |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 UL AAI subframes for FDD or all UL AAI subframes for TDD If number of DL AAI subframes, D, is less than number of UL AAI subframes, U, Long TTI Indicator= 0b1 |
| HFA | 3 | HARQ Feedback Allocation |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | 3 | HARQ channel identifier |
| Reserved | 1 | Reserved bit |
| } |  |  |

-------------------------------  End of text proposal #1 --------------------------------------------

<Insert the following paragraph on page 403 on line 5 (16.3.6.5.2.4.2) >

-------------------------------  Start of text proposal #2 --------------------------------------------

MsilentPilostStream: specify the number of ‘silent’ pilot streams. Its value is between 0 and Mt-1, where Mt is number of streams. If MsilentPilostStream  >0,  AMS transmits pilots on pilot streams 0 to Mt- MsilentPilostStream-1 and does not transmit pilot streams Mt- MsilentPilostStream to Mt-1. The ABS could use the silent symbols on the pilot streams Mt- MsilentPilostStream to Mt-1 to collect the information of interference. When the pilot streams are silent, their associated data streams will not be transmitted.

-------------------------------  End of text proposal #2 --------------------------------------------