| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal to modify and describe the pilot-based UL midamble-like non-adaptive precoder in UL traffic transmission |  |
| Date Submitted | 2009-12 |  |
| Source(s) | Yanxin Na; Hang Jin; Li Guo; Genyuan Wang; Hanqing Lou; Herbert Ruck; Bror Peterson Cisco Systems | yanxna@cisco.com |
| Re: | P802.16m/D3 |  |
| Abstract | The contribution proposes text to describe the usage of the pilot-based UL midamble-like non-adaptive precoder in UL traffic transmission |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 4) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal to modify and describe the pilot-based UL midamble-like non-adaptive precoder in UL traffic transmission

Yanxin Na; Hang Jin; Li Guo; Genyuan Wang; Hanqing Lou; Herbert Ruck; Bror Peterson

Cisco Systems

- Introduction

The contribution proposes the text to add and describe the usage of the pilot-based UL midamble-like non-adaptive precoder in UL traffic transmission. It provides the mechanism for AMS to provide the channel information with pilot-based midamble-like transmission structure to ABS, from which ABS will compute and feedback the PMI to AMS for UL MIMO transmission. Also the ABS could use the UL pilot-based midamble-like transmission to decide the DL adaptive precoding for the DL MIMO transmission.

There is DL midamble for DL close loop MIMO transmission, but there isn’t the UL midamble for UL close loop MIMO transmission.  If each AMS transmits an UL midamble for UL close loop MIMO transmission, the overhead is too much. Thus we propose a scheme where the pilots with special non-adaptive precoders can also be used as UL midambles for UL traffic transmission. This scheme wouldn’t add any overhead to the UL transmissions and but yet still serves the purpose of UL midamble.

Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and struck through and existing AWD text is shown in black.

- References

[1] P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D3, Dec 2009.

- Text proposal for inclusion in the 802.16m AWD

< Modify the text in the section 16.3.10.2.1.2 Precoding of SU-MIMO modes in the page 568 from line 60 to 62.>

-------------------------------  Start of text proposal #1 --------------------------------------------

16.3.10.2.1.2 Precoding of SU-MIMO modes

•MIMO mode 0: Non-adaptive precoding with Mt=2 MIMO streams shall be used with MIMO mode 0 except for the case where STC rate=1 and Nt (number of transmit antennas) = Mt (Number of MIMO streams). In this case, the precoder [1 0] should be used for the first AMS antenna and the precoder [0 1] should be used for the second AMS antenna. This will provide the pilot-based UL midamble-like mode for UL traffic transmission.

-------------------------------  End of text proposal #1 --------------------------------------------