| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal to add and describe the option of adaptive precoder in open loop case in UL MIMO transmission |  |
| Date Submitted | 2009-12 |  |
| Source(s) | Yanxin Na; Hang Jin; Li Guo; Genyuan Wang; Hanqing Lou; Herbert Ruck; Bror Peterson Cisco Systems | yanxna@cisco.com |
| Re: | P802.16m/D3 |  |
| Abstract | The contribution proposes text to describe the usage of option of adaptive precoder in openloop case in UL MIMO transmission. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 4) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal to add and describe the option of adaptive precoder in open loop case in UL MIMO transmission

Yanxin Na; Hang Jin; Li Guo; Genyuan Wang; Hanqing Lou; Herbert Ruck; Bror Peterson

Cisco Systems

- Introduction

The contribution proposes the text to add and describe the usage of option of adaptive precoder in open loop case in UL MIMO transmission. It provides the mechanism so that ABS can inform the AMS that UL MIMO transmission is in open loop case with vendor specific adaptive precoder.

MIMO transmission in open loop mode could provide over-all high system throughput. Adaptive precoder could provide better performance than non-adaptive precoder; DLRU could provide better frequency diversity. Combining adaptive precoder with the DLRU will leverage both benefits of adaptive precoder and DLRU for UL MIMO transmissions with UL open MIMO mode.  Thus we propose to add the option of adaptive precoder in open loop mode in UL MIMO transmission.

Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and struck through and existing AWD text is shown in black.

- References

[1] P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D3, Dec 2009.

- Text proposal for inclusion in the 802.16m AWD

< Modify the table 742—MIMO Bitmap information for UL in section 16.2.9.4.1 Bitmaps in Group Resource Allocation in the page 202 from line 39 to 52. Add the adaptive option in Table 915—Uplink MIMO modes in the page 567 from line 25 to 32. Modify the text in the section 16.3.10.2.1.2 — Precoding of SU-MIMO modes in the page 568 from line 60 to 64>

-------------------------------  Start of text proposal #1 --------------------------------------------

Table 742—MIMO Bitmap Information for UL

| MIMO Mode Set | Existance of Second Bitmap | Numberof Bit Per Scheduled AMS | MIMO Mode Indication |
| --- | --- | --- | --- |
| 0b00 | NoYes | N/A 1 | 0b0: OL SU-MIMO (SFBC with non-adaptive precoder) 0b1: OL SU-MIMO (SFBC with vendor specific adaptive precoder) |
| 0b01 | Yes | 1 2 | 0b00: OL SU-MIMO (SFBC with non-adaptive precoder) with Mt=2 0b01: OL SU-MIMO (SFBC with vendor specific adaptive precoder) with Mt=2 0b10: OL SU-MIMO (SM with non-adaptive precoder) with Mt=2 0b11: OL SU-MIMO (SM with vendor specific adaptive precoder) with Mt=2 |
| 0b10 | No | N/A | CL SU-MIMO with Mt=1, TNS=2 |

Table 915—Uplink MIMO modes

| Index | Description | MIMO encoding format (MEF) | MIMO Precoding |
| --- | --- | --- | --- |
| Mode 0 | OL SU-MIMO | SFBC | non-adaptive, adaptive (vendor specific) |
| Mode 1 | OL SU-MIMO (SM) | VE | non-adaptive, adaptive (vendor specific) |
| Mode 2 | CL SU-MIMO (SM) | VE | adaptive |
| Mode 3 | OL Collaborative spatial multiplexing (MU-MIMO) | VE | non-adaptive |
| Mode 4 | CL Collaborative spatial multiplexing (MU-MIMO) | VE | adaptive |

16.3.10.2.1.2 Precoding of SU-MIMO modes

•MIMO mode 0: Non-adaptive precoding, or vendor specific precoding, with MIMO streams shall be used with MIMO mode 0.

•MIMO mode 1: Non-adaptive precoding, or vendor specific precoding, with Mt MIMO streams shall be used with MIMO mode 1.

-------------------------------  End of text proposal #1 --------------------------------------------