| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal to add and describe the option of adaptive precoder in open loop case in DL MIMO transmission |  |
| Date Submitted | 2009-12 |  |
| Source(s) | Hang Jin; Yanxin Na; Li Guo; Genyuan Wang; Hanqing Lou; Herbert Ruck Cisco systems |  |
| Re: | P802.16m/D3 |  |
| Abstract | The contribution proposes text to describe the usage of option of adaptive precoder in openloop case in DL MIMO transmission. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 3) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal to add and describe the option of adaptive precoder in open loop case in DL MIMO transmission

Hang Jin; Yanxin Na; Li Guo; Genyuan Wang; Hanqing Lou; Herbert Ruck

Cisco systems

- Introduction

The contribution proposes the text to add and describe the usage of option of adaptive precoder in open loop case in DL MIMO transmission. It provides the mechanism so that ABS can inform the AMS that DL MIMO transmission is in open loop case with vendor specific adaptive precoder.

MIMO transmission in open loop mode could provide over-all high system throughput. Adaptive precoder could provide better performance than non-adaptive precoder; and DLRU could provide better frequency diversity.. Combining adaptive precoder with the DLRU will leverage both benefits of adaptive precoder and DLRU for DL MIMO transmissions with DL open MIMO mode. Thus we propose to add the option of adaptive precoder in open loop mode in DL MIMO transmission.

Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and struck through and existing AWD text is shown in black.

- References

[1] P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D3, Dec 2009.

- Text proposal for inclusion in the 802.16m AWD

< Modify the tables 741 —MIMO Bitmap Information for DL (in the section 16.2.9.4.1 Bitmaps in Group Resource Allocation in page 202 from line 23 to 35) that is transmitted in DL group resource allocation A-MAP IE.  Add the adaptive option in Table 833—Downlink MIMO modes in page 450 from line 8 to 12>

-------------------------------  Start of text proposal #1 --------------------------------------------

Table 741— MIMO Bitmap Information for DL

| MIMO Mode Set | Existance of Second Bitmap | Number of Bit Per Scheduled AMS | MIMO Mode Indication |
| --- | --- | --- | --- |
| 0b00 | NoYes | N/A 1 | 0b0: OL SU-MIMO (SFBC with non-adaptive precoder) 0b1: OL SU-MIMO (SFBC with vendor specific adaptive precoder) |
| 0b01 | Yes | 1 2 | 0b00: OL SU-MIMO (SFBC with non-adaptive precoder) 0b01: OL SU-MIMO (SFBC with vendor specific adaptive precoder) 0b10: OL SU-MIMO (SM with non-adaptive precoder) with Mt=2 0b11: OL SU-MIMO (SM with vendor specific adaptive precoder) with Mt=2 |
| 0b10 | No | N/A | CL SU-MIMO with Mt=1 |

Table 833—Downlink MIMO modes

| Mode index | Description | MIMO encoding format (MEF) | MIMO precoding |
| --- | --- | --- | --- |
| Mode 0 | OL SU-MIMO (Tx diversity) | SFBC | non-adaptive, adaptive (vendor specific) |
| Mode 1 | OL SU-MIMO (SM) | VE | non-adaptive, adaptive(vendor specific) |
| Mode 2 | CL SU-MIMO (SM) | VE | adaptive |
| Mode 3 | OL MU-MIMO (SM) | HE | non-adaptive |
| Mode 4 | CL MU-MIMO (SM) | HE | adaptive |
| Mode 5 | OL SU-MIMO (Tx diversity) | CDR | non-adaptive |

-------------------------------  End of text proposal #1 --------------------------------------------