| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for AMS scanning of neighboring CSG Femto ABSs by dedicated periodic ranging (16.4.8.1.2) |  |
| Date Submitted | 2009-12-28 |  |
| Source(s) | Chie Ming Chou, Yung-Han Chen, Frank Ren, Chun-Yen Wang, Jung-Mao Lin ITRI Wern Ho Sheen Chaoyang University of Technology / ITRI | E-mail: chieming@itri.org.tw whsheen@itri.org.tw |
| Re: | Change Request on 802.16m Draft Amendment, IEEE P802.16m/D3 |  |
| Abstract | The contribution proposes the text of AMS scanning of neighboring Femto ABSs (Section 16.4.8.1.2). |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D3. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for AMS scanning of neighboring CSG Femto ABSs by dedicated periodic ranging

Chie Ming Chou, Yung-Han Chen, Frank Ren, Chun-Yen Wang, Jung-Mao Lin

ITRI

Wern Ho Sheen

Chaoyang University of Technology / ITRI

- Introduction and discussion

In D3 draft, A CSG Femto ABS can monitor UL signaling of accessible AMS to determine the proximity of AMS in its coverage. How to let CSG Femto ABS which is not in low-duty mode to support this operation by dedicated periodic ranging was not well described. In this contribution, we propose the details for dedicated periodic ranging to support AMS scanning of neighboring CSG Femto ABSs.

- Proposed Text

[Accept the following modifications marked in blue underline]

---------------------------------------------------- Start of Proposed Text----------------------------------------------------

[Modify the text in Section 16.4.8.1.2]

16.4.8.1.2 AMS scanning of neighbor Femto ABSs

Alternatively, an overlapped Macro ABS may recommend CSG Femto ABS to monitor UL signaling of accessible AMS which is served by the Macro ABS. If CSG Femto ABS is in low-duty mode, the CSG Femto ABS can try to receive A-MAP information from overlapped Macro ABS and then monitor accessible AMS's UL signaling in associated allocations. If CSG Femto ABS is not in low-duty mode, overlapped Macro ABS may trigger dedicated periodic ranging (see 1516.4.7.4.2) for the AMS. A CSG Femto ABS can monitor the ranging preamble at the dedicated ranging slot of Macro ABS. When the received signaling quality (e.g RSSI) of an AMS is stronger than a threshold, CSG Femto ABS can request overlapped Macro ABS to send unsolicited AAI_SCN-RSP for the AMS to scan the CSG Femto ABS.

[Insert the sub-section in Section 16.4.7]

16.4.7.2 Dedicated periodic ranging

To support scanning and decision of AMS’s proximity, a Macro ABS can request AMS to perform dedicated periodic ranging by sending AAI_RNG-RSP message. When an AMS receives a AAI_RNG-RSP message with dedicated periodic ranging information, it shall periodically send the assigned ranging preamble code to serving ABS at the dedicated ranging slot. The assigned ranging preamble code will be transmitted with a fixed pre-defined power. The dedicated ranging slot shall be in one of the ranging channel for synchronized AMSs. Macro ABS can send AAI_RNG-ACK message with IR abort indication to terminate the dedicated periodic ranging if needed.

[Insert the new column in Table 675]

Table 675 Parameters for AAI_RNG-RSP

| Name | Value | Usage |
| --- | --- | --- |
| Start frame | 0~63 | It shall be included when an ABS wants to initiate dedicated periodic ranging. Least Significant 6 bits of Frame Number. Used to indicate when to start the dedicated periodic ranging, (in frame unit) |
| Transmission power | TBD | It shall be included when an ABS wants to initiate dedicated periodic ranging. AMS shall transmit the ranging preamble code with this recommended power. |
| Ranging preamble code | 0~63 | It shall be included when an ABS wants to initiate dedicated periodic ranging. Used to indicate the reserved periodic ranging preamble codes for dedicated ranging. |
| Ranging slot | TBD | It shall be included when an ABS wants to initiate dedicated periodic ranging. Indicates the OFDMA time symbols reference, subchannel reference and frame reference for transmitting the ranging preamble code |
| Period | 0b00: every superframe 0b01: every 4 superframe 0b10: every 8 superframe 0b11: every 12 superframe | It shall be included when an ABS wants to initiate dedicated periodic ranging. Indicates the periodicity of transmitting the dedicated ranging preamble |

---------------------------------------------------- End of Proposed Text ------------------------------------------------