| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Trigger averaging definition (16.2.6/16.2.3) |  |
| Date Submitted | 2010-01-07 |  |
| Source(s) | Daniel Cohn Alvarion Joseph Schumacher Motorola | Daniel.cohn@alvarion.com j.schumacher@motorola.com |
| Re: | Letter Ballot #30b on the Draft Amendment (IEEE P802.16m/D3) |  |
| Abstract | Definition of AMS metric averaging process for triggers |  |
| Purpose | To be discussed and adopted by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Trigger averaging definition (16.2.6/16.2.3)

Daniel Cohn, Alvarion

Joseph Schumacher, Motorola

Problem Statement

Triggers defined in D3 section 16.2.6.2 include a "trigger averaging duration" (TAD) field that specifies the "time measured in number of frames over which the metric measurements are averaged". In addition the AAI_SCD message includes a field called "DefaultHORSSI_CINRaveraggingparameters" which is not defined or mentioned anywhere else in the standard.

The standard however does not define how the metric averaging process is performed, leading to a situation where the same ABS-advertised TAD parameter can result in different handover performance for different AMS vendors. For instance, one AMS vendor could use the arithmetic average of all metrics taken during the last TAD frames, while another AMS vendor could implement a single-pole IIR (aka exponential smoother) with an equivalent TAD time constant.

This potential non-standard behavior among AMS vendors complicates the network fine tuning process for operators.

In addition, the relationship between the two above-mentioned parameters is not defined in the standard.

Proposed Solution

This contribution proposes the use of a single-pole IIR (aka exponential smoother), as specified in IEEE 802.16-2009 in section 8.4.12, equation 158.

The proposed averaging filter accounts for non-uniformly spaced metric samples (e.g. obtained in different inter-frequency scan profiles).

The averaging parameter for the filter is be taken from the trigger averaging parameter field in the trigger TLV (renamed from trigger averaging duration) if present. Otherwise, the default trigger averaging parameter (renamed from DefaultHORSSI_CINRaveraggingparameters) in the AAI_SCD message is be used.

Let x[k] be the metric the AMS is required to average (e.g. RSSI) measured at frame k. Letx[k]be the averaged metric at frame k. Then:

x[k]=x[k0] k=k0 (1−αavg)n+1x[k−n]+(1−(1−αavg)n+1)x[k]k>k0

Where:

αavg = trigger averaging parameter

Suggested Standard Modifications

1.- Add the following text in section 16.2.6.2 page 160, after line 38:

Metric averaging is performed according to equation (xxx)

Let x[k] be the metric the AMS is required to average (e.g. RSSI) for the k-th measurement, expressed in linear units. Then x[k], the averaged metric expressed in linear units, equals:

x[k]=x[0] k=0 (1−αavg)n+1x[k−1]+(1−(1−αavg)n+1)x[k]k>0 	(xxx)

Where:

αavg = trigger averaging parameter in relevant trigger TLV. If this field is not present, the default trigger averaging parameter in AAI_SCD is used.

n = number of consecutive frames in which no measurement was made

In frames in which no measurement is made, the AMS shall report the latest averaged results.

2.- Modify table 731 in section 16.2.6.2 as follows:

| Name | Type | Length (bytes) | Value |
| --- | --- | --- | --- |
| Type/Function/Action | xx.1 | 1 | See Table 732—for description |
| Trigger Value | xx.2 | 1 | Trigger value is the value used in comparing measured metric for determining a trigger condition |
| Trigger averaging duration parameter | xx.3 | 1 | Trigger averaging duration is the time measured in number of frames over which the metric measurements are averaged. The averaging parameter used for averaging this trigger metric according to equation xxx. If not present, the default trigger averaging parameter in AAI_SCD is used. 0x0: 1 0x1: 1/2 0x2: 1/4 0x3: 1/8 0x4: 1/16 0x5: 1/32 0x6: 1/64 0x7: 1/128 0x8: 1/256 0x9: 1/512 0x10 to 0xFF: reserved |
| ABS type | xy.1 | 1 | ABS type of target ABS for this Trigger TLV: (Macro Hot-zone ABS, Femto ABS, etc.) |
| Type/Function/Action | xy.2 | 1 | See Table YY for description |
| Trigger Value | xy.3 | 1 | Trigger value is the value used in comparing measured metric for determining a trigger condition from serving ABS to target ABS (which is defined in ABS type). |
| Trigger averaging duration parameter | xy.4 | 1 | The averaging parameter used for averaging this trigger metric according to equation xxx for target ABS (which is defined in ABS type). If not present, the default trigger averaging parameter in AAI_SCD is used 0x0: 1 0x1: 1/2 0x2: 1/4 0x3: 1/8 0x4: 1/16 0x5: 1/32 0x6: 1/64 0x7: 1/128 0x8: 1/256 0x9: 1/512 0xA to 0xFF: reserved .Trigger averaging duration for target ABS (which is defined in ABS type) is the time measured in number of frames over which the metric measurements are averaged. |

3.- Modify section 16.2.3.29, page 64 line 59 as follows:

Default trigger averaging parameter (used in equation xxx)	INTEGER (0..9)

0x0: 1

0x1: 1/2

0x2: 1/4

0x3: 1/8

0x4: 1/16

0x5: 1/32

0x6: 1/64

0x7: 1/128

0x8: 1/256

0x9: 1/512

DefaultHORSSI_CINRaveraggingparameters :: = SEQUENCE {}