| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-Up of Multi-BS MIMO Trigger Mechanism (16.5.1.4) |  |
| Date Submitted | 2009-12-28 |  |
| Source(s) | Luciano Sarperi, Hui Xiao Fujitsu | luciano.sarperi@uk.fujitsu.com hui.xiao@uk.fujitsu.com |
| Re: | IEEE 802.16-09/0073: IEEE 802.16 Working Group Letter Ballot Recirc #30b Announcement |  |
| Abstract | This contribution proposes clean-up text for the multi-BS MIMO trigger mechanism. |  |
| Purpose | Discussion and approval by TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Clean-Up of Multi-BS MIMO Trigger Mechanism

Luciano Sarperi, Hui Xiao

Fujitsu

# Introduction

This contribution contains clean-up text for the multi-BS MIMO trigger mechanism in [1]. Additionally, some of the multi-BS MIMO related text adopted at the last session #64 has not been implemented in [1] and is therefore included here again.

# Proposed modification to the 802.16 Amendment Draft (IEEE P802.16m/D3)

Black text: existing text

Red striked trough text: deleted text

Blue text: new text

## Proposed Text 1 ##

<< [Modify Section 16.5.1.4, Page 611] >>

------------------------------------------------------  Text 2 Start  ---------------------------------------------------------------

16.5.1.4 Multi-BS MIMO trigger mechanism

ABS broadcasts normalized interference power (NIP) thresholds for 2 types of multi-BS MIMO schemes and one common CINR threshold. The NIP is defined as ratio of average interference power (with or without transmitter precoder) from one dominant interference BS to the total interference power plus noise received by AMS. The AMS may accordingly request the preferred multi-BS MIMO scheme. The operation procedure of the multi-BS MIMO trigger follows the steps below.

1)AMS in normal process of Single-Cell CL MIMO feeds back information for CL MIMO operation.

2)ABS selects a NIP threshold NIP_th_1 for single-BS precoding with multi-BS coordination, a threshold NIP_th_2 for multi-BS joint MIMO processing and a common CINR_th based on network measurements. Then the ABS broadcasts the NIP thresholds NIP_th_1, NIP_th_2 and CINR_th in AAI_DL_IM message.

3)AMS computes its average NIP and checks the trigger conditions. If NIP ≥ NIP_th_1 and

NIP < NIP_th_2 and CINR < CINR_th, AMS may request operation with single-BS precoding with multi-BS coordination using the AAI_MULTI_BS_MIMO-REQ message.

a)AMS feedback its event-driven request to operate single-BS precoding with multi-BS coordination to its serving BS through AAI_MULTI_BS_MIMO-REQ which includes a 2bit NIP difference measurement with respect to NIP_th_1.

b)Once an AMS receives a Feedback Polling A-MAP IE, the procedure outlined in 16.5.1.2.1 will be followed.

4)If  NIP ≥ NIP_th2 and CINR < CINR_th, AMS may request operation with multi-BS joint MIMO processing using the AAI_MULTI_BS_MIMO-REQ message.

a)AMS feedback its event-driven request to operate with multi-BS joint MIMO processing to its serving BS through AAI_MULTI_BS_MIMO-REQ which includes a 2bit NIP difference measurement with respect to NIP_th_2.

b)Once an AMS receives a Feedback Polling A-MAP IE, the procedure outlined in 16.5.1.3.1 will be followed.

------------------------------------------------------  Text 2 End  ---------------------------------------------------------------

## Proposed Text 2 ##

<< [Add the following row in Table 704, Page 96-97] >>

------------------------------------------------------  Text 3 Start  ---------------------------------------------------------------

Table 704—Parameters for AAI_DL-IM Message

| Name | Value | Usage |
| --- | --- | --- |
| Resource_Metric_FP2 | TBD |  |
| Resource_Metric_FP3 | TBD |  |
| BC_SI | 8 bits for Nt = 2 and 16 bits for Nt = 4 or 8 |  |
| NIP_th_1 | TBD | NIP threshold for Single BS precoding with Multi-BS Coordination trigger |
| NIP_th_2 | TBD | NIP threshold for DL Multi-BS Joint MIMO Processing trigger |
| CINR_th | TBD | Used together with NIP_th_1 or NIP_th_2 for Multi-BS MIMO trigger |
| FP1_power | TBD |  |
| FP2_power | TBD |  |
| : | : | : |

------------------------------------------------------  Text 3 End  ---------------------------------------------------------------

## Proposed Text 3 ##

<< [Modify the caption of Table 707, Page 102] >>

------------------------------------------------------  Text 4 Start  ---------------------------------------------------------------

Table 707—Parameters for AAI_MULTI_BS_MIMO-REQ message format

| Name | Value | Usage |
| --- | --- | --- |
| : | : | : |

------------------------------------------------------  Text 4 End  ---------------------------------------------------------------

# References

[1] 	Draft Amendment to IEEE Standard for Local and Metropolitan Area Networks, Part 16: Air Interface for 	Broadband Wireless Access Systems, P802.16m/D3 December 2009.