| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed AAI_NBR-ADV message format (16.2.3.12) |  |
| Date Submitted | 2009-12-26 |  |
| Source(s) | Jaehyuk Jang, Hyunjeong Kang, Jung Je Son, Rakesh Taori Samsung Electronics | E-mail: jack.jang@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-09/0073: IEEE 802.16 Working Group Letter Ballot Recirc #30b Section 16.2.3.12 AAI_NBR-ADV |  |
| Abstract | This contribution provides detailed format of the AAI_NBR-ADV message |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed AAI_NBR-ADV message format (16.2.3.12)

Jaehyuk Jang, Hyunjeong Kang, Jung Je Son, Rakesh Taori
Samsung Electronics

# Introduction

This contribution provides detailed format of the AAI_NBR-ADV message.

D3 text states “for Wireless-MAN-OFDMA reference system, only 48-bit BS-ID and Preamble index are included in AAI_NBR-ADV,” but this is not implemented in the message format. Most of fields for ABSs in the AAI_NBR-ADV are not needed for YBSs, so we suggest using additional cell type for the legacy BSs.

And, even for multi-carrier BSs, MAC version is only one. This should be fixed.

In addition, two values in SFH_encoding_format which was in D2 are accidentally deleted in D3.

# Proposed Text

Update the following section in D3.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

16.2 Medium access control

16.2.3 AAI MAC Management Messages

16.2.3.12 AAI_NBR-ADV

AAI_NBR-ADV message may sort neighbor ABSs (RSs) according to their deployment types, which is categorized by the following parameters:

- ABS type (macro, micro, macro hotzone, Femto, relay, legacy/LZoneTBD)

- carrier frequency

- MAC version

- TDD/FDD and related definitions (expected to be the same given carrier frequency)

- BW, CP info: may not be required if carried in P/S-SCH channel

- Multicarrier capability and configuration

…

Table 684—AAI_NBR-ADV message format

| Field | Size (bit) | Description |
| --- | --- | --- |
| AAI_NBR-ADV_Message_format() { | — | — |
| Management Message Type = NN | 8 | — |
| Change Count | 3 | NBR-ADV change count |
| Number of cell types | 3 | — |
| Cell type in this message | 3 | Cell type in this message 0b000 macro 0b001 micro 0b010 macro hotzone 0b011 femto 0b100 relay 0b101 legacy/LZone 0b110-0b111 reserved |
| Total Number of Segments | TBD4 | Number of segments for this cell type |
| Segment Index | TBD4 | Segment index of this message in the specific cell type |
| BS number M | TBD8 | Total number of BSs to be included in this segment |
| Multi-carrier configuration change count | 4 | Refer to the change count in AAI_MC-ADV |
| If (Cell type == 0b101) { |  |  |
| For (i=0; i<M; i++) { | — | — |
| BSID | 48 |  |
| FA Index | 8 |  |
| Preamble Index/Subchannel Index | 8 | For the OFDMA PHY this parameter defines the PHY specific preamble. For OFDMA PHY, bit 7 is used to indicate the reuse factor of the neighbor for purpose of CINR measurement for handoff. A value of '0' indicates a reuse factor of 1 and a value of '1' indicates reuse factor of 3. For OFDMA PHY, bit 6 to bit 0 represent the preamble index of the neighbor BS. |
| DCD Configuration Change Count | 4 | This represents the 4 LSBs of the neighbor BS current DCD configuration change count. |
| UCD Configuration Change Count | 4 | This represents the 4 LSBs of the neighbor BS current UCD configuration change count. |
| } |  |  |
| } else { | — | — |
| For (i=0; i<M; i++) { | — | — |
| Index of BS | 8 | The index of the BS in the whole AAI_NBR-ADV |
| BSID | 48 | — |
| Number of carriers (NC) | 3 | Number of carriers of the BS |
| For ( j=0; j<NC; j++) { | — | — |
| A-PREAMBLE index | TBD10 | — |
| A-PREAMBLE transmit power | 8 | — |
| PA-Preamble transmit power | 6 | Values indicate power levels in 1dB steps starting from 0 dBm |
| SA-Preamble transmit power | 6 | Values indicate power levels in 1 dB steps starting from 0 dBm |
| Physical carrier index | 6 | Refer to the change count in AAI_MC-ADV |
| Number of MAC protocol versions supported: P | 4 | Greenfield: 1 Mixed-mode: >1 |
| for (i=0; i<P; i++) { | — | — |
| MAC protocol version | 4 | Consistent with REV.2 definition, with new MAC protocol version 9 defined for 16m. |
| } | — | — |
| MAC protocol version | 8 | MAC protocol version of the BS Consistent with REV.2 definition, with new MAC protocol version 9 defined for 16m. |
| } | — | — |
| SFH_encoding_format | 2 | 0b00: full Subpkt information 0b01: delta encoding with TBD format (the 1st BS in this cell type shall use full Subpkt encoding ) 0b10: no SFH included 0b11: reserved For macrocell ABS, the bitmap shall be either 0b00 or 0b01 |
| Control_bitmap | 4 | Each bit maps to one SFH subpacket or extended broadcast information. For macrocell ABS, the bitmap shall be 1111 |
| If( SFH_encoding_format =00) { | — | //encoding format type-1 |
| If( Control_bitmap[0] ==1 ){ | — | — |
| SFH Subpkt 1 | TBD | //exclude those fields already in cell type info |
| } | — | — |
| If( Control_bitmap[1] ==1 ){ | — | — |
| SFH Subpkt 2 | TBD | //exclude those fields already in cell type info |
| } | — | — |
| If( Control_bitmap[2] ==1 ){ | — | — |
| SFH Subpkt 3 | TBD | //exclude those fields already in cell type info |
| } | — | — |
| } | — | — |
| If( SFH_encoding_format =01) { | — | — |
| Delta information | — | Delta encoding, w.r.t. the reference BS |
| } | — | — |
| } | — | — |
| } | — | — |
| } | — | — |

------------------------------------------------------ End of Proposed Text ------------------------------------------------------