| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed BR header format (16.2.2.1.3) |  |
| Date Submitted | 2009-12-26 |  |
| Source(s) | Jaehyuk Jang, Jeongho Park, Yeongmoon Son, Hyunjeong Kang, Jung Je Son, Rakesh Taori Samsung Electronics | E-mail: jack.jang@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-09/0073: IEEE 802.16 Working Group Letter Ballot Recirc #30b Section 16.2.2.1.3 MAC Signaling Header |  |
| Abstract | This contribution provides detailed format of the BR header. |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed BR header format (16.2.2.1.3)

Jaehyuk Jang, Jeongho Park, Yeongmoon Son, Hyunjeong Kang, Jung Je Son, Rakesh Taori
Samsung Electronics

# Introduction

This contribution provides detailed format of the Bandwidth Request (BR) header.

802.16-2009 supports two types of BW request: One is an aggregate type which is supported in D3, and the other is an incremental type which is NOT in D3. To report uplink transmission power during BR, an incremental BR type which saves 8 bits also needs to be supported in the 16m specification.

# Proposed Text

Update the following sections in D3.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

16.2 Medium access control

16.2.2.1.3 MAC Signaling Header

...

Table 656—Type field encodings for MAC signaling header type

| Type field (4 bits) | MAC Signaling Header Type |
| --- | --- |
| 0000 | BR aggregate with STID |
| 0001 | BR incremental with STID |
| 00010010 | BR without STID |
| 00100011 | Service specific BR without STID |
| 00110100 | Sleep Control |
| 0100- 1111 | Reserved |

16.2.2.1.3.1 BR aggregate with STID Header

BR format with STID header is defined in Table 657.

Table 657—BR aggregate with STID header format

| Syntax | Size(bit) | Notes |
| --- | --- | --- |
| BR with STID () { |  |  |
| FID | 4 | Flow Identifier. This field indicates MAC signaling header. |
| Type | 4 | MAC signaling header type = 0b0000 |
| BR Size | 19 | Aggregated bandwidth request size in bytes |
| BR FID | 4 | The FID for which UL bandwidth is requested. |
| STID | 12 | STID of the AMS which requests UL bandwidth. |
| Reserved | 5 | Reserved. This field shall be filled by 0 |
| } |  |  |

16.2.2.1.3.2 BR incremental with STID Header

BR format with STID header is defined in Table 657.

Table 658—BR incremental with STID header format

| Syntax | Size(bit) | Notes |
| --- | --- | --- |
| BR with STID () { |  |  |
| FID | 4 | Flow Identifier. This field indicates MAC signaling header. |
| Type | 4 | MAC signaling header type = 0b0001 |
| BR Size | 11 | Incremental bandwidth request size in bytes |
| BR FID | 4 | The FID for which UL bandwidth is requested. |
| STID | 12 | STID of the AMS which requests UL bandwidth. |
| TI | 1 | 0: No Tx Power report 1: Tx Power information present |
| If (TI==1) { |  |  |
| UL Tx Power | 8 | UL Tx power level in dBm for the burst that carries this header (as described in 11.1.1). |
| Reserved | 4 | Shall be filled by 0 |
| } else { |  |  |
| Reserved | 12 | Shall be filled by 0 |
| } |  |  |
| } |  |  |

{To editor : please update the following section numbers}

16.2.2.1.3.23 BR without STID Header

16.2.2.1.3.34 Service Specific BR without STID Header

16.2.2.1.3.45 Sleep Control header (SCH)

------------------------------------------------------ End of Proposed Text ------------------------------------------------------