| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on broadcast control message signaling (16.2.10/16.2.1) |  |
| Date Submitted | 2009-01-06 |  |
| Source(s) | Daniel Cohn Alvarion Philip Barber Huawei Kelvin Chou, Yih-Shen Chen, I-Kang Fu, Paul Cheng MediaTek Inc. | daniel.cohn@alvarion.com pbarber@huawei.com kelvin.chou@mediatek.com |
| Re: | Letter Ballot #30b on the Draft Amendment (IEEE P802.16m/D3) |  |
| Abstract | Clarification on STID and FID to be used in broadcast messages |  |
| Purpose | To be discussed and adopted by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on broadcast control message signaling (16.2.10/16.2.1)

Daniel Cohn
 Alvarion

Phil Barber
 Huawei

Kelvin Chou, Yih-Shen Chen, I-Kang Fu, Paul Cheng
MediaTek Inc

Problem Statement

The standard (802.16m/D3) does not specify which STID and FID should be used in PDUs transmitted in the broadcast connection.

Only in the particular case of the AAI_RNG-ACK message does the standard specify the “broadcast STID” should be used, but the value of this STID remains undefined.

Proposed Solution

All PDUs transmitted in the broadcast connection should be use the reserved 0xFFF STID. When FID is required, the unicast control connection FID should be used.

Suggested Standard Modifications

1.- Add new section 16.2.10.3 as follows:

16.2.10.3 Broadcast Control Connections

Broadcast control messages are control messages transmitted by the ABS intended for reception by all AMS that may be listening, not to any specific AMS. Broadcast control messages shall be transmitted to a uniquely reserved STID value of 0xFFF which shall not be assigned to any individual AMS or used for any other purpose. When FID is required to be present in a broadcast control message, the FID value shall be set to the unicast control connection value (see Table 652).

2.- Modify section 16.2.1.2.1 as follows:

The ABS shall assign a 12 bit STID to each AMS during network entry <15.2.15>. The STID shall uniquely identify the AMS in the domain of the ABS. An AMS may be assigned a new STID during network reentry.

The STID value of 0xFFF is reserved for broadcast control messages.

3.- Modify table 652 in section 16.2.1.2.2, page 19 as:

Table 652 – Flow Identifiers

| Value | Description |
| --- | --- |
| 0000 | Basic Management FID (unicast control FID when in conjunction with the AMS unique STID; broadcast control FID when in conjunction with the reserved broadcast STID) |
| 0001 | Primary Management FID |
| 0010 | Signaling header indicator |
| 0011 | Pre provisioned Transport connection for BE Service |
| 0100 - 1111 | Transport FID |