| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Support for neighbor-specific handover triggers (16.2.6) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Daniel Cohn Alvarion Joseph Schumacher Motorola Muthaiah Venkatachalam Intel | daniel.cohn@alvarion.com j.schumacher@motorola.com muthaiah.venkatachalam@intel.com |
| Re: | Letter Ballot #30b on the Draft Amendment (IEEE P802.16m/D3) |  |
| Abstract | This contribution adds support for neighbor-specific handover triggers |  |
| Purpose | To be discussed and adopted by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Support for neighbor-specific handover triggers (16.2.6)

Daniel Cohn

Alvarion

Joseph Schumacher

Motorola

Muthaiah Venkatachalam

Intel

Problem Statement

One of the phases of a wireless network deployment is the optimization phase. This activity involves modifying the values of system parameters to achieve optimum network coverage and throughput. A key part of the network optimization phase involves fine tuning handover triggers. The optimal handover trigger value often differs depending on the potential target network base station, e.g. as a function of the overlap area between the base stations coverage or mobility environment.

For example there can be a case where transitions from BS1 to BS2 occur at a highway while transitions from BS1 to BS3 occur in an urban, pedestrian area. In this case it will be beneficial for the operator to optimize handover triggers for speed and for stability respectively. In other cases the operator will want to prevent MSs from handing over to a specific neighbor BS altogether.

IEEE 802.16-2009 supports the definition of neighbor-specific handover triggers, i.e. triggers that specify when the MS should handover to a specific neighbor base station. IEEE 802.16m/D3 lacks this support.

Proposed Solution

This contribution adds support for neighbor-specific triggers in IEEE 802.16m/D3 by adding an optional “neighbor-specific BS triggers” TLV to the AAI_NBR-ADV message where the ABS can optionally broadcast handover triggers that apply only to a specific neighbor.

Suggested Standard Modifications

1.- Modify table 684 - AAI_NBR-ADV message format as follows.

Table 684—AAI_NBR-ADV message format

| Syntax | Size (bit) | Note |
| --- | --- | --- |
| AAI_NBR-ADV_Message_format() { | — | — |
| Management Message Type = NN | 8 | — |
| Change Count | 3 | NBR-ADV change count |
| Number of cell types | 3 |  |
| Cell type in this message | 3 |  |
| Total Number of Segments | TBD | Number of segments for this cell type |
| Segment Index | TBD | Segment index of this message in the specific cell type |
| BS number M | TBD | Total number of BSs to be included in this segment |
| Multi-carrier configuration change count | 4 | Refer to the change count in AAI_MC-adv |
| for (i=0; i<M; i++){ |  |  |
| Index of BS | 8 | The index of the BS in the whole AAI_NBR-ADV |
| BSID | 48 |  |
| Number of carriers (NC) |  |  |
| for( j=0; j<NC; j++) { |  |  |
| A-PREAMBLE index | TBD |  |
| A-PREAMBLE transmit power | 8 |  |
| Physical carrier index | 6 | Refer to the change count in AAI_MC-ADV |
| Number of MAC protocol versions supported:P | 4 | Greenfield: 1 Mixed-mode: >1 |
| For (i=0; i<P; i++) { |  |  |
| MAC protocol version | 4 | Consistent with REV.2 definition, with new MAC protocol version 9 defined for 16m. |
| } |  |  |
| } |  |  |
| SFH_encoding_format | 2 | 00: full Subpkt information 01: delta encoding with TBD format (the 1st BS in this cell type shall use full Subpkt encoding ) |
| Control_bitmap | 4 | Each bit maps to one SFH subpacket or extended broadcast information. For macrocell ABS, the bitmap shall be 1111 |
| If( SFH_encoding_format =00{ |  | //encoding format type-1 |
| If( Control_bitmap[0] ==1 ){ |  |  |
| Subpkt 1 | TBD | //exclude those fields already in cell type info |
| } |  |  |
| If( Control_bitmap[1] ==1 ) { |  |  |
| Subpkt2 | TBD | //exclude those fields already in cell type info |
| } |  |  |
| If( Control_bitmap[2] ==1 ) { |  |  |
| Subpkt3 | TBD | //exclude those fields already in cell type info |
| } |  |  |
| } |  |  |
| If( SFH_encoding_format =01) { |  |  |
| Delta information |  | Delta encoding, w.r.t. the reference BS |
| } |  |  |
| Neighbor-specific trigger TLVs | Variable | Optional neighbor-specific triggers with encoding defined in table 731 |
| } |  |  |
| } |  |  |

2.- Add the following text in section 16.2.6.2, page 161 after table 732:

The ABS may define neighbor-specific triggers by including neighbor-specific trigger TLVs in the AAI_NBR-ADV message. The AMS evaluates neighbor-specific triggers only for the specific neighbor BS metric.

Neighbor-specific trigger TLVs use the format in table 731, where only function types  0x1, 0x2, 0x3 and 0x4 and actions types 0x1 and 0x2 are allowed. When present, neighbor-specific handover triggers override any general trigger TLVs (define in the AAI_SCD message) of the same type, function and action.