| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Interference Avoidance and Interference Mitigation for Femto ABS (16.4.11) |  |
| Date Submitted | 2010/1/12 |  |
| Source(s) | Yoshikazu Watanabe, Yasuhiro Mizukoshi, Linghang Fan, Nader Zein, Tetsu Ikeda NEC | Voice: +81443963530 E-mail: y-watanabe@fa.jp.nec.com |
| Re: | LB comment to 802.16m Amendment Working Document D3 P802.16m/D3 Femto |  |
| Abstract | This contribution proposes text for the Draft Amendment regarding interference avoidance and interference mitigation for Femto ABS. |  |
| Purpose | To be discussed and adopted by TGm for the P802.16m/D4 Amendment text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Interference Avoidance and Interference Mitigation for Femto ABS (16.4.11)

Yoshikazu Watanabe, Yasuhiro Mizukoshi, Linghang Fan, Nader Zein, Tetsu Ikeda

NEC

# Introduction

This contribution proposes text for the Draft Amendment [1] regarding interference avoidance and interference mitigation for Femto ABS. When a Femto ABS is located within an RS cell, it is useful for the Femto ABS to know the relay zone configuration which the ARS uses in order to avoid and mitigate interference between the overlay RS cell and the femto cell.

# Discussion

The frame structure for ARS is divided into 16m Access zone and 16m Relay zone as described in section 16.6.3.1 of D3. An ARS communicates with subordinate AMSs using 16m Access zone. An ARS communicates with superordinate ABS using 16m Relay zone. In case of DL, the ARS is receiving data from the ABS and not transmitting to the AMSs during DL relay zones. When a Femto ABS is located within an RS cell, less interference between the RS cell and the femto cell is expected in relay zones than in access zones. Therefore it is useful for the Femt ABS to know the relay zone configuration of the surrounding ARS. The Femto ABS can preferentially use the subframes corresponding to the relay zones in order to avoid and mitigate the interference.

# Proposed Text

Modify the text in 16.4.11 as follows.

-------------------------------------------------- Start of the proposed Text --------------------------------------------------

# 16.4.11 Interference Avoidance and Interference Mitigation

An AMS may be requested by its serving macro ABS or Femto ABS to report the signal strength measurement of neighbor ABSs, including macro and/or Femto BSs. The reported information can be used by the serving ABS to coordinate with its neighbor BSs to mitigate the interference at the MSs.

Large interference from an inaccessible Femto ABS may trigger a nearby AMS to report the interference to the serving ABS, and the report information should include system information of the inaccessible Femto ABS (e.g., BS_ID of the Femto ABS).

The serving ABS and/or the network may request the interfering Femto ABS to mitigate the interference by reducing transmission power, and/or blocking some resource region.

In order to enable the interference avoidance or mitigation schemes, the Femto ABS may be capable of scanning the signals transmitted from neighbor ABSs.

The interference between Femto and/or macro can be mitigated by static or semi-static radio resource reservation and resource sharing using FDM and/or TDM manner and/or DL power control. The operation of resource reservation shall not contradict with the FFR operation defined in 20.1. Femto ABSs shall also utilize FFR partitions in the DL and UL using the same signaling and procedures as used by the macro ABSs. FFR partitions used by the Femto ABSs may be different in terms of size, subchannel assignment, and transmit power level than those used on macro ABSs. One or more FFR partitions may be used as the radio resource region where Femto nonABSs are not allowed to transmit. The blocked region size may increase if the number of AMSs interfered by Femto ABS becomes larger ; and the blocked region size may decrease if the number of AMSs interfered by Femto ABS becomes smaller. A Femto ABS may detect and reserve the resources autonomously, or in cooperation with the overlay macro ABS. A Femto ABS may obtain the relay zone configuration used in the system, and use the subframes corresponding to the relay zones preferentially.

A Femto ABS may select the carrier frequency, or frequency partition to avoid the mutual interference between macro/micro and Femto ABSs or among Femto ABSs based on the measurement result of surrounding reception power.

Femto ABS may measure the signal strength for the carrier frequency, or frequency partition of the neighbor macro/micro/femto cells. In addition, the Femto ABS may receive A-Preamble from the neighbor macro/micro/femto cells and obtain information on cell type. The Femto ABS may select its carrier frequency, or frequency partition based on signal strength and the cell type information of its neighbors.

-------------------------------------------------- End of the proposed Text ---------------------------------------------------

# References

[1]	IEEE P802.16m/D3 Draft