| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Extended Header Format |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Joey Chou Intel | E-mail: joey.chou@intel.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text for Extended Header Format. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Extended Header Format

- Joey Chou,

- Intel

- Introduction

Table 661 does not reflect all components in Figure 386, and is not consistent with the header definition format.

- Proposed text

---------------------------------------- Start of proposed text ----------------------------------------------

16.2.2.2 	Extended header formats

Table 661—Extended Header Format Group Fields

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Extended Header Format() { |  |  |
| Extended Header Group Length | 8 | The Extended header Group Length field indicates the total length of the extended header group, including all the extended headers and the Extended header Group length byte. |
| For (i = 0 ; i < n ; i++) ( | 4 | i = the number of extended headers |
| Extended Hader Type | 4 | Type of extended header as defined in Table 662. |
| Extended header Body | variable | The size of the extended header is determined by extended header type as specified in Table 662. The extended header including the extended header type is byte aligned. |
| } |  |  |
| } |  |  |

---------------------------------------- End of proposed text ----------------------------------------------