| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes for OL region at 802.16m/D3 (Section 16.3.5.3.1) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Huaning Niu, JK Fwu, Alexei Davydov, Yuan Zhu, Yang-Seok Choi Intel | huaning.niu@intel.com |
| Re: | IEEE 802.16m/D3 Section 16.3.5.3.1 |  |
| Abstract | The contribution provides permutation changes for OL region support |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D4 Draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes for OL region at 802.16m/D3 (Section 16.3.5.3.1)

Huaning Niu, JK Fwu, Alexei Davydov, Yuan Zhu, Yang-Seok Choi

Intel

- Introduction

This contribution proposes OL region related changes on permutation and signaling.

---------------------------------------------------------------------------------------

--------------------------------------------- Proposed Text 1 for Starts -----------------------------------------------

Modify the text and equation in Section 16.3.5.3.1

[Page 329, Line 17]

The mapping from PRUFPi to CRUFPi (for 0 ≤ i < FPCT) is given by

CRUFPi[j]=PRUFPi[j],0≤j<LSB−CRU,FPiPRUFPi[k+LSB−CRU,FPi]LSB−CRU,FPi≤j<LCRU,FPi

CRUFPi[j]=PRUFPi[j]0≤j<LCRU,FPi                      (193)

where k = s[j – LSB-CRU,FPi].

s[] is the CRU/DRU allocation sequence defined in Equation (194) and 0 ≤ s[j] < FPSi – LSB-CRU,FPi.

s[j]=PermSeq(j)+DL_PermBasemod(FPSi−LSB−CRU,FPi)                         (194)

In Equation (194), PermSeq() is the permutation sequence of length (FPSi- LSB-CRU,FPi) and is determined by SEED = {IDcell*343} mod 210 . The permutation sequence is generated by the random sequence generation algorithm specified in 16.3.5.3.3. DL_PermBase is set to preamble IDcell.

The mapping of CRU to SLRU is given by

SLRUFPi[j]=CRUFPi[j]0≤j<LSB−CRU,FPi                         (194)

The mapping of CRU to NLRU is given by

NLRUFPi[j]=CRUFPi[k+LSB−CRU,FPi]LSB−CRU,FPi≤j<LCRU,FPi−LDRU,FPi          (195)

where k = s[j – LSB-CRU,FPi].

s[] is the NLRU allocation sequence defined in Equation (196) and 0 ≤ s[j] < FPSi –LSB-CRU,FPi- LDRU,FPi.

s[j]=PermSeq(j)+DL_PermBasemod(FPSi−LSB−CRU,FPi−LDRU,FPi)            (196)

In Equation (196), PermSeq() is the permutation sequence of length (FPSi-LSB-CRU,FPi- LDRU,FPi) and is determined by SEED = {IDcell*343} mod 210. DL_PermBase is set to preamble IDcell.

The mapping of PRUFPi to DRUFPi is given by:

DRUFPi[j]=PRUFPi[k+LSB−CRU,FPi], 0≤j<LDRU,FPi               (195)

where k = s[j + LCRU,FPi – LSB-CRU,FPi].

DRUFPi[j]=PRUFPi[j+LSB−CRU,FPi], 0≤j<LDRU,FPi			(197)

------------------------------------------------- Proposed Text 1 Ends -------------------------------------------------

--------------------------------------------- Proposed Text 2 for Starts -----------------------------------------------

Modify the text in Section 16.2.3.29, page 94, line 46

16.2.3.29 AAI_System Configuration Descriptor (SCD) Message

An AAI_SCD shall be transmitted by the ABS at a periodic interval to define a system configuration.

-- ASN1START

AAI_SCDMessage :: SEQUENCE {

Change Configuration ChangeINTEGER (0 ~ 15)

BS_Restart_CountINTEGER (0 ~ 15)

SA_PreamblePartitionforBStype

Trigger TLV encoding :: = SEQUENCE{}

DefaultHORSSI_CINRaveraggingparameters :: = SEQUENCE {}

NormalizedCINR :: = SEQUENCE {}

OL_MIMO_Parameters :: = SEQUENCE {

OL-Region-ONINTEGER (0..1) OPTIONAL

OL-Rank1-ConfigurationINTEGER (0..7) OPTIONAL

SB-OL-Region-2SizeINTEGER (0..15) OPTIONAL

}

MIMO_OL_Region _Parameters :: = SEQUENCE {

OL-Region-Type0-ON INTEGER (0,1) OPTIONAL

OL-Region-Type1-NLRU-ON INTEGER (0,1) OPTIONAL

OL-Region-Type1-SLRU-Size INTEGER (0..15) OPTIONAL

OL-Region-Type2-SLRU-Size INTEGER (0..15) OPTIONAL

}

Parameters_GRA :: = SEQUENCE {}

PeriodicRNGParameters :: = SEQUENCE {}

GAMMA_IOT_FP0INTEGER (0..15) OPTIONAL

GAMMA_IOT_FP1INTEGER (0..15) OPTIONAL

GAMMA_IOT_FP2INTEGER (0..15) OPTIONAL

GAMMA_IOT_FP3INTEGER (0..15) OPTIONAL

Alpha (a)INTEGER (0.. 7) OPTIONAL

Beta (ß)INTEGER (0..1) OPTIONAL

SINRminINTERGER (0..15) OPTIONAL

BR_Channel Configuration MIN Access ClassINTERGER (0..4) OPTIONAL

Sounding_multiplexing_type CHOICE {

fdmFDM

cdmCDM

}

Decimation_value_D/Max_Cyclic_Shift_Index_P_for_soundingENUMERATED {n4, n9, n16,

n18, n36}

RELAY_Support_Parameters :: = SEQUENCE {

16m_Relay_zone_AMS_allocation_indicatorINTEGER (0..1) OPTIONAL

}

}

-- ASN1STOP

------------------------------------------------- Proposed Text 2 Ends -------------------------------------------------

--------------------------------------------- Proposed Text 3 for Starts -----------------------------------------------

Modify the text in Section 16.3.7.2.5.1, page 455, line 1

Dynamic switching between MIMO mode 1 and MIMO mode 3 in downlink transmissions in OL region type 2 is allowed. The rank-2 precoders for transmission with MIMO mode 1 or MIMO mode 3 in OL region type 2 are the same on a given subband.

The OL region type 0 is present if OL-Region-Type0-ON is indicated in a downlink broadcast message AAI_SCD message. All DRUs in frequency partition 0 will be in OL region type 0.

The OL region type 1 NLRU is present if OL-Region-Type1-NLRU-ON is indicated in a downlink broadcast message AAI_SCD message. All NLRUs in frequency partition 0 will be in OL region type 1. The OL region type 1 SLRU is present if OL-Region-Type1-SLRU-Size is greater than 0 in a downlink broadcast message AAI_SCD message. The PRUs allocated to type 1 OL region SLRU are the are the first (4* OL-Region-Type1-SLRU-Size) PRUs obtained by the mapping equation (187), i.e., PRUFP0(j), where j = 0 to (4 * OL-Region-Type1-SLRU-Size) -1.

The OL region type 2 SLRU is present if OL-Region-Type2-SLRU-Size is greater than 0 in a downlink broadcast message AAI_SCD message. The PRUs allocated to type 2 OL region are the (4* OL-Region-Type2-SLRU-Size) PRUs obtained by the mapping equation (187), i.e., PRUFP0(j), where j = (4 * OL-Region-Type1-SLRU-Size) to (4 * OL-Region-Type1-SLRU-Size) + (4 * OL-Region-Type2-SLRU-Size) -1.

All base stations that are coordinated over the same open loop region should use the same number of MIMO streams, in order to guarantee low interference fluctuation and thus improve the CQI prediction at the AMS. All pilots are precoded by non-adaptive precoding with MaxMt MIMO streams in the open-loop region. CQI measurements should be taken by the AMS on the precoded demodulation pilots rather than on the downlink midamble.

------------------------------------------------- Proposed Text 3 Ends -------------------------------------------------