| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes for OL region at 802.16m/D3 (Section 16.3.5.3.1) |  |
| Date Submitted | 2010-01-09 |  |
| Source(s) | Huaning Niu, JK Fwu, Alexei Davydov, Yuan Zhu, Yang-Seok Choi Intel Wookbong Lee, HanGyu Cho, Bin-Chul Ihm LG Electronics | huaning.niu@intel.com wookbong.lee@lge.com |
| Re: | Call for comments for Letter Ballot 30b |  |
| Abstract | The contribution provides permutation changes for OL region support |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D4 Draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes for OL region at 802.16m/D3 (Section 16.3.5.3.1)

Huaning Niu, JK Fwu, Alexei Davydov, Yuan Zhu, Yang-Seok Choi

Intel

Wookbong Lee, HanGyu Cho, Bin-Chul Ihm

LG Electronics

- Introduction

This contribution proposes OL region related changes on permutation and signaling.

- Motivation/Rationale

An open-loop region is defined as a time-frequency resource using same number of pilot pattern and a given open-loop MIMO mode. The open-loop region allows base stations to coordinate their open-loop MIMO transmissions, in order to offer a stable interference environment where the precoders and number of MIMO streams are not time-varying.

Currently there are three kinds of open-loop region, OL region type 0, OL region type 1, and OL region type 2.

The following table shows details of OL region.

Types of open-loop regions

|  | MaxMt | MIMO Mode | MIMO feedback mode | Supported permutation |
| --- | --- | --- | --- | --- |
| OL Region type 0 | 2 streams | MIMO mode 0 MIMO mode 1 (Mt = 2 streams) | 0 | DRU |
| OL Region type 1 | 1 streams | MIMO mode 5 (Mt = 1 stream) | 1 | Miniband based CRU (diversity allocation) |
|  |  |  | 2 | Subband based CRU (localized allocation) |
| OL Region type 2 | 2 streams | MIMO mode 1 (Mt = 2 streams) MIMO mode 3 (Mt = 2 streams) | 5 | Subband based CRU (localized allocation) |

To enable the open-loop region, we need to make sure the resource units assigned for OL region shall be aligned across cells.

Since subbands are already aligned across cells, there is no problem for OL Region type 1 subband mode and OL Region type 2.

However, current subchannelization rule can’t make the OL region alignment across cell for OL Region type 0 and OL Region type 1 miniband mode since different cells have different number of subbands, minibands, or DRU.

Moreover, even though each cell has same number of subbands, minibands and DRU, the location of minibands and DRUs are not aligned due to sector/cell specific CRU/DRU allocation sequence applied for miniband and DRU mapping. (See equation (194) for more details)

This proposal is to make OL region aligned across cells for OL region type 1 miniband mode and OL region type 0.

To minimize additional complexity due to OL region, we propose to reserve number of PRUs for DLRU for subchannelization. The performance of DLRU will be almost maintained since we have another permutation for DLRU (subcarrier permutation).

Here is summary of proposed OL Region operation.

|  | Subchannelization | SCD message | Feedback Allocation/Polling A-MAP IE |
| --- | --- | --- | --- |
| Without any type of OL Region | Reserve PRUs for SLRU and DLRU before CRU/DRU permutation | Indicates there is no OL Region | Indicates measurement shall be done in midamble |
| With OL Region type 1 (SLRU mode) and/or OL Region type 2 |  | Indicates number of subbands for OL Region type 1 (SLRU) and/or OL Region type 2 | Indicates measurement shall be done in OL Region type 2 SLRU to OL MU MS (OL Region type 2) and OL Region type 1 SLRU to CDR MS (OL Region type 1 SLRU) |
| With OL Region type 0 and/or OL Region type 1 (NLRU) |  | Indicates whether OL Region type 0 and/or OL Region type 1 (NLRU) is on or not | Indicates measurement shall be done in DLRU to OL SU MS (OL Region type 0) and minibands to CDR MS (OL Region type 1 NLRU) |

- References

[1] IEEE P802.16m/D3, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems”

---------------------------------------------------------------------------------------

Instruction to editor;

Black text: existing D2 text

Red and strike out text: for deletion

Blue and underline text: for addition

--------------------------------------------- Proposed Text 1 for Starts -----------------------------------------------

Modify the text in lines 10-42 in page 329 in 16.3.5.3.1 as follows

The number of DRUs in each frequency partition is denoted by LDRU,FPi, where

LDRU,FPi =FPSi-LCRU,FPi 	for 0≤ i <FPCT 			(192)

And FPSi is the number of PRUs allocated to FPi.

The mapping from PRUFPi to CRUFPi (for 0≤ i <FPCT) is given by:

CRUFPi[j]=PRUFPi[j]    0≤j<LSB−CRU,FPiPRUFPi[k+LSB−CRU,FPi]    LSB−CRU,FPi≤j<LCRU,FPi	(193)

where k=s[j−LSB−CRU,FPi].

s[] is the CRU/DRU allocation sequence defined in Equation (194) and 0≤s[j]<FPSi−LSB−CRU,FPi 0≤s[j]<FPSi−LSB−CRU,FPi−LDRU,FPi.

s[j]=PermSeq(j)+DLPermBasemodFPSi−LSB−CRU,FPi s[j]=PermSeq(j)+DLPermBasemodFPSi−LSB−CRU,FPi−LDRU,FPi (194)

In Equation (194), PermSeq() is the permutation sequence of length (FPSi-LSB-CRU,FPi-LDRU,FPi) and is determined by SEED = {IDcell*343} mod 210. The permutation sequence is generated by the random sequence generation algorithm specified in 16.3.5.3.3. DL_PermBase is set to preamble IDcell.

The mapping of PRUFPi to DRUFPi is given by:

DRUFPi[j]=PRUFPi[k+LSB−CRU,FPi],0≤j<LDRU,FPi  DRUFPi[j]=PRUFPi[j],FPSi−LDRU,FPi≤j<FPSi (195)

where k = s[j + LCRU,FPi -LSB-CRU,FPi].

------------------------------------------------- Proposed Text 1 Ends -------------------------------------------------

--------------------------------------------- Proposed Text 2 for Starts -----------------------------------------------

Modify the text in Section 16.2.3.29, page 94, line 46

16.2.3.29 AAI_System Configuration Descriptor (SCD) Message

An AAI_SCD shall be transmitted by the ABS at a periodic interval to define a system configuration.

-- ASN1START

AAI_SCDMessage :: SEQUENCE {

Change Configuration ChangeINTEGER (0 ~ 15)

BS_Restart_CountINTEGER (0 ~ 15)

SA_PreamblePartitionforBStype

Trigger TLV encoding :: = SEQUENCE{}

DefaultHORSSI_CINRaveraggingparameters :: = SEQUENCE {}

NormalizedCINR :: = SEQUENCE {}

OL_MIMO_Parameters :: = SEQUENCE {

OL-Region-ONINTEGER (0..1) OPTIONAL

OL-Rank1-ConfigurationINTEGER (0..7) OPTIONAL

SB-OL-Region-2SizeINTEGER (0..15) OPTIONAL

}

MIMO_OL_Region _Parameters :: = SEQUENCE {

OL-Region-Type0-ON INTEGER (0,1) OPTIONAL

OL-Region-Type1-NLRU-ON INTEGER (0,1) OPTIONAL

OL-Region-Type1-SLRU-Size INTEGER (0..15) OPTIONAL

OL-Region-Type2-SLRU-Size INTEGER (0..15) OPTIONAL

}

Parameters_GRA :: = SEQUENCE {}

PeriodicRNGParameters :: = SEQUENCE {}

GAMMA_IOT_FP0INTEGER (0..15) OPTIONAL

GAMMA_IOT_FP1INTEGER (0..15) OPTIONAL

GAMMA_IOT_FP2INTEGER (0..15) OPTIONAL

GAMMA_IOT_FP3INTEGER (0..15) OPTIONAL

Alpha (a)INTEGER (0.. 7) OPTIONAL

Beta (ß)INTEGER (0..1) OPTIONAL

SINRminINTERGER (0..15) OPTIONAL

BR_Channel Configuration MIN Access ClassINTERGER (0..4) OPTIONAL

Sounding_multiplexing_type CHOICE {

fdmFDM

cdmCDM

}

Decimation_value_D/Max_Cyclic_Shift_Index_P_for_soundingENUMERATED {n4, n9, n16,

n18, n36}

RELAY_Support_Parameters :: = SEQUENCE {

16m_Relay_zone_AMS_allocation_indicatorINTEGER (0..1) OPTIONAL

}

}

-- ASN1STOP

------------------------------------------------- Proposed Text 2 Ends -------------------------------------------------

--------------------------------------------- Proposed Text 3 for Starts -----------------------------------------------

Modify the text in Section 16.3.7.2.5.1, page 455, line 1

Dynamic switching between MIMO mode 1 and MIMO mode 3 in downlink transmissions in OL region type 2 is allowed. The rank-2 precoders for transmission with MIMO mode 1 or MIMO mode 3 in OL region type 2 are the same on a given subband.

The OL region type 0 is present if OL-Region-Type0-ON is indicated in a downlink broadcast message AAI_SCD message. All DRUs in frequency partition 0 will be in OL region type 0.

The OL region type 1 NLRU is present if OL-Region-Type1-NLRU-ON is indicated in a downlink broadcast message AAI_SCD message. All NLRUs in frequency partition 0 will be in OL region type 1. The OL region type 1 SLRU is present if OL-Region-Type1-SLRU-Size is greater than 0 in a downlink broadcast message AAI_SCD message. The PRUs allocated to type 1 OL region SLRU are the are the first (4* OL-Region-Type1-SLRU-Size) PRUs obtained by the mapping equation (187), i.e., PRUFP0(j), where j = 0 to (4 * OL-Region-Type1-SLRU-Size) -1.

The OL region type 2 SLRU is present if OL-Region-Type2-SLRU-Size is greater than 0 in a downlink broadcast message AAI_SCD message. The PRUs allocated to type 2 OL region are the (4* OL-Region-Type2-SLRU-Size) PRUs obtained by the mapping equation (187), i.e., PRUFP0(j), where j = (4 * OL-Region-Type1-SLRU-Size) to (4 * OL-Region-Type1-SLRU-Size) + (4 * OL-Region-Type2-SLRU-Size) -1.

All base stations that are coordinated over the same open loop region should use the same number of MIMO streams, in order to guarantee low interference fluctuation and thus improve the CQI prediction at the AMS. All pilots are precoded by non-adaptive precoding with MaxMt MIMO streams in the open-loop region. CQI measurements should be taken by the AMS on the precoded demodulation pilots rather than on the downlink midamble.

------------------------------------------------- Proposed Text 3 Ends -------------------------------------------------