| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to the feedback polling A-MAP IE (16.3.6.5.2.4.11) |  |
| Date Submitted | 2009-01-09 |  |
| Source(s) | Huaning Niu, Alexei Davydov, Yuan Zhu, Yang-Seok Choi Intel Corp Wookbong Lee, Bin-Chul Ihm, Jin-Sam Kwak LG Electronics | huaning.niu@intel.com wookbong.lee@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | P802.16m/D3 LB30b |  |
| Abstract | Modifications are proposed for the feedback polling A-MAP IE |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 3) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to the feedback polling A-MAP IE (16.3.6.5.2.4.11)

Huaning Niu, Alexei Davydov, Yuan Zhu, Yang-Seok Choi

Intel Corp

Wookbong Lee, Bin-Chul Ihm, Jin-Sam Kwak

LG Electronics

# Introduction

Long TTI should be used for reliable transmission when UL link budget is limited. If long TTI is not supported, the link budget issue cannot be solved even with multiple LRUs is used for transmission. Therefore we propose to add the Long TTI Indication bit in the IE.

We propose to remove the CS bit which is used to indicate the codebook subset. In single BS MIMO case, CS bit is used to reduce the feedback overhead in highly correlated channel. However the feedback overhead reduction has limited impact on MAC control message given all the overhead in forming a LRU. In multi BS MIMO, the 4 bit subset should always be used.

The MFM 3,4,6,7, correlation matrix needs to be feedback using long period, while CQI and PMI information will be feedback using short period. We propose to signal the short and long period combination using 16 combinations. Since the long period implicitly signaled the correlation matrix transmission, the correlation matrix indication bit and the CM bit can be removed to reduce overhead.

- References

[1] DRAFT amendment to IEEE standard for local and metropolitan area networks, P80216m/D3, Dec 2009.

# Text proposal for inclusion in the 802.16m amendment

<Modify section “16.3.6.5.2.4.11 Feedback Polling A-MAP IE” on line 1 page 437 as follows>

-------------------------------  Start of Text Proposal ---------------------------------------------------

16.3.6.5.2.4.11 Feedback Polling A-MAP IE

The information element shown in Table 827 is used by the ABS to schedule MIMO feedback transmission by the AMS. The AMS sends the MIMO feedback using a MAC control message or an extended header, depending on the requested feedback content.

When the Dedicated UL Allocation bit is set to 0b1, a dedicated UL allocation shall be included in this IE. The dedicated UL allocation shall be used by the AMS to transmit feedback at the designated feedback transmission frame defined by this IE. When the Dedicated UL Allocation bit is set to 0b0, no dedicated UL allocation shall be included. Instead, at the designated transmission frame defined by this IE, the AMS shall compose the feedback and the ABS shall include a UL allocation for the transmission using UL Basic Assignment A-MAP IE or UL Subband Assignment A-MAP IE.

The MS shall send the feedback in an extended header when the following feedback information is requested:

- wideband information for MIMO feedback modes 0, 1, 4 and 7

- subband information for 1 subband for MIMO feedback modes 2, 3, 5 and 6

The MS shall send the feedback in a MAC control message when the following feedback information is requested:

- subband information for more than one subband for MIMO feedback modes 2, 3, 5 and 6

- multi-BS feedback

The coefficients of the quantized transmit correlation matrix should be feedback in Correlation Matrix Feedback Extended Header (CMFEH) when no AAI_SingleBS_MIMO_FBK message is sent in the same packet if the ABS is equipped with 2 or 4 transmit antennas. Otherwise, the coefficients of the quantized transmit correlation matrix should be feedback in AAI_SingleBS_MIMO_FBK message. The coefficients of the quantized transmit correlation matrix shall be feedback in AAI_SingleBS_MIMO_FBK message if the ABS is equipped with 8 transmit antennas.

In case of feedback for MIMO feedback modes 0, 1, 4 or 7, the MS shall feedback the CQI for FP0 if FPCT equals 1 or 4FPS0>0, or for FPk if FPCT equals 2 or 3FPS0=0, where FPk is determined by k = floor(IDCell/256) )+1 for FPCT=3 or k = floor(IDcell/384)+1 for FPCT=2.

Table 827 - Feedback Polling A-MAP IE

| Syntax | Size in bits | Notes |
| --- | --- | --- |
| A-MAP IE Type { | 4 | Feedback_Polling_IE |
| Allocation Duration (d) | 3 | The allocation is valid for 2(d–1) superframes starting from the superframe defined by allocation relevance. If d == 0b000, the pre-scheduled feedback header transmission is released. If d == 0b111, the pre-scheduled feedback header transmission shall be valid until the BS commands to release it. |
| If (d ==0b000){ |  | Feedback de-allocation |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| HFA | 3 | HARQ feedback channel allocation for Feedback Channel De-allocation confirmation |
| } else if (d != 0b000){ |  | Feedback allocation |
| Dedicated UL allocation | 1 | 0b0: No dedicated UL resource is allocated. BS shall provide UL allocation for the MIMO feedback IE transmission through UL A-MAP IE at each designated transmitting frame defined by this IE. 0b1: Dedicated UL resource is included in this IE |
| If (Dedicated UL allocation ==0b1){ |  |  |
| ISizeOffset | 5 | Offset used to compute burst size index |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| MEF | 1 | MIMO encoder format for uplink feedback transmission Non-adaptive precoding shall be used at the AMS. 0b0: SFBC 0b1: VE with Mt = 2 if Nt > 1, or VE with Mt = 1 if Nt = 1 Nt is the number of transmit antennas at the AMS. |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: AAI subframe (default) 0b1: 4 UL AAI subframes for FDD or all UL AAI subframes for TDD If number of DL AAI subframes, D, is less than number of UL AAI subframes, U, Long TTI Indicator = 0b1 |
| } |  |  |
| Period (p) | 3 | Transmit feedback header every 4p frame. The first report shall start at the next frame. |
| MIMO_feedback_IE_type | 1 | - 0b0: feedback for single-BS MIMO operation - 0b1: feedback for multi-BS MIMO operation |
| If (MIMO_feedback_IE_type == 0b0){ |  | - Single-BS MIMO feedback request |
| Transmit_Correlation_Matrix | 1 | - 0b0: feedback of the transmit correlation matrix is indicated by CM - 0b0: feedback of the transmit correlation matrix is indicated by CM - 0b1: feedback of the quantized BS transmit correlation matrix only - Transmit correlation matrix shall be feedback if CM = 0b1 |
| - MFM | - 3 | - MIMO Feedback Mode for which the AMS shall transmit feedback. |
| Period | 4 | 16 combination of short feedback period p (frames) and long feedback period q (superframes). The first report shall start at the next frame. 0b0000: p = 1, q = 0 0b0001: p = 2, q = 0 0b0010: p = 4, q = 0 0b0011: p = 1, q = 1 0b0100: p = 2, q = 1 0b0101: p = 1, q = 2 0b0110: p = 2, q = 2 0b0111: p = 4, q = 2 0b1000: p = 1, q = 4 0b1001: p = 2, q = 4 0b1010: p = 4, q = 4 0b1011: p = 0, q = 1 0b1100: p = 0, q = 2 0b1101: p = 0, q = 4 0b1110: reserve 0b1111: reserve |
| If (Transmit_Correlation_Matrix == 0b0){ |  | ABS requests AMS to feedback CQI And CSI for a specific MFM. |
| MaxMt | Variable 1 or 2 | Variable number of bits - depends on number of transmit antenna Nt Nt at the ABS - If Nt=2 (Any MFM): - 0b0: 1 - 0b1: 2 - If Nt =4 (Any MFM): - 0b00: 1 - 0b01: 2 - 0b10: 3 - 0b11: 4 - If Nt =8 (SU-MIMO MFM 0, 1, 2, 3, 4): - 0b00: 1 - 0b01: 2 - 0b10: 4 - 0b11: 8 - If Nt =8 (MU-MIMO MFM 5, 6, 7): - 0b00: 1 - 0b01: 2 - 0b10: 3 - 0b11: 4 |
| - MFM | - 3 | - MIMO Feedback Mode for which the AMS shall transmit feedback. |
| If (MFM = 2, 3, 5, 6) { |  | - Feedback of CQI and CSI for localized resource units |
| Num_best_subbands | 2 | 0b00: report all subbands 0b01: 1 best subband 0b10: min{6, YSB} best subbands 0b11: min{12, YSB} best subbands 1< Num_best_subbands ≤ YSB |
| } |  |  |
| If (MFM == 3,4,6,7) { |  | Feedback of CQI and CSI for CL SU and MU MIMO |
| CM CCE | [TBD] 1 | [TBD] Codebook Coordination. This field shall be ignored if long period q > 0. 0b0: base mode with codebook coordination disabled 0b1: base mode with codebook coordination enabled |
| CS | 1 | 0b0: report PMI from the base codebook 0b1: report PMI from the codebook subset |
| } |  |  |
| If(MFM=0, 1, 2, 5){ |  | Indication of report for OL region |
| Measurement Method Indication | 1 | 0b0: Use the midamble for CQI measurements 0b1: Use pilots in OL region with MaxMt streams for CQI measurements |
| } |  |  |
| } |  |  |
| } |  |  |
| Else { |  | - Multi-BS MIMO feedback request |
| Period (p) | 3 | Transmit feedback header every 4×p frames. - The first report shall start at the next frame. |
| TRU | 2 | - Target RU indicating which RUs or which type of RU to work on for feedback 0b00: Latest best subbands reported for single BS MIMO 0b01: Whole bandwidth 0b10: FFR partition 0 0b11: boosted FFR partition |
| ICT | 2 | 0b00: PMI restriction for single-BS precoding; 0b01: PMI recommendation for single-BS precoding; 0b10: CL-MD for multi-BS precoding; 0b11: Co-MIMO for multi-BS precoding; |
| CS Codebook_subset | 1 | Codebook_subset shall be ignored if Nt ≠ 4. 0b0: report PMI from the base codebook 0b1: report PMI from the codebook subset |
| N_multiBS_reports | 3 | N_multiBS_reports indicates the number of reports. |
| If (ICT = 0b11) { |  |  |
| MaxUser | 2 | Maximum number of users supported in Co-MIMO in the same resource. 0b00: 2 users 0b01: 3 users 0b10: 4 users 0b11: reserved |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| Padding | variable | Padding to reach byte boundary |

Period: The setting of short and long periods p and q shall obey the rules below when MIMO_feedback_IE_type == 0b0.

For MFM 0 or 1: when p = 0, q > 0, the wideband CQI and wideband STC rate shall be transmitted using long feedback period. When p > 0, q = 0, the wideband CQI and wideband STC rate shall be transmitted using short feedback period. p > 0 and q > 0 shall not be set at the same time with MFM 0 or 1.

For MFM 2 or 5: when p = 0, q > 0, the Best_subbands_index, subband CQI and Stream index shall be transmitted using long feedback period. When p > 0, q = 0, the Best_subbands_index, subband CQI and Stream index shall be transmitted using short feedback period. p > 0 and q > 0 shall not be set at the same time with MFM 2 or 5.

For MFM 3: when p > 0 and q = 0, the Best_subbands_index, STC_Rate, subband CQI and subband PMI from the base codebook shall be transmitted using short period. When p = 0 and q > 0, only the correlation matrix shall be transmitted using long feedback period. When p > 0 and q > 0, the Best_subbands_index, STC_Rate, subband CQI and subband PMI from the transformed codebook shall be transmitted using short period, while the correlation matrix shall be transmitted using long period.

For MFM 4: when p > 0 and q = 0, the wideband CQI, STC rate and PMI from the base codebook shall be transmitted using short feedback period. When p > 0 and q > 0, the wideband CQI and STC rate shall be transmitted using short feedback period, and the correlation matrix shall be transmitted using long feedback period. When p = 0 and q > 0, only the correlation matrix shall be transmitted using long feedback period.

For MFM 6: when p > 0 and q > 0, the Best_subbands_index, subband CQI and subband PMI from the transformed codebook shall be transmitted using short period, while the correlation matrix shall be transmitted using long period. When p = 0 and q > 0, only the correlation matrix shall be transmitted using long feedback period. q = 0 shall not be set with MFM 6.

For MFM 7: when p > 0 and q = 0, the wideband CQI and PMI from the base codebook shall be transmitted using short feedback period. When p > 0 and q > 0, the wideband CQI shall be transmitted using short feedback period, and the correlation matrix shall be transmitted using long feedback period. When p = 0 and q > 0, only the correlation matrix shall be transmitted using long feedback period.

-------------------------------  End of Text Proposal  ---------------------------------------------------