| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal for Transformation Codebook Refinement and Tracking on Section 16.3.7.2.5.6.1 |  |
| Date Submitted | 2009-12-30 |  |
| Source(s) | Jiann-An Tsai, Jerry Pi, David Mazzarese Samsung Electronics Co., Ltd. | jtsai@sta.samsung.com |
| Re: | Contribution in support of a comment on LB30B Section 16.3.7.2.5.6.1 Transformation codebook feedback mode |  |
| Abstract | The contribution proposes the text of transformation codebook to be included in the 802.16m AWD (P802.16m/D3). |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m AWD (P802.16m/D3). |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AWD Text Proposal for Transformation Codebook Refinement and Tracking on Section 16.3.7.2.5.6.1

Jiann-An Tsai, Jerry Pi, David Mazzarese

Samsung Electronics Co., Ltd.

- Introduction

The contribution proposes the text of transformation codebook refinement and tracking (TCRT) to be included in the 802.16m AWD ([4]). The proposed text is developed so that it can be readily combined with IEEE P802.16 Rev2/D8 ([1]), it is compliant to the 802.16m SRD ([2])).

- References

[1] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Dec. 2008.

[2] IEEE 802.16m-07/002r7, “IEEE 802.16m System Requirements Document”

[3] IEEE 802.16m-08/003r8, “IEEE 802.16m System Description Document”

[4] IEEE 802.16m/D3, “802.16m Amendment Working Document (AWD)”

- Text proposal for inclusion in the 802.16m AWD/D3

-------------------------------  Text Start  ---------------------------------------------------------------

Inserting text on page 478, line 61,section 16.3.7.2.5.6.1 with the following text:

For eight transmit antenna, AMS and ABS shall set the transmit correlation matrix to the feedback value of R when the feedback report is transmitted. Between two feedback reports of transmit correlation matrix, both the AMS and the ABS shall simultaneously track and refine the transmit correlation matrix R. The tracking equation for estimating R in a feedback period is

R[t]=αR[t−1]+(1−α)vi[t]viH[t]+βurandom[t]urandomH[t]--- Equation xxxx

where t is the PMI feedback period index; α is the forgetting factor, which is designed to track the mobility of mobile channel; β is the random factor; vi[t]is the i-th codeword of transformation codebook in PMI feedback period t, as shown in Equation (218). urandom[t] is an complex random vector used in the tracking equation in PMI feedback period t. The feedback period index t is defined as

t = (FRAME_NUMBER – FEEDBACK_FRAME_OFFSET) / SHORT_TERM_FEEDBACK_PERIOD,

where FEEDBACK_FRAME_OFFSET and SHORT_TERM_FEEDBACK_PERIOD are defined in the Feedback Allocation A-MAP IE.

ABS shall signal the value of the forgetting factor α using the following 3-bit table, namely <b2b1b0>, as shown in Table xxx.

Table xxx. 3-bit of values for signalingα

| <b2b1b0> | α |
| --- | --- |
| 000 | 0.1 |
| 001 | 0.2 |
| 010 | 0.4 |
| 011 | 0.5 |
| 100 | 0.7 |
| 101 | 0.8 |
| 110 | 0.9 |
| 111 | 0.95 |
| 011 | 0.4 |
| 100 | 0.5 |
| 101 | 0.6 |
| 110 | 0.7 |
| 111 | 0.9 |

urandom is generated based on a binary pseudorandom sequence produced by a Linear Feedback Shift Register (LFSR) with the polynomial generator g(x) = x16 + x15 + x2 + 1, as shown in Figure xxx.

Figure xxx. PRBS generator for random vector generation

The binary pseudorandom sequence generator shall be initialized in each feedback period by the seed b15b14b13b12b11…b2b1b0. The 12 LSBs of the seed shall be the AMS’s STID. The 4 MSBs of the seed shall be the 4 LSBs of the feedback period index t. The random vector, urandom, shall be generated as follows:

- At the beginning of PMI feedback period t, both the ABS and the AMS initialized the LFSR with the same seed. b15b14b13b12b11…b2b1b0. The 12 MSBs of the seed shall be the AMS’s STID. The 4 LSBs of the seed shall be the 4 LSBs of the feedback period index t.

- Each entry of the random vector urandom is quantized into 8 bits and there are Nt entries in the random vector urandom, where Nt is the number of transmit antennas at the ABS. Both the ABS and the AMS clock the LFSR 8 × Nt times with the first 8 binary output of the LFSR forms the first entry of the vector, the second 8 binary output of the LFSR forms the second entry of the vector, and so on. Denote the generated vector by urandom, unnormalized. To form an entry in the random vector, the first 4 bits form the real part of the entry and the last 4 bits form the imaginary part of the entry. The signage of the real part or the imaginary part is indicated by the first bit of each group of 4 bits.

- If ||urandom, unnormalized||2 = 0, Go back to step 2; Otherwise, normalize the random vector by dividing the generated random vector by its norm,

urandom=urandom, unnormalized||urandom, unnormalized||.

-------------------------------  Text End  -----------------------------------------------------------------