| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for LBS measurement in idle mode (16.2.17) |  |
| Date Submitted | 2009-12-30 |  |
| Source(s) | Lei Zhou, Xufeng Zheng, Chuan Zhong, Fangmin Xu Beijing Samsung Telecom R&D Center Sangheon Kim, Brian Shim, Naehyun Lim, Hokyu Choi, Heewon Kang Samsung Electronics Co., Ltd. | l.zhou@samsung.com +86-10-59253333 |
| Re: | P802.16m/D3 |  |
| Abstract | The contribution proposes to clean up LBS measurement indication in Paging message because for LBS measurement AMS has to re-entry network and then perform LBS measurement |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for LBS measurement in idle mode

Lei Zhou, , Xufeng Zheng, Fangmin Xu, Chuan Zhong

Beijing Samsung Telecom R&D Center

Sangheon Kim, Brian Shim, Naehyun Lim,Hokyu Choi, Heewon Kang

Samsung Electronics Co., Ltd.

- Introduction

This contribution proposes to clean up LBS measurement indication in paging message.

The AMS in idle mode should perform network re-entry and LBS measurement with attached ABS and neighbor ABSs. So we needn’t indicate LBS measurement in paging message.

- The Proposed Text in AWD

-----------------------------------------------  Text Start  -----------------------------------------------------

-----------------------------------------  Proposed Text 1 Start  ----------------------------------------------

[adopt the following text modification started from line# 42 Page 262 in P802.16m/D3]

16.2.17.2.1 Broadcast paging message

A Paging message is an AMS notification message which either indicates the presence of DL traffic pending for the specified AMS or it is intended to poll an AMS and request a location update without requiring a full network entry or to request an AMS to perform LBS measurement. In addition, an emergency alert indicator is included in the paging message to notify the idle AMSs reception of the emergency information. Upon reception of the emergency alert indicator that is set to '1', the AMS shall decode the A-A-MAP to obtain the emergency information.

A single Paging message may include the information for multiple AMSs.

Paging message includes identification of the AMSs to be notified of DL traffic pending or location update, or LBS measurement.

The Paging message also includes an action code directing each AMS notified via the inclusion of its identifier as appropriate:

•0b0: Perform network re-entry

•0b1: Perform ranging to establish location

•0b10: Perform LBS measurement

-----------------------------------------  Proposed Text1 End  -------------------------------------------------

-----------------------------------------  Proposed Text2 Start  -------------------------------------------------

[adopt the following text modification started from line# 20 Page 263 in P802.16m/D3]

Table 756—Parameters for AAI_PAG-ADV message format

| Name | Value | Usage |
| --- | --- | --- |
| Paging_Group_IDs bitmap | Each bit in the bitmap indicates: 0: the paging information for the corresponding PGID is not included 1: the paging information for the corresponding PGID is included | The size of Paging_Group_IDs bitmap equals to the number of paging group IDs in the PGID info message. |
| Num_AMSs | Number of paged AMSs in a particular paging group | For each non-zero bit in the Paging_Group_IDs bitmap, there shall be one Num_AMSs. |
| Deregistration Identifier | 0~1023 | Deregistration Identifier and Paging Cycle are used to identify each paged AMS. |
| Paging Cycle | 0b0000~0b1111 | Deregistration Identifier and Paging Cycle are used to identify each paged AMS. |
| Action Code | 0b00 = Perform network re-entry 0b01 = Perform ranging to establish location 0b10 = Perform LBS measurement 0b11 = Reserved 0b0 = Perform network re-entry 0b1 = Perform ranging to establish location | Paging action instruction to each paged AMS. |
| Extension Flag | 0 = There is no remaining part of this paging message 1 = The remaining part of this paging message will be transmitted in the subsequent frame or sub-frame | If the ABS cannot transmit the entire paging message in a predetermined region, the Extension Flag will be set to 1. Then the remaining part of the paging message is transmitted in the earliest subsequent frame or subframe. |
| Emergency Alert Indication | 0 = There is no emergency information 1 = There is emergency information | If the emergency alert indicator is set to '1', the AMS shall decode the A-A-MAP to obtain the emergency information. |

----------------------------------------- Proposed Text2 End  ------------------------------------

-----------------------------------------  Text End  -----------------------------------------------