| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Cleanup Text for AAI_FFR-REP (16.2.3.20) for the IEEE 802.16m/D3 |  |
| Date Submitted | 2009-12-29 |  |
| Source(s) | Yuan Zhu, Qiuping Huang, Xiaogang Chen, Yi Hsuan, Hujun Yin Intel Corporation | yuan.y.zhu@intel.com |
| Re: | Proposed text changes to P802.16m/D3 16.2.3.19 |  |
| Abstract |  |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D4 Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Cleanup Text for AAI_FFR-REP (16.2.3.20) for the IEEE 802.16m/D3

Yuan Zhu, Qiuping Huang, Xiaogang Chen, Hujun Yin

Intel Corporation

- Introduction

This contribution proposes some cleanup text for AAI_FFR-REP Message section 16.2.3.20 of P802.16m/D3 [1]. The first change is to remove “Full Report Flag”, which is redundant. The second change is to change the bit length of “Reporting Type” from 1 to 4, and then ABS can individually select each report type the AMS needs to feedback accordingly to ABS’s need. The third change is to define the range for each reporting quantities.

- References

[1] Draft Amendment, P802.16m/D3, December 2009.

-------------------------------------------------- Text Change #1 starts------------------------------------------------

Add one row in section 16.2.3, Table 673, page 38

| Message name | Message description | Security | Connection |
| --- | --- | --- | --- |
| AAI_FFR-REP | FFR measurement report | Encrypted/ICV | Unicast |

-------------------------------------------------- Text Change #1 ends-------------------------------------------------

-------------------------------------------------- Text Change #2 starts------------------------------------------------

Make the following changes in section 16.2.3.20, page 79, line 51

16.2.3.19 AAI_FFR-REP (FFR Report) Message

An AAI_FFR-REP message is sent by an AMS to report the interference statistics of frequency partition(s). Table 697 shows the parameters for AAI_FFR-REP message:

Table 697—AAI_FFR-REP message format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| AAI_FFR-REP_Message_format(){ |  |  |
| Control Message Type = XX | 8 |  |
| Full Report Flag | 1 | 0: Report the measurement result for specified frequency partition 1: Report the measurement result for full frequency partition |
| If(Full Report Flag==0){ |  |  |
| Frequency Partition Bitmap | N_FPFPCT | Each bit indicates the interference statistics report status of corresponding Frequency Partition. 0b0: no report interference statistics 0b1: report interference statistics N_FPFPCT: the number of Frequency Partitions defined in Table 777 to Table 781, Frequency Partition Bitmap shall have at least one bit having value ‘1’. The LSB indicates FP0 and MSB indicates highest FP. |
| Reporting Type | 14 | 0: Interference Measurement Report 1: SINR Measurement Report Each bit indicates if one type of report is required to be sent by AMS. If one bit has value ‘1’, it indicates the specific report type is required to be sent, otherwise it indicates the specific report type is not required to be sent. At least 1 bit needs to be set to value ‘1’. LSB#0: Interference-Mean LSB#1: Interference-Variance LSB#2: SINR-Mean LSB#3: SINR-Variance |
| If (Reporting Type == 0){ |  |  |
| For(n=0;n<N_Bitmap_1;n++){ |  | N_Bitmap_1:number of bit ‘1’ in Frequency Partition bBitmap |
| If(LSB#0 in Reporting Type == 1){ |  |  |
| Interference Measurement Report for nth FP-Mean | 8 | Per subcarrier receiving power(Noise and intercell interference) -134dBm to -30dBm in units of 1dB. -134dBm is encoded as 0x00, -30dB is encoded as 0x68, 0x69 to 0xFF is reserved. |
| } |  |  |
| If(LSB#1 in Reporting Type == 1){ |  |  |
| Interference Measurement Report for nth FP-Variance | 84 | 0dB to 15dB in units of 1dB |
| } |  |  |
| } |  |  |
| }else{ |  |  |
| For(n=0;n<N_Bitmap_1;n++){ |  | N_Bitmap_1:number of bit ‘1’ in Frequency Partition bitmap |
| If(LSB#2 in Reporting Type == 1){ |  |  |
| SINR Measurement Report for nth FP-Mean | 8 | -16dB to 53dB in units of 0.5dB -16dB is encoded as 0x00, 53dB is encoded as 0x8A, 0x8B-0xFF are reserved |
| } |  |  |
| If(LSB#3 in Reporting Type == 1){ |  |  |
| SINR Measurement Report for nth FP-Variance | 84 | 0dB to 15dB in units of 1dB |
| } |  |  |
| } |  |  |
| } |  |  |
| }else{ |  |  |
| Reporting Type | 1 | 0: Interference Measurement Report 1: SINR Measurement Report |
| For(n=0;n<N_FP;n++){ |  | N_FP: number of total frequency partition |
| If(Reporting Type==0){ |  |  |
| Interference Measurement Report for FPn-Mean | 8 |  |
| Interference Measurement Report for FPn-Variance | 8 |  |
| }else{ |  |  |
| SINR Measurement Report for FPn-Mean |  |  |
| SINR Measurement Report for FPn-Variance |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| Padding | variable | if needed for alignment to byte boundary |
| } |  |  |

-------------------------------------------------- Text Change #2 ends-------------------------------------------------