| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Cleanup Text for AAI_DL_IM (16.2.3.31) for the IEEE 802.16m/D3 |  |
| Date Submitted | 2009-12-29 |  |
| Source(s) | Yuan Zhu, Qiuping Huang, Xiaogang Chen, Yi Hsuan, Hujun Yin Intel Corporation | yuan.y.zhu@intel.com |
| Re: | Proposed text changes to P802.16m/D3 16.2.3.31 |  |
| Abstract |  |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D4 Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Cleanup Text for AAI_DL_IM (16.2.3.31) for the IEEE 802.16m/D3

Yuan Zhu, Qiuping Huang, Xiaogang Chen, Yi Hsuan, Hujun Yin

Intel Corporation

- Introduction

This contribution proposes some cleanup text for AAI_DL_IM Message section 16.2.3.31 of P802.16m/D3 [1].

- References

[1] Draft Amendment, P802.16m/D3, December 2009.

-------------------------------------------------- Text Change starts------------------------------------------------

Make the following changes in section 16.2.3.31, page 96, line 13

16.2.3.31 AAI_DL_IM Message

An AAI_DL_IM broadcast DL FFR partition resource metric, BC_SI, NIP_threshold, power level for each frequency partition.

--ASN1START

AAI_DL_IM Message ::=SEQUENCE {

FFR partition resource metric  :: = SEQUENCE {

Resource_Metric_FP2			INTEGER	(0..15)	OPTIONAL

Resource_Metric_FP3			INTEGER	(0..15)	OPTIONAL

}

BC_SI						INTEGER	(0..15) OPTIONAL

NIP_th_1					INTEGER	(TBD) OPTIONAL

NIP_th_2					INTEGER	(TBD) OPTIONAL

CINR_th					INTEGER	(TBD) OPTIONAL

FP1_power					INTEGER	(TBD) OPTIONAL

FP2_power					INTEGER	(TBD) OPTIONAL

FP3_power					INTEGER	(TBD) OPTIONAL

Diversity_Set_Num				INTEGER	(0..15) OPTIONAL

Change_count					INTEGER	(0..255) OPTIONAL

Temp_BSID ::= SEQUENCE OF {

BS_index				INTEGER	(0..255) OPTIONAL

}

}

--ASN1STOP

Table 704—Parameters for AAI_DL-_IM Message

| NameSyntax | ValueSize(bits) | UsageNotes |
| --- | --- | --- |
| Resouce_Metric_FP2 | TBD4 or 0 | Resource Metric of the first power de-boosted frequency partition which is defined in Table 761. There is no this field if FPCT=1 which is defined in Table 777 to Table 781 |
| Resouce_Metric_FP3 | TBD4 or 0 | Resource Metric of the second power de-boosted frequency partition which is defined in Table 761. There is no this filed if FPS3=0 which is defined in Table 777 to Table 781 |
| BC_SI | 8 bits for Nt=2 and 16 bits for Nt=4 or 8 | If LSB#n is ‘1’, the nth codeword within the codebook or its subset is recommended, otherwise the codeword is restricted. The code books or its subset are defined in section 16.3.7.2.5.6 for Nt=2, 4 or 8 respectively. |
| NIP_th_1 | TBD4 |  |
| NIP_th_2 | TBD4 |  |
| CINR_th | 4 |  |
| FP1_power(p) | TBD3 or 0 | Power boosting values of FP1 compared with FP0. There is no this field if FPCT=1, which is defined in Table 777 to Table 781. The power loading values of FP1 is defined in Table 705. |
| FP2_power | TBD |  |
| FP3_power | TBD |  |
| Diversity_Set_NUM | 4 | The number of ABSs coordinating with the serving ABS for single BS precoding with multi-BS coordination. |
| Change_count | 8 | Change count in AAI_NBR-ADV for the following BS_index |
| BS_index | 8 | Indicates diversity set members Refers to the ABS index of the AAI_NBR_ADV. Temp_BSID is derived from BS_index by TBD manner |

FP1_power(p): When FPCT=3 or 4 and FPSi>0 for 1≤i≤3, the transmit power boosting for FP1 is 10×log10(1+0.25×(p+1)) dB and the transmit power de-boosting for FP2 and FP3 is -10×log10((2-0.25×(p+1)) /2) dB. When FPCT=2 or 3 and FPS3=0, the transmit power boosting for FP1 is 10×log10(1+0.125×(p+1)) dB and the transmit power de-boosting for FP2 is -10×log10(1-0.125×(p+1)) dB. The power boosting/de-boosting values are listed in table 704.

Table 704—Power boosting/de-boosting values

| FP1_power(p) | FPCT=3 or 4 and FPSi>0 for 1≤i≤3 |  |  | FPCT=2 or 3 and FPS3=0 |  |
| --- | --- | --- | --- | --- | --- |
|  | FP1_power(dB) | FP2_power(dB) | FP3_power(dB) | FP1_power(dB) | FP2_power(dB) |
| 0b000 | 0.9691 | -0.5799 | -0.5799 | 0.5115 | -0.5799 |
| 0b001 | 1.7609 | -1.2494 | -1.2494 | 0.9691 | -1.2494 |
| 0b010 | 2.4304 | -2.0412 | -2.0412 | 1.3830 | -2.0412 |
| 0b011 | 3.0103 | -3.0103 | -3.0103 | 1.7609 | -3.0103 |
| 0b100 | 3.5218 | -4.2597 | -4.2597 | 2.1085 | -4.2597 |
| 0b101 | 3.9794 | -6.0206 | -6.0206 | 2.4304 | -6.0206 |
| 0b110 | 4.3933 | -9.0309 | -9.0309 | 2.7300 | -9.0309 |
| 0b111 | 4.7712 | -inf | -inf | 3.0103 | -inf |

-------------------------------------------------- Text Change ends-------------------------------------------------