| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for MIMO feedback extended header (16.2.2.2.7) for the IEEE 802.16m/D3 |  |
| Date Submitted | 2009-12-29 |  |
| Source(s) | Yuan Zhu, Huaning Niu, Alexei Davydov, Yang-seok Choi Intel Corporation | yuan.y.zhu@intel.com huaning.niu@intel.com alexei.davydov@intel.com |
| Re: | Proposed text changes to P802.16m/D3 16.2.2.2.7 |  |
| Abstract |  |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D4 Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for MIMO feedback extended header (16.2.2.2.7) for the IEEE 802.16m/D3

Yuan Zhu, Huaning Niu, Alexei Davydov, Yang-seok Choi

Intel Corporation

- Introduction

MIMO feedback extended header (MFEH) is defined in D3 [1] as a response to Feedback Polling A-MAP IE. The bit width of many fields in MFEH is dependent on the value of the fields of the latest received Feedback Polling A-MAP IE. This creates some risk for the MFEH format mismatch at both AMS and ABS. For example when ABS firstly polls one type of feedback using one Feedback Poling A-MAP IE and secondly reconfigures the parameter of the same feedback using another Feedback Polling A-MAP IE and the second Feedback Polling A-MAP IE is lost over the air, in this case AMS will send MFEH according to the MFEH format decided by the first Feedback Polling A-MAP IE but ABS will parse the MFEH according to the second Feedback Polling A-MAP IE. Though it is possible for ABS to allocate one HARQ channel for the AMS to send one ACK in the allocated HARQ channel if AMS has received the Feedback Polling A-MAP IE of interests, ABS has to use energy detection and the reliability may not be good enough.

This contribution proposes some text changes to remove the format dependency of MFEH on the latest Feedback Polling IE and ABS can parse the MFEH correctly regardless of which Feedback Polling A-MAP IE is lost over the air. Though it will increase some bits for the MFEH contents, the overall size after padding to byte boundary is not largely affected. The proposed text can be adopted in section 16.2.2.2.7 of P802.16m/D3 [1].

- References

[1] Draft Amendment, P802.16m/D3, December 2009.

-------------------------------------------------- Text Change starts------------------------------------------------

Make the following changes in section 16.2.2.2.7, page 35, line 33

16.2.2.2.7 MIMO feedback extended header (MFEH)

This MIMO feedback extended header format is defined in Table 670. This header is used by AMS as a response to a Feedback Polling A-MAP IE requesting to feedback only the wideband information for MFM 0, 1, 4 or 7, or only the subband information for one subband for MFM 2, 3, 5 or 6.

Table 670—MFEH Format

| Syntax | Size(bit) | Notes |
| --- | --- | --- |
| MFEH(){ |  |  |
| Type | 4 | MFEH type |
| MFM | 3 |  |
| If(MFM==0 or 1){ |  |  |
| Wideband CQI | 4 |  |
| Wideband STC rate | 1 to 2 | Depending on MaxMt |
| } |  |  |
| If(MFM==4 or 7){ |  |  |
| Wideband CQI | 4 |  |
| Wideband PMI | 3 to 6 | Depending on Nt and CS |
| } |  |  |
| If(MFM==2){ |  |  |
| Subband index | 3 to 5 | Depending on bandwidth |
| Subband CQI | 4 |  |
| Subband STC rate | 1 to 3 | Depending on MaxMt |
| } |  |  |
| If(MFM==3){ |  |  |
| Subband index | 3 to 5 | Depending on bandwidth |
| Subband CQI | 4 |  |
| Subband STC rate | 1 to 3 | Depending on MaxMt |
| Subband PMI | 3 to 6 | Depending on Nt and CS |
| } |  |  |
| If(MFM==5){ |  |  |
| Subband index | 3 to 5 | Depending on bandwidth |
| Subband CQI | 4 |  |
| Subband stream index | 1 to 3 | Depending on MaxMt |
| } |  |  |
| If(MFM==6){ |  |  |
| Subband index | 3 to 5 | Depending on bandwidth |
| Subband CQI | 4 |  |
| Subband PMI | 3 to 6 | Depending on Nt and CS |
| } |  |  |
| Reserved | Variable | For byte alignment |
| } |  |  |

-------------------------------------------------- Text Change ends-------------------------------------------------