| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Clean-up Text of Frame Structures for 8.75MHz and 7MHz (16.3.2, 16.3.3) |  |
| Date Submitted | 2010-01-11 |  |
| Source(s) | Jaeweon Cho, Hokyu Choi, Samsung Electronics Bill Hillery, Motorola Yu-Tao Hsieh, Chia-Lung Tsai, Pang-An Ting, ITRI | +82-31-279-5796 jaeweon.cho@samsung.com bill.hillery@motorola.com ythsieh@itri.org.tw |
| Re: | IEEE 802.16-09/0073, “IEEE 802.16 Working Group Letter Ballot Recirc #30b” |  |
| Abstract | This contribution proposes several text changes in description of 8.75MHz and 7MHz frame structures for the purpose of clean-up. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Clean-up Text of Frame Structures for 8.75MHz and 7MHz (16.3.2, 16.3.3)

Jaeweon Cho, Hokyu Choi

Samsung Electronics.

Bill Hillery,

Motorola

Yu-Tao Hsieh, Chia-Lung Tsai, Pang-An Ting,

ITRI

- Introduction

This contribution proposes several text changes in description of 8.75MHz and 7MHz frame structures for the purpose of clean-up.

- Reduce the number of OFDM symbols per TDD frame and Change the last UL subframe type, for 8.75MHz with G = 1/4.
[Remedy 1, 2 and 3].

  - To be consistent with SDD (section 11.3) and IMT-Adv RIT Proposal (part 3; TDT section 4.2.3.2.8.6)

- Correct the number of subframes per frame for 8.75MHz and 7MHz.
[Remedy 4]

- Change the minimum value of FRAME_OFFSET to two, for 8.75MHz legacy support mode.
[Remedy 5, 6 and 7]

  - Three OFDM symbols are not enough for the 1st 16e zone (LZone) operation.

- Correct the wrong implementation of the accepted comment #832 in IEEE 802.16-09/0058r4.
[Remedy 7]

- Correct the position of UL subframe type for 8.75MHz legacy support mode with FDM.
[Remedy 8]

  - To be consistent with frame configuration table (Table 772).

- Remove the TDD frame configuration of D:U=2:4, for 7MHz with G = 1/16.
[Remedy 9]

- Correct the wrong number of DL offset for 7MHz legacy support mode.
[Remedy 9]

- Proposed Text Changes in P802.16m/D3

[Remedy 1: In Table 766 (line 24~29, page 292, in 16.3.2.4.), Change the text as follows:]

| CP ratio, G = 1/4 | OFDMA symbol time, Ts (µs) |  | 114.286 | 160 | 128 | 114.286 | 114.286 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FDD | Number of OFDMA symbols per 5ms frame | 43 | 31 | 39 | 43 | 43 |
|  |  | Idle time (µs) | 85.694 | 40 | 8 | 85.694 | 85.694 |
|  | TDD | Number of OFDMA symbols per 5ms frame | 42 | 30 | 38 37 | 42 | 42 |
|  |  | TTG + RTG (µs) | 199.98 | 200 | 136 264 | 199.98 | 199.98 |

[Remedy 2: Change the text from line 1 to 4 on the page 304, in 16.3.3.4., as follows:]

For a channel bandwidth of 8.75MHz, an FDD frame shall have three type-1 AAI subframes and three type-2 AAI subframes. And a TDD frame shall have four five type-1 AAI subframes and two one type-2 AAI subframes. In TDD, the AAI subframe preceding a DL to UL switching point shall be a type-1 AAI subframes.

[Remedy 3: In Figure 473 (the page 304, in 16.3.3.4.), Change the number of symbols in the last UL
 subframe of TDD frame from ‘7’ to ‘6’. I.e., “UL SF5 (7 6)” ]

[Remedy 4: Change the text from line 53 to 60 on the page 295, in 16.3.3.1., as follows:]

When using the same OFDMA parameters as in Table 766 with the channel bandwidth of 5 MHz, 10 MHz, or 20 MHz, each 5 ms radio frame further consists of eight AAI subframes for G = 1/8 and 1/16. For G = 1/4, the 5 ms radio frame consists of seven AAI subframes. With the channel bandwidth of 8.75 MHz, the 5 ms radio frame consists of seven AAI subframes for G = 1/8 and 1/16, and six AAI subframes for G = 1/4. With the channel bandwidth of 7 MHz, the 5 ms radio frame consists of six AAI subframes for G = 1/16, and five AAI subframes for G = 1/8 and G = 1/4. With the channel bandwidth of 8.75 and 7 MHz, each 5 ms radio frame further consists of seven and six AAI subframes, respectively for G = 1/8 and 1/16. In the case of G = 1/4, the number of AAI subframes per frame is one less than that of other CP lengths for each bandwidth case.

[Remedy 5: Change the text from line 62 to 65 on the page 304, in 16.3.3.5.1., as follows:]

The number of symbols in the first WirelessMAN-OFDMA DL Zone is 3+6·(FRAME_OFFSET - 1) for 8.75 MHz and 9+6·(FRAME_OFFSET - 1) for 7 MHz. The maximum value of parameter K is equal to the number of DL AAI subframes minus two. The minimum value of FRAME_OFFSET shall be two for 8.75 MHz channel bandwidth, and the minimum value of FRAME_OFFSET shall be one for other bandwidths.

[Remedy 6: Change the text from line 29 to 33 on the page 312, in 16.3.3.7., as follows:]

The frame structure supporting the WirelessMAN-OFDMA frame defined in 15.3.3.5 16.3.3.5.1, shall be configured with one of the following configuration numbers (i.e. 'No.' in the tables): 11 to 22 in Table 771 for 5/10/20 MHz channel bandwidths;  9 to 14 in Table 772 for 8.75 MHz channel bandwidth;  8 to 9 in Table 773 for 7 MHz channel bandwidth. from 11 to 22 in the Table 771— for 5/10MHz channel bandwidth, 9 to 20 in theTable 772— for 8.75MHz channel bandwidth, 8 to 9 in the Table 773— for 7MHz channel bandwidth.

[Remedy 7: In Table 772 (the page 314, in 16.3.3.7.), Change the text as follows:]

| - No. | - BW | - CP | - Frame configuration index | - Duplex | - D:U | - Subframe provision |  |  | - Subframe Type |  |  |  |  |  |  |  | - TTG /RTG - (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | - DL offset | - DL length | - UL length | - #0 | - #1 | - #2 | - #3 | - #4 | - #5 | - #6 | - #7 |  |
| - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … |
| - 13 | - 8.75 | - 1/8 | - 8 | - TDD | - 5:2 | - 2 | - 2 | - 2 | - DL Type1 | - DL Type1 | - Not used | - UL Type1 | - UL Type41 | - Not used | - Not used |  | - 82.2/ - 74.4 |
| - 14 | - 8.75 | - 1/8 | - 9 | - TDD | - 5:2 | - 2 | - 2 | - 1 | - DL Type1 | - DL Type1 | - Not used | - Not used | - UL Type1 | - Not used | - Not used |  | - 82.2/ - 74.4 |
| - 15 | - 8.75 | - 1/8 | - 10 | - TDD | - 5:2 | - 1 | - 4 | - 2 | - DL - Type1 | - DL - Type1 | - DL - Type1 | - DL - Type1 | - UL - Type1 | - UL - Type4 | - Not Used |  | - 189.6/ - 74.4 |
| - 16 | - 8.75 | - 1/8 | - 11 | - TDD | - 5:2 | - 1 | - 4 | - 1 | - DL - Type1 | - DL - Type1 | - DL - Type1 | - DL - Type1 | - Not Used | - UL - Type1 | - Not Used |  | - 189.6/ - 74.4 |
| - 17 | - 8.75 | - 1/8 | - 12 | - TDD | - 5:2 | - 1 | - 3 | - 2 | - DL - Type1 | - DL - Type1 | - DL - Type1 | - Not Used | - UL - Type1 | - UL - Type4 | - Not Used |  | - 189.6/ - 74.4 |
| - 18 | - 8.75 | - 1/8 | - 13 | - TDD | - 5:2 | - 1 | - 3 | - 1 | - DL - Type1 | - DL - Type1 | - DL - Type1 | - Not Used | - Not Used | - UL - Type1 | - Not Used |  | - 82.2/ - 74.4 |
| - 19 | - 8.75 | - 1/8 | - 14 | - TDD | - 5:2 | - 1 | - 2 | - 2 | - DL - Type1 | - DL - Type1 | - Not Used | - Not Used | - UL - Type1 | - UL - Type4 | - Not Used |  | - 82.2/ - 74.4 |
| - 20 | - 8.75 | - 1/8 | - 15 | - TDD | - 5:2 | - 1 | - 2 | - 1 | - DL - Type1 | - DL - Type1 | - Not Used | - Not Used | - Not Used | - UL - Type1 | - Not Used |  | - 82.2/ - 74.4 |
| - 21 - 15 | - 8.75 | - 1/4 | - 0 | - TDD | - 4:2 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type2 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type12 |  |  | - (TBD) - 189.6/ - 74.4 |
| - 22 - 16 | - 8.75 | - 1/4 | - 1 | - TDD | - 3:3 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type2 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type12 |  |  | - (TBD) - 189.6/ - 74.4 |
| - 23 - 17 | - 8.75 | - 1/4 | - 2 | - TDD | - 2:4 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type2 | - UL Type1 | - UL Type1 | - UL Type1 | - UL Type12 |  |  | - (TBD) - 189.6/ - 74.4 |
| - 24 - 18 | - 8.75 | - 1/4 | - 2 | - FDD | - N/A | - N/A | - N/A | - N/A | - D/U Type1 | - D/U Type2 | - D/U Type1 | - D/U Type2 | - D/U Type1 | - D/U Type2 |  |  | - N/A |

[Remedy 8: Change the text from line 29 to 32 on the page 305, in 16.3.3.5.1., as follows:]

In the case of 8.75 MHz with 15 UL OFDM symbols, the [first] UL AAI subframe is type-4 a type-1 AAI subframe and the second UL AAI subframe is type-1 a type-4 AAI subframe.

[Remedy 9: In Table 773 (the page 315, in 16.3.3.7.), Change the text as follows:]

| - No. | - BW | - CP | - Frame configuration index | - Duplex | - D:U | - Subframe provision |  |  | - Subframe Type |  |  |  |  |  |  |  | - TTG /RTG - (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | - DL offset | - DL length | - UL length | - #0 | - #1 | - #2 | - #3 | - #4 | - #5 | - #6 | - #7 |  |
| - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … |
| - 2 | - 7 | - 1/16 | - 1 | - TDD | - 3:3 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type1 | - DL Type3 | - UL Type1 | - UL Type1 | - UL Type1 |  |  | - 180/ 60 |
| - 3 | - 7 | - 1/16 | - 2 | - TDD | - 2:4 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type3 | - UL Type1 | - UL Type1 | - UL Type1 | - UL Type1 |  |  | - 180/ 60 |
| - 4 | - 7 | - 1/16 | - 3 | - FDD | - N/A | - N/A | - N/A | - N/A | - D/U Type1 | - D/U Type1 | - D/U Type1 | - D/U Type1 | - D/U Type1 | - D/U Type1 |  |  | - N/A |
| - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … |
| - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … |
| - 8 | - 7 | - 1/8 | - 3 | - TDD | - 3:2 | - 2 1 | - 2 | - 2 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - Not Used |  |  |  |  |
| - 7 | - 7 | - 1/8 | - 4 | - TDD | - 3:2 | - 2 1 | - 2 | - 1 | - DL Type1 | - DL Type1 | - Not Used | - UL Type1 | - Not Used |  |  |  |  |
| - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … |