| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | FPEH Overhead reduction & Quick Decryption (16.2.2) |  |
| Date Submitted | 2010-01-14 |  |
| Source(s) | Anil Agiwal, Youngbin Chang, Rakesh Taori, Jungje Son Samsung Electronics Kiseon Ryu LGE | anilag@samsung.com, yb.chang@samsung.com, *<http://standards.ieee.org/faqs/affiliationFAQ.html> ksryu@lge.com |
| Re: | IEEE 802.16-09/0073, IEEE 802.16 Working Group Letter Ballot Recirc #30b / Topic: FPEH Format (Section 16.2.2.2.1) |  |
| Abstract | This contribution proposes a method of FPEH Overhead reduction & Quick Decryption of control connection MAC PDU |  |
| Purpose | To be discussed and adopted by TGm for P802.16m/D4. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

FPEH Overhead reduction & Quick Decryption

Anil Agiwal, Youngbin Chang, Rakesh Taori, Jungje Son

Samsung Electronics

# Introduction

This contribution summarizes the changes needed (with editorial instructions) for the proposal in the slides S802.16m-09/2745.pdf.

# Proposed text

[Change 1: Modify the section 16.2.2.2.2, page 31, lines 1-47 as follows]

16.2.2.2.2 MAC Control extended header (MCEH)

The MCEH shall be used when MAC PDU contains payload from a control connection. The MCEH format is defined in Table 665.

Table 665: MCEH Format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| MCEH (){ |  |  |
| Type | 4 | MCEH Type |
| EC | 1 | Encryption Control indicator 0 = Payload is not encrypted 1 = Payload is encrypted |
| Channel ID | 1 | Channel ID to identify separate fragmentation / reassembly state machines 0: channel 1 1: channel 2 |
| SN Indicator | 1 | 0 = no FC and sequence number 1= FC and sequence number are followed |
| If ( SN Indicator == 0) |  |  |
| { |  |  |
| Reserved | 52 | For byte alignment |
| }else |  |  |
| { |  |  |
| Polling | 1 | 0 = no acknowledgement required 1 = acknowledge required upon receiving the MAC message |
| FC | 2 | Fragmentation control (see Table 664) |
| SN | 87 | Payload sequence number |
| reserved | 2 | For byte alignment |
| } |  |  |
| } |  |  |

[Change 2: Modify the section 16.2.4.5, page 125, lines 4-20 as follows]

16.2.4.5 Encryption of MAC PDUs

When transmitting a MAC PDU on a connection that is mapped to an SA, the sender shall perform encryption and data authentication of the MAC PDU payload as specified by that SA. When receiving a MAC PDU on a connection mapped to an SA, the receiver shall perform decryption and data authentication of the MAC PDU payload, as specified by that SA.

The Advanced Generic MAC Header and extended headers shall not be encrypted.  The receiver determines whether the payload in the MAC PDU is encrypted or not from the Flow ID in the AGMH or CMH for the transport connections. In case of control connection MAC PDU, control connection payload in the MAC PDU is encrypted if the Flow ID in AGMH is set to 0x1. Control connection payload in the MAC PDU is not encrypted, if the Flow ID in AGMH is set to 0x0. The encryption information needed to decrypt a payload at the receiving station is present at the beginning and at the end of the connection payload. For e.g. in case of AES CCM, PN & EKS are present at the beginning of connection payload and ICV is appended at the end of the connection payload in MAC PDU as shown in Figure 394.

[Change 3: Modify the section 16.2.4.5, page 125, line 45-55 as follows]

If multiple connection payloads are transmitted in same burst and the connections are mapped to same SA then multiple connection payload may be multiplexed before encryption and multiplexed payload is encrypted together. The receiver shall perform the decryption and data authentication on the multiplexed payload, as specified by the SA. The receiver determines whether the payloads in the MAC PDU is encrypted or not from the Flow ID in the AGMH for the transport connections. The encryption information needed to decrypt the multiplexed payload at the receiving station is present at the beginning of the first connection payload and at the end of the last connection payload. For e.g. in case of AES CCM, PN & EKS are present at the beginning of connection payload 1 and ICV is appended at the end of the connection payload n in MAC PDU as shown in Figure 395.

[Change 4: Modify the section 16.2.5.2.2, page 140, lines 17-21 as follows]

The MAC control fragment extended header is used only for management flows. The Flow ID EC bit in the AGMH Fragment extended header is used to indicate whether the PDU contains control message encrypted based on security level. Whether each control message is encrypted or not is decided based on the security level which the message is associated with. The indication to the receiver that the PDU is encrypted or not is indicated by the FlowID 0x0 and 0x1 in AGMH respectively.

[Change 5: Modify the section 16.2.5.3.3, page 153, lines 61-65 as follows].

The selective confidentiality protection over control messages is indicated by the Flow ID EC bit in the AGMH MCEH. Contrary to the transport flows where the established SA is applied to all data, the SA is selectively applied to the management flows. EC bit in the is used only for management flows to indicate whether PDU contains the control message that is encrypted based on control message type and its usage.

[Change 6: Modify the section 16.2.3, page 38, lines 1-15 as follows]

16.2.3 MAC Control messages

The peer-to-peer protocol of MAC layers in ABS and AMS communicate using the MAC control messages to perform the control plane functions. MAC control messages shall be carried in a MAC PDU to be transported in broadcast, unicast or random access connections. There is a single unicast Control connection. HARQ shall be enabled for MAC control messages sent on the unicast Control connection. Encryption may be enabled for unicast MAC control messages. MAC control messages may be fragmented. Table 673 lists the MAC control messages that shall be defined in the ASN.1 format, as shown in <<<Appendix X>>>. The indication to the receiver whether that the PDU is encrypted or not is indicated by the FlowID 0x0 and 0x1 EC=1 in AGMH respectively MCEH extended header. Whether the encryption is applied on a MAC control message or not shall be determined by the message type and MAC procedure context, which is define in Table 673. A messages included in a PDU whose encryption status indicated by the FlowID in AGMH EC bit value does not match the combined message type and corresponding context defined in the Table 673 shall be discarded.

# Reference

[1] IEEE P802.16m/D3, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”