| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Length Field Description in FPEH (section 16.2.2.2.1) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Anil Agiwal, Youngbin Chang, Rakesh Taori, Jungje Son Samsung Electronics | anilag@samsung.com, yb.chang@samsung.com, *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-09/0073, IEEE 802.16 Working Group Letter Ballot Recirc #30b / Topic: FPEH Format (Section 16.2.2.2.1) |  |
| Abstract | This contribution clarifies the length field description in FPEH format |  |
| Purpose | To be discussed and adopted by TGm for P802.16m/D4. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Length Field Description in FPEH (section 16.2.2.2.1)

Anil Agiwal, Youngbin Chang, Rakesh Taori, Jungje Son

Samsung Electronics

# Introduction

Length field description in FPEH format in D3 [1] is not clear. The length field description needs to be clarified for both multiplexing and non multiplexing case.

# Proposed text

[Modify the table 663, page 29, as follows]

Table 663: FPEH Format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| FPEH (){ |  |  |
| Type | 4 | FPEH Type |
| FC | 2 | Fragmentation Control bits (see Table 664) |
| If (MAC Header == AGMH) { |  |  |
| SN | 10 | SN is maintained per connection. For non ARQ connection, 'SN' represents the MAC PDU Payload Sequence Number and the 'SN' value increments by one (modulo 1024) for each MAC PDU. For ARQ connection, 'SN' represents the ARQ block sequence number. |
| AFI | 1 | ARQ feedback IE indicator 0 = ARQ feedback IE is not present in the MAC PDU 1 = ARQ feedback IE follows after FPEH |
| AFP | 1 | ARQ feedback poll indicator 0 = No ARQ feedback poll 1 = ARQ feedback poll for the connection indicated in AGMH or MEH if multiplexing is used |
| RI | 1 | ARQ Re-arrangement information indicator 0= no ARQ re-arrangement 1= ARQ re-arrangement |
| If (RI == 1) { |  |  |
| LSI | 1 | Last ARQ sub-block indicator 0 = indicating the last ARQ sub-block from the single ARQ block is not included in this MAC PDU 1 = indicating the last ARQ sub-block from the single ARQ block is included in this MAC PDU |
| SSN | 8 | SUB-SN of the first ARQ sub-block |
| } |  |  |
| } |  |  |
| Do { |  |  |
| End | 1 | Indication of more information 0 = Indicating another 'Length' and 'End' fields are followed 1 = Indicating no more 'Length' and 'End' fields are followed |
| If (End == 0) { |  |  |
| Length | 11 | This field indicates the length of SDU or SDU fragment in a MAC PDU payload. If a MAC PDU payload consists of 'N' SDU/SDU fragments, N-1 'Length' fields are present in FPEH. These to represent the length of first 'N-1' SDU/SDU fragments #1 to #N-1 in payload. The length of the last SDU or SDU fragment in the payload is = MAC PDU payload length - sum of 'N-1' length fields in FPEH. No multiplexing: Length of last SDU/SDU fragment = Length of MAC PDU (given by length field in GMH/CMH & MLEH) - Length of GMH/CMH – Extended header group length (if present) - Length of PN& EKS (3bytes, if present) - Length of ICV ( if present) – sum of length of SDU/SDU fragment s #1 to #N-1 With multiplexing: Length of last SDU/SDU fragment = Length of connection payload corresponding to this FPEH (see table 666, MEH format) – sum of length of SDU/SDU fragments #1 to #N-1 |
| } |  |  |
| } while (!End) |  |  |
| Padding | Variable | For byte alignment |
| } |  |  |

# Reference

[1] IEEE P802.16m/D3, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”