| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ACK Timer Operation (16.2.21) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Anil Agiwal, Youngbin Chang, Rakesh Taori, Jungje Son Samsung Electronics | anilag@samsung.com, yb.chang@samsung.com, *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-09/0073, IEEE 802.16 Working Group Letter Ballot Recirc #30b / Topic: Management Message Reliability (Section 16.2.21) |  |
| Abstract | This contribution clarifies the ACK Timer operation in Management Message Reliability |  |
| Purpose | To be discussed and adopted by TGm for P802.16m/D4. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

ACK Timer Operation (section 16.2.21)

Anil Agiwal, Youngbin Chang, Rakesh Taori, Jungje Son

Samsung Electronics

# Introduction

In the current ACK Timer description in D3, MAC has to wait for indication from Physical layer to determine that message or message fragment transmission is completed. This has following issues:

1) When local nack is not enabled, a mechanism is needed between PHY and MAC to indicate message transmission completion

2) When local nack is enabled, PHY has to send indication even when HARQ ACK is there. This happens 99% of time and is an overhead in the system.

# Proposed text

[Modify the lines 49-62 on page 284, section 16.2.21 as follows:]

The transmitter may request receiver to send a MAC layer acknowledgement to determine the status of the transmitted control message. When a MAC layer acknowledgement is used, the transmitter shall set the Polling bit to 1 in MCEH of the MAC PDU containing the complete control message or the last pending fragment of the control message. and The transmitter shall start a ACK timer immediately after completing the transmission of unfragmented control message or all fragments of the control message, to wait for AAI_MSG_ACK message or Message ACK extended header. If the local NACK is received for a MAC PDU carrying the MCEH with polling bit set to 1, transmitter shall retransmit the polling while retransmitting the last pending fragment. The ACK timer is stopped if the local NACK is received. The ACK timer stopped by local NACK shall be started again while retransmitting the last pending fragment. The ACK timer is stopped if AAI_MSG_ACK message or Message ACK extended header is received from the receiver. If the ACK timer expires before an acknowledgement is received or the retransmission timer expires, the transmitter may initiate the retransmission of the entire message. The transmitter shall stop the ACK timer if the retransmission timer expires. The transmitter shall reset the retransmission timer, if it initiates the retransmission, after the ACK timer or retransmission timer expiry.

# Reference

[1] IEEE P802.16m/D3, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”