| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Piggybacked BR for One Service Flow (16.2.2) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Jaesun Cha, Soojung Jung, Eunkyung Kim, Sungcheul Chang, Hyun Lee, Kwangjae Lim, Chulsik Yoon ETRI | jscha@etri.re.kr, *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirculation #30b on P802.16m/D3 |  |
| Abstract | This contribution proposes to define a new EH for piggybacked BR for one service flow |  |
| Purpose | To be discussed and adopted by TGm for 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Piggybacked BR for One Service Flow (16.2.2)

Jaesun Cha, Soojung Jung, Eunkyung Kim, Sungcheol Chang, Hyun Lee, Kwangjae Lim, Chulsik Yoon

ETRI

# Introduction

PBREH is used when an AMS performs piggybacked bandwidth request for one or more flows. Our Question is whether the AMS performs piggybacked BR for multiple flows frequently or not.

The length of the current PBREH is 4 bytes including padding bits for byte alignment. If PBREH is used only for the flow indicated by AGMH, then ‘FID’ and ‘End’ fields can be removed and the total length of PBREH will be 3 bytes.

If Piggybacked BR for multiple flows is not a usual case, then 1 byte UL resource is wasted whenever PBREH is used.

In this contribution, we propose to change the PBREH to be used only for piggybacked BR for the service flow indicated by AGMH. In addition, we propose to define a new EH, of which format is exactly same as that of the current PBREH, to be used only for piggybacked BR for multiple flows.

# Text Proposal

---------------------------------------------------------Start of the Text----------------------------------------------------------

[Modify section 16.2.2.2.8 as follows]

16.2.2.2.8 Piggybacked bandwidth request extended header (PBREH)

PBREH shall be used when an AMS performs piggybacked bandwidth request for the flow indicated by AGMHone or more flows. Table 671 is the format of PBREH.

Table 671 – PBREH format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| PBREH () { |  |  |
| Type | 4 | PBREH type |
| Do { |  |  |
| FID | 4 | Flow Identifier |
| Request type | 1 | 0: aggregate 1: incremental |
| BR Size | 19 | Amount of bandwidth requested |
| End | 1 | Indication of more information 0: another “FID”, Request type”, and “BR size” fields follow 1: no more “FID”, “Request type” or “BR size” fields follow |
| } while (!End) |  |  |
| Reserved | variable | Padding for byte alignment |
| } |  |  |

[Add the following new section after section 16.2.2.2.9]

16.2.2.2.x Extended piggybacked bandwidth request extended header (EPBREH)

EPBREH shall be used when an AMS performs piggybacked bandwidth request for one or more flows. Table xxx is the format of EPBREH.

Table xxx – EPBREH format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| EPBREH () { |  |  |
| Type | 4 | EPBREH type |
| Do { |  |  |
| FID | 4 | Flow Identifier |
| Request type | 1 | 0: aggregate 1: incremental |
| BR Size | 19 | Amount of bandwidth requested |
| End | 1 | Indication of more information 0: another “FID”, Request type”, and “BR size” fields follow 1: no more “FID”, “Request type” or “BR size” fields follow |
| } while (!End) |  |  |
| Reserved | variable | Padding for byte alignment |
| } |  |  |

---------------------------------------------------------End of the Text----------------------------------------------------------