| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to MAEH (16.2.2) |  |
| Date Submitted | 2010-01-13 |  |
| Source(s) | Jaesun Cha, Soojung Jung, Eunkyung Kim, Sungcheul Chang, Hyun Lee, Kwangjae Lim, Chulsik Yoon ETRI Lei Zhang, Yang Liu, Mary Chion ZTE Corporation | jscha@etri.re.kr, *<http://standards.ieee.org/faqs/affiliationFAQ.html> zhang.lei13@zte.com.cn |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirculation #30b on P802.16m/D3 |  |
| Abstract | This contribution proposes to modify MAEH to unify multiple acknowledgement methods into one method. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to MAEH (16.2.2)

Jaesun Cha, Soojung Jung, Eunkyung Kim, Sungcheol Chang, Hyun Lee, Kwangjae Lim, Chulsik Yoon

ETRI

Lei Zhang, Yang Liu, Mary Chion

ZTE Corporation

# Introduction

In the current draft, there are various acknowledgment methods as follow;

| Transmitting entity | Receiving entity |
| --- | --- |
| A control message with poll bit in MCEH set to 1 | MAEH or AAI_MSG-ACK |
| SCEH with Response indication = 0 | SCEH with Response indication = 1 |
| SCH with Response indication = 0 | SCH with Response indication = 1 |
| Service specific BR without STID with SCID change indicator = 1 | TBD |

Regarding the acknowledgement methods, we have to consider the following things;

- The length of signaling header is fixed to 6 bytes regardless of its contents. It’s a waste of bandwidth to use 6-byte signaling header just to transmit acknowledgment.

- Whenever we define a new EH or signaling header which requires acknowledgement, we have to add one more parameter such as response indication to distinguish the request and response. Or we have to modify the structure of the existing headers to support acknowledgement.

- How to acknowledge service specific BR without STID is still TDB. To transmit BR header in DL direction doesn’t make sense.

In order to solve the aforementioned problems, we propose to modify the existing MAEH to be used for a unified acknowledgment method for all kinds of requests.

# Text Proposal

---------------------------------------------------------Start of the Text----------------------------------------------------------

[Modify section 16.2.2.2.4 as follows]

16.2.2.2.4 MAC Control Message ACK Extended Header

The MAC ControlMessage ACK extended header format is defined in Table 667. This header may be used by ABS and AMS to indicate the reception of a specific, previously received MAC control message, signaling header or extended header. When an ABS or AMS receives a MAC control message or MAC control message fragment with the Polling bit set to 1 in the MCEH, the ABS or AMS shall transmit an MAEH as an acknowledgement after receiving the complete message with the SN of the MAC control message PDU or the SN of the last received fragment if fragmented. When an ABS or AMS receives a signaling header or extended header which requires an acknowledgment, the ABS or AMS shall transmit an MAEH as an acknowledgement and the MAEH shall contain the type of the received header and LSB of frame number of the frame on which the header is received.

Table 667 – MAEH Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MAEH () { |  |  |
| Type | 4 | MAEH type |
| ACK_type | 1 | 0: acknowledgement to a control message 1: acknowledgement to a signaling header or an extended header |
| If (ACK_type == 0) { |  |  |
| Channel ID | 1 | Channel ID that the MAC control message is received. |
| ACK_SN | 8 | SN retrieved from the MCEH of the MAC PDU with the Polling bit set to 1 |
| Reserved | Variable2 | For byte alignment |
| } |  |  |
| else { |  |  |
| ACK_header_type | 5 | Bit #0: indicates the header type to be acknowledged; 0: signaling header 1: extended header Bit #1-4: header type (See Table 656 & 662) |
| Frame_LSB | 6 | LSB of frame sequence. Indicates the frame on which the header to be acknowledged is transmitted. |
| } |  |  |
| } |  |  |

[Modify section 16.2.3.32 as follows]

16.2.3.32 AAI_MSG-ACK

The ABS and AMS may use AAI_MSG-ACK to indicate the reception of a MAC management, signaling header or extended header. When receiving a message over management connection with Polling bit set to 1 in MCEH, the ABS and AMS shall transmit AAI_MSG-ACK as an acknowledgement to the reception of the message. When an ABS or AMS receives a signaling header or extended header which requires an acknowledgment, the ABS or AMS shall transmit an MAEH as an acknowledgement and the MAEH shall contain the type of the received header and LSB of frame number of the frame on which the header is received.The AAI_MSG-ACK shall include the following information:

- SN retrieved from MCEH of the received MAC PDU

- Channel ID that the MAC management message is received

Table xxx – parameters for AAI_MSG-ACK

| Name | Value | Usage |
| --- | --- | --- |
| ACK type | 0: acknowledgement to a control message 1: acknowledgement to a signaling header or an extended header | It shall be included in AAI_MSG-ACK message |
| ACK_SN | SN retrieved from MCEH of the received MAC PDU | It shall be included when ACK type is equal to 0. |
| Channel ID | Channel ID that the MAC management message is received | It shall be included when ACK type is equal to 0. |
| ACK_header_type | Bit #0: indicates the header type to be acknowledged; 0: signaling header 1: extended header Bit #1-4: header type (See Table 656 & 662) | It shall be included when ACK type is equal to 1. |
| Frame_LSB | LSB of frame sequence. Indicates the frame on which the header to be acknowledged is transmitted. | It shall be included when ACK type is equal to 1. |

---------------------------------------------------------End of the Text----------------------------------------------------------