| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modifications to Broadcast and Unicast Parameters of Sounding Channel (Section 16.2.3.29 of the IEEE 802.16m/D3) |  |
| Date Submitted | 2009-12-30 |  |
| Source(s) | Alexei Davydov | alexei.davydov@intel.com |
| Re: | Category: LB30b / Area: Chapter 16.2.3 |  |
| Abstract | The contribution proposes modifications to broadcast and unicast parameters of sounding channel |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 4) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal on Broadcast Parameters for Sounding Channel (Section 16.2.3.29 of the IEEE 802.16m/D3)

Alexei Davydov

Intel Corporation

# Introduction

In session 64 it has been decided to move sounding subframe indication parameter to SFH SP-1 and to broadcast the remaining parameters such as multiplexing type, FDM decimation value and CDM code length using AAI_SCD message. To improve the cross correlation properties of a baseline sequence in FDM and allow operation in reuse-3 for CDM it has been also decided to broadcast the baseline sequence shift value, but the parameter ‘u’ was placed in SFH SP-3 instead of AAI_SCD message. Also for sounding subframe indication parameter only three bits were allocated in SFH SP-1 without description of encoding of these bits. Finally, in Sounding Command AMAP IE maximum 12 bits were reserved for indication of sounding subbands. In this case for channels employing 2048 FFT one sounding subband occupies two physical subbands, which may lead to inefficient usage of the sounding channel resources especially for deployments with reuse-3.

# Proposed remedies

In this contribution the following remedies are proposed:

- The baseline sequence shift parameter is moved from SFH SP-3 to AAI_SCD message

- A detailed description of the sounding subframe field has been provided

- Support of 24 subband bitmap is added by means of removing antenna bitmap option

- The list of supported decimation offsets and code lengths has been extended to 8 values

- Antenna indexing is aligned across all sections related to sounding operation

# Text proposal for inclusion in the 802.16m/D4

--------------------------------------------------  Text Start #1  ------------------------------------------------

{Modify contents of AAI_SCD message specified on page 95, line 24, section 16.2.3.29 as follows}

-- ASN1START

AAI_SCDMessage :: SEQUENCE {

Change Configuration ChangeINTEGER (0..15)

BS_Restart_CountINTEGER (0..15)

SA_PreamblePartitionforBStype

Trigger TLV encoding :: = SEQUENCE{}

DefaultHORSSI_CINRaveraggingparameters :: = SEQUENCE {}

NormalizedCINR :: = SEQUENCE {}

OL_MIMO_Parameters :: = SEQUENCE {

OL-Region-ONINTEGER (0..1) OPTIONAL

OL-Rank1-ConfigurationINTEGER (0..7) OPTIONAL

SB-OL-Region-2SizeINTEGER (0..15) OPTIONAL

}

Parameters_GRA :: = SEQUENCE {}

PeriodicRNGParameters :: = SEQUENCE {}

GAMMA_IOT_FP0INTEGER (0..15) OPTIONAL

GAMMA_IOT_FP1INTEGER (0..15) OPTIONAL

GAMMA_IOT_FP2INTEGER (0..15) OPTIONAL

GAMMA_IOT_FP3INTEGER (0..15) OPTIONAL

Alpha (a)INTEGER (0.. 7) OPTIONAL

Beta (ß)INTEGER (0..1) OPTIONAL

SINRminINTERGER (0..15) OPTIONAL

Sounding multiplexing type 	  	CHOICE {

fdm 		FDM

cdm		CDM

}

Decimation value D/Max Cyclic Shift Index P for sounding	ENUMERATED

{n4, n6, n8, n9, n12, n16, n18, n36}

Shift_Value_u_for_sounding_symbol		(0..255)

RELAY_Support_Parameters :: = SEQUENCE {

16m_Relay_zone_AMS_allocation_indicatorINTEGER (0..1) OPTIONAL

}

}

-- ASN1STOP

--------------------------------------------------  Text End #1  ------------------------------------------------

--------------------------------------------------  Text Start #2  ------------------------------------------------

{Modify contents of Table 801 on page 379, line 6, section 16.3.6.5.1.2 as follows}

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP1 IE format () { |  |  |
| MSB of superframe number | 8 | Remaining bit of SFN except LSB of SFN in P-SFH |
| ... | ... | ... |
| Cell bar information | 1 | If Cell Bar bit = 1, this cell is not allowed for any new initial entry |
| Uplink subframes for sounding | 3 | This value represents the UL subframe indexes where the sounding symbols are located. This value represents the number of uplink subframes with sounding symbols. 0b000 – no sounding symbols 0b001 – 1 subframe 0b010 – 2 subframes 0b011 – 3 subframes 0b100 – 4 subframes 0b101-111 – reserved The sounding symbols shall be placed in subframes in accordance to their type. First, sounding symbols shall be allocated in uplink subframes of type 2 starting from the first in time subframe of type 2. If the number of uplink subframe of type 2 is less than the number of subframes for sounding, sounding symbols shall be allocated in the subframes of other types in the following order: type 1, type 4. For these types of uplink subframes sounding symbols shall be allocated in the similar way as for type 2. Type 3 uplink subframes are not used for sounding. |
| Reserved | TBD |  |
| } |  |  |

--------------------------------------------------  Text End #2  ------------------------------------------------

--------------------------------------------------  Text Start #3  ------------------------------------------------

{Modify content of Table 803 on page 381 line 48 as follows}

| Syntax | Size(bits) | Notes |
| --- | --- | --- |
| S-SFH SP3 IE format () { |  |  |
| Start superframe offset where new SP3 information is used | 2 |  |
| … |  |  |
| Bandwidth request backoff end | 4 | Final backoff window size for contention BRs, expressed as a power of 2. Values of n range 0-15 |
| SZ Shift Value | 8 | Shift value (u) for sounding symbol |
| Reserved | TBD |  |
| } |  |  |

--------------------------------------------------  Text End #3 ------------------------------------------------

--------------------------------------------------  Text Start #4  ------------------------------------------------

{Modify content of Table 822 and text on page 429 line 16 as follows}

Table 822—UL Sounding Command A-MAP IE Syntax

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| UL Sounding Command IE format () { |  |  |
| A-MAP IE Type | 4 |  |
| Sounding subframe | 3 | Indicates the sounding subframe |
| Sounding subband bitmap | variable max. 1224 | - FFT size dependant |
| If (Multiplexing type == 0) { |  |  |
| Decimation offset d | 5 | Unique decimation offset |
| } else { |  |  |
| Cyclic time shift m | 5 | Unique cyclic shift |
| } |  |  |
| Periodicity (p) | 3 | 0b000 = Single command, not periodic, or terminate the periodicity. Otherwise, repeat sounding once per 2(p-1) frames, where p is decimal value of the periodicity field |
| Antenna switching | 1 | 0b0: Antenna switching 0b1: No antenna switching |
| If (Antenna switching == 1) { |  |  |
| Transmit antenna bitmap | 4 | Indicates an active set of transmit antennas |
| } |  |  |
| Padding | variable | Padding |
| } |  |  |

*A 16bit CRC is generated based on the contents of the UL Sounding Command A-MAP IE. The CRC is masked by the Station ID.

Table 684 specifies the fields of UL Sounding Command A-MAP IE used by the ABS to request sounding transmission by the AMS. Decimal equivalent of the sounding subframe indicates the subframe number with sounding symbol (the first subframe in frame is indexed 0). The sounding subband bitmap field is used to indicate the sounding subbands used in the sounding allocation. For that purpose, the Nused contiguous subcarriers are divided into sounding subbands, where each sounding subband compromises N1 * Nsc adjacent subcarriers with N1= 4 and Nsc = 18 for NFFT = 512, 1024 and Nsc = 36 for NFFT = 2048. The MSB of the Sounding subband bitmap field corresponds to the sounding subband with lowest subcarrier indexes. The three periodicity bits are used to indicate the MS to periodically repeat the sounding transmission. Setting periodicity bits to 0b000 indicates a single sounding command or terminates the sounding if periodic sounding command is being performed.

For Antenna switching flag equals to 0 the MS sounds with antenna switching, while for Antenna switching flag equals to 1 the AMS sounds all active transmit antennas. If Antenna switching field equals to 1 then the ith active antenna of the MS corresponds to the actual decimation offset g = d + i - 1 for multiplexing type 0 or to the actual cyclic shift index n = m + i - 1 for multiplexing type 1.  The AMS transmit antennas are indexed starting from one. To indicate a set of active antennas Transmit antenna bitmap field is used, where MSB bit of the field corresponds to the first antenna of AMS. Before assignment of the actual decimation offset value g or cyclic shift index n to the transmit antenna, the active antennas of AMS shall be renumbered starting from one.

--------------------------------------------------  Text End #4 ------------------------------------------------

--------------------------------------------------  Text Start #5  ------------------------------------------------

{Modify text on page 530, line 51, section 16.3.9.2.3.2 as follows}

For sounding with antenna switching, the AMS shall transmit sounding symbol with the ith antenna (01, …, Nt-1) on frames n = j×T+i-1, where n = 0 corresponds to the frame where UL sounding command A-MAP IE is received, T is periodicity in UL sounding command A-MAP IE, and j is a running index ( j = 0, 1, 2, … for T¹ 0 and j = 0 for T= 0). For sounding with antenna switching and periodical sounding allocation (T ¹ 0), the assigned periodicity T shall be larger or equal to the number of AMS transmit antennas chains Nt.

--------------------------------------------------  Text End #5 ------------------------------------------------