| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal on CQI for Sounding Based MIMO (Section 16.2.3.29 of the IEEE 802.16m/D3) |  |
| Date Submitted | 2009-12-30 |  |
| Source(s) | Alexei Davydov Huaning Niu Yuan Zhu | alexei.davydov@intel.com |
| Re: | Category: LB30b / Area: Chapter 16.2.3 |  |
| Abstract | The contribution proposes CQI for sounding based MIMO |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 4) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal on CQI for Sounding Based MIMO (Section 16.3.7 of the IEEE 802.16m/D3)

Alexei Davydov, Huaning Niu, Yuan Zhu

Intel Corporation

# Introduction

For sounding based DL MIMO the ABS can measure the DL channel response, but there is no information about interference level at the AMS. To assist operation of DL MIMO with sounding based beam forming CQI feedback from AMS is required.

# Problem Statement

In contrast to codebook based precoding, AMS doesn’t have information about type of the beam forming used at the ABS in sounding based DL MIMO. So, it is not possible for AMS to feedback the ‘correct’ CQI value for that mode. It should be noted that channel response is known at the ABS and only interference level is required. A simple approach to provide such information to the ABS is to use wideband CQI metric using MFM = 0. The interference level in this case can be deduced by the ABS from the reported in straightforward way.

# Text proposal for inclusion in the 802.16m/D4

--------------------------------------------------  Text Start #1  ------------------------------------------------

{Modify text on page 456, line 14, section 16.3.7.2.5.4 as follows}

MIMO feedback mode 0 is used for the OL-SU SFBC and SM adaptation in diversity permutation. The AMS estimates the wideband CQI for both SFBC and SM, and reports the CQI and STC Rate. STC Rate 1 means SFBC with precoding and STC Rate 2 means rank-2 SM with precoding. MIMO feedback mode 0 may also be used for CQI feedback for sounding based beamforming defined in section 16.3.7.2.5.7. The AMS shall estimate the wideband CQI for SFBC mode (MaxMt = 0b00), and report the CQI.

--------------------------------------------------  Text End #1  ------------------------------------------------

--------------------------------------------------  Text Start #2  ------------------------------------------------

{Modify contents of Table 839 specified on page 457, line 2, section 16.3.7.2.5.5 as follows}

Table 839—MIMO feedback modes

| MIMO Feedback Mode | Description and type of RU | Feedback content | Supported MIMO transmission mode outside the OL region (when Measurement Method Indication = 0b0) | Supported MIMO transmission mode inside the OL region (when Measurement Method Indication = 0b1) |
| --- | --- | --- | --- | --- |
| 0 | OL SU MIMO SFBC/SM (Diversity: DLRU, NLRU) Sounding based CL SU and MU MIMO | 1. STC Rate 2. Wideband CQI | MIMO mode 0 and MIMO mode 1. Flexible adaptation between the two modes STC Rate = 1: SFBC CQI 2 <= STC Rate <= 4: SM CQI In DLRU: Mt=2 for SM. In NLRU: Mt >= 2 for SM For sounding based CL SU MU MIMO, STC Rate = 1: SFBC CQI | MIMO mode 0 and MIMO mode 1. Flexible adaptation between the two modes STC Rate = 1: SFBC CQI STC Rate = 2: SM CQI In DLRU only. |
| 1 | OL SU MIMO SM (Diversity: NLRU) | 1. STC Rate 2. Wideband CQI | MIMO mode 1 1 <= STC <= 4 | MIMO mode 5 STC Rate = 1/2 |
| ... | ... | ... | ... | ... |
| 7 | CL MU MIMO (Diversity: NLRU) | 1. Wideband CQI 2. Wideband PMI 3. Wideband correlation matrix | MIMO mode 4 | N/A |

--------------------------------------------------  Text End #2  ------------------------------------------------

--------------------------------------------------  Text Start #3  ------------------------------------------------

With an MFM supporting SU MIMO operation, MaxMt indicates the maximum STC rate allowed in the feedback report of the AMS. The ABS also uses MaxMt to make sure that the STC rate feedback is in accordance to STC rate for the AMS. For an MFM requesting OL MIMO feedback for operation with a diversity allocation, setting MaxMt = 2 enforces feedback for DLRU, otherwise the AMS may feedback STC rate up to 4. With MFM 0, MaxMt=1 indicates that the AMS shall feedback the wideband CQI for SFBC.

--------------------------------------------------  Text End #3  ------------------------------------------------