| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal to remove MFM 1 outside OL region (Section 16.3.7 of the IEEE 802.16m/D3) |  |
| Date Submitted | 2009-12-30 |  |
| Source(s) | Alexei Davydov Andrey Pudeyev Alexander Maltsev Yang-seok Choi Intel | alexei.davydov@intel.com |
| Re: | Category: LB30b / Area: Chapter 16.3.7 |  |
| Abstract | The contribution proposes to remove MFM 1 |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 4) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal to remove MFM 1 outside OL region (Section 16.3.7 of the IEEE 802.16m/D3)

Alexei Davydov, Andrey Pudeyev, Alexander Maltsev, Yang-seok Choi

Intel Corporation

# Introduction

There are two MFM that are used to support OL MIMO in N-LRU allocations outside OL region: MFM 0 and MFM 1. Both MFM supports wideband CQI feedback for STC rate up to 4. The difference is in CQI feedback for STC rate 1. In MFM=0, CQI for MIMO mode 0 is used, while in MFM=1 CQI for MIMO mode 1 with STC rate 1 is used. To simplify standard and minimize the number of options removal of MFM 1 outside OL region is proposed.

# Simulation analysis

To evaluate OL MIMO performance with MFM 0 and 1 in N-LRU allocation a system level simulations were carried out in UMa test environment with different antenna spacing configurations. Interference aware receiver has been assumed for the analysis. Simulation results for 4x2 antenna configuration, 0.5 wavelength antenna spacing and 10 MHz channel are summarized in Table 1.

Table 1 System level performance for MFM 0 and 1

| Scenario (MaxMt = 2) | Cell SE / Cell Edge SE [bps/hz/sect] |
| --- | --- |
| MFM 0 | 1.61 / 0.098 |
| MFM 1 | 1.59 / 0.094 |

As it can be seen the MFM 0 and 1 shows a similar performance.

The same analysis has been carried out for MFM 0 and 1 on the link level for MaxMt = 1. The performance results are presented on Figure 1 for the noise limited scenario.

Figure 1. Link level performance comparison for MFM 0 and 1

It can be seen form the simulation results that MFM 0 outperforms MFM 1.

# Text proposal for inclusion in the 802.16m/D4

--------------------------------------------------  Text Start #1  ------------------------------------------------

{Modify text on page 456, line 19, section 16.3.7.2.5.4 as follows}

MIMO feedback mode 1 is used for the OL-SU CDR with STC rate 1/2 SM with rank adaptation in diversity permutation. The transmission rank is determined by the STC Rate. STC Rate 1 means the rank-1 precoding.

--------------------------------------------------  Text End #1 ------------------------------------------------

--------------------------------------------------  Text Start #2  ------------------------------------------------

{Modify contents of Table 839 on page 457, line 20, section 16.3.7.2.5.5 as follows}

| MIMO Feedback mode | Description and type of RU | Feedback content | Supported MIMO transmission mode outside the OL region (when Measurement Method Indication = 0b0) | Supported MIMO transmission mode inside the OL region (when Measurement Method Indication = 0b1) |
| --- | --- | --- | --- | --- |
| 0 | OL SU MIMO SFBC/SM (Diversity: DLRU, NLRU) | 1. STC Rate 2. Wideband CQI | MIMO mode 0 and MIMO mode 1. Flexible adaptation between the two modes STC Rate = 1: SFBC CQI 2 ≤ STC Rate ≤ 4: SM CQI In DLRU: Mt=2 for SM. In NLRU: Mt≥2 for SM | MIMO mode 0 and MIMO mode 1. Flexible adaptation between the two modes STC Rate = 1: SFBC CQI STC Rate = 2: SM CQI In DLRU only. |
| 1 | OL SU MIMO SM (Diversity: NLRU) | 1. STC Rate 2. 1. Wideband CQI | MIMO mode 1 1 ≤ STC Rate ≤ 4 n.a. | MIMO mode 5 STC Rate = 1/2 |
| 2 | OL SU MIMO SM (localized: SLRU) | 1. STC Rate 2. Subband CQI 3. Subband Selection | MIMO mode 1 1 ≤ STC Rate ≤ 8 | MIMO mode 5 STC Rate = 1/2 |
| … |  |  |  |  |

--------------------------------------------------  Text End #2 ------------------------------------------------