| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for management of multiple AMS's flows within same resource allocation group (16.2.9.3/16.2.9.4/16.2.3.44) |  |
| Date Submitted | 2009-12-29 |  |
| Source(s) | Yaniv Weizman Alvarion | yaniv.weizman@alvarion.com |
| Re: |  |  |
| Abstract | Proposal for management of multiple AMS's flows within same resource allocation group (16.2.9.3/16.2.9.4/16.2.3.44) |  |
| Purpose | To be discussed and adopted by TGm into P802.16m/D4 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for management of multiple AMS's flows within same resource allocation group (16.2.3.44/16.2.9.3/16.2.9.4/)

- Introduction

Support the option to add multiple AMS flows into same group when using group resource allocation. The AMS will be allocated multiple User Bitmap Indexes, each for a flow. The change will allow scheduling of several allocations of different flows of AMS having different characteristics within same group.

- Proposed Harmonized Text

[Modify the following text in Section 16.2.3.44]

Group Configuration MAC Control message is used for group management when using Group Resource Allocation. This message is used to inform an AMS that one of its flows has been added to or deleted from a GRA group. Note that one or more AMS's flows can be added into same group. A Group Configuration MAC control message should be sent per AMS flow addition to or deletion from a group. The format of the message is given in Table 722.

[Modify the following text in Section 16.2.9.3.1.1]

When an ABS decides to use group resource allocation for an AMS, the ABS adds the AMS into an appropriate group among existing groups. If the existing groups are not appropriate to the AMS, the ABS may form a new group. ABS shall indicate group configuration information via Group Configuration MAC control message which includes the Group ID of the group to which the AMS is added and the one or more assigned User Bitmap Indices to the AMS.

[Modify the following text in Section 16.2.9.3.1.2]

Upon receiving Group Configuration MAC control message, the AMS knows the group ID of the group to which it is added, the periodicity of group resource allocation and the AMS's one or more indices in the group's user bitmap. In addition, the AMS receives the required information to interpret the assigned MIMO mode, HARQ burst size and resource size from the bitmaps in the corresponding Group Resource Allocation A-MAP IE. Once the AMS successfully acknowledges a Group Configuration MAC control message, starting the next frame, the AMS shall monitor its allocation in the corresponding Group Resource Allocation A-MAP IE until it is deleted from the group.

[Modify the following text in Section 16.2.9.3.2.1]

Once the Group configuration MAC control message for deletion is sent to an AMS, no allocations shall be provided to the AMS in the group in the subsequent frames. After sending the deletion information, the ABS shall wait for an ACK from the AMS. The ABS shall not allocate the corresponding bitmap positions to another AMS until an ACK for deletion has been received.

[Modify the following text in Section 16.2.9.4]

An ABS shall provide one or more contiguous allocations to the AMSs belonging to a group, which have the corresponding bits in the user bitmap set to '1'. The order of resource allocation shall be the same as the order in which they appear in the group's user bitmap. If there is no transmission for a given AMS in a certain AAI subframe, then the ABS shall set the corresponding bits in the user bitmap to '0'.

When an AMS receives a Group Resource Allocation IE in which the corresponding bits in user bitmap is set to '1', then the AMS shall decode the remaining bitmaps to determine other attributes of the allocated resource. An AMS shall determine the location of its allocations by counting the resource allocation sizes of other AMSs appearing before it in the user bitmap. If an AMS does not receive the Group Resource Allocation IE in any of the AAI subframes of a frame in which the IE was expected, then the AMS shall assume no allocations for the group in that frame.

[Modify the following text in Section 16.2.9.4.1]

The ABS uses bitmaps to signal resource allocation information for AMSs within a group. These bitmaps are sent in the Group Resource Allocation A-MAP IE. The first bitmap is the User Bitmap which uses 1 or more bits per AMS to signal which users are scheduled in the frame. The user bitmap size can be 8, 16 or 32 bits. Each AMS belonging to the group shall be assigned one or more unique indices in the User Bitmap of that group. The bitmap size for a given group shall remain fixed. As AMSs are deleted from the group, some bit indices in the user bitmap may become empty or unassigned. These empty bits may be assigned to new users as they are added to the group.

[Add the following table in Section 16.2.9.3.1]

Note that an AMS can be added into a group with one or more User Bitmap Indices to allow multiple allocations for AMS within a subframe using a single group resource allocation A-MAP IE.