| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up for frequency partition configuration (16.3.5.2.3, 16.3.8.2.3) |  |
| Date Submitted | 2010-01-09 |  |
| Source(s) | Taeyoung Kim, Mihyun Lee Samsung Electronics | Email : ty33.kim@samsung.com |
| Re: | IEEE 802.16-09/0073: IEEE 802.16 Working Group Letter Ballot Recirc #30b Section 16.3.5.2.3 and 16.3.8.2.3 |  |
| Abstract | This contribution provides the clear table for frequency partition configuration |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean-up for frequency partition configuration (16.3.5.2.3, 16.3.8.2.3)

Taeyoung Kim, Mihyun Lee

Samsung Electronics

# Proposed Text

Update the following section in D3.

---------------------------------------------------- Proposed Text #1 ------------------------------------------------

Make the following changes to Table 777 through Table 781 in Section 16.3.5.2.3 on Page 323 beginning Line 12

| - Mapping between DFPC and frequency partitioning for 20 MHz |  |  |  |  |
| --- | --- | --- | --- | --- |
| DFPC | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| 0 | 1 : 0 : 0 : 0 | 1 | NPRU | 0 |
| 1 | 0 : 1 : 1 : 1 | 3 | 0 | NPRU * 1/3 |
| 2 | 1 : 1 : 1 : 1 | 4 | NPRU * 1/4 | NPRU * 1/4 |
| 3 | 3 : 1 : 1 : 1 | 4 | NPRU * 1/2 | NPRU * 1/6 |
| 4 | 5 : 1 : 1 : 1 | 4 | NPRU * 5/8 | NPRU * 1/8 |
| 5 | 9 : 1 : 1 : 1 | 4 | NPRU * 9/12 | NPRU * 1/12 |
| 6 | 9 : 5 : 5 : 5 | 4 | NPRU * 3/8 | NPRU * 5/24 |
| 7 | 0:1:1:0 | 2 | 0 | NPRU * 1/2 for i=1,2 0 for i=3 |
| 8 | 1:1:1:0 | 3 | NPRU * 1/3 | NPRU * 1/3 for i=1,2 0 for i=3 |
| 9-15 | Reserved |  |  |  |

| Table 778 - Mapping between DFPC and frequency partitioning for 2048 FFT size20 MHz>BW>10 MHz |  |  |  |  |
| --- | --- | --- | --- | --- |
| DFPC | Freq. Partitioning (Appro.) (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| 0 | 1 : 0 : 0 : 0 | 1 | NPRU | 0 |
| 1 | 0 : 1 : 1 : 1 | 3 | 0 | FPS1 =NPRU – 2*floor(NPRU/3) FPS2 = floor(NPRU/3) FPS3 = floor(NPRU/3) |
| 2 | 1 : 1 : 1 : 1 | 4 | NPRU-(FPCT-1)*FPSi (i>0) NPRU-3* floor(NPRU/4) | floor(NPRU/4) |
| 3 | 3 : 1 : 1 : 1 | 4 | NPRU-(FPCT-1)*FPSi (i>0) NPRU-3* floor(NPRU/6) | floor(NPRU/6) |
| 4 | 5 : 1 : 1 : 1 | 4 | NPRU-(FPCT-1)*FPSi (i>0) NPRU-3* floor(NPRU/8) | floor(NPRU/8) |
| 5 | 9 : 1 : 1 : 1 | 4 | NPRU-(FPCT-1)*FPSi (i>0) NPRU-3* floor(NPRU/12) | floor(NPRU/12) |
| 6 | 9 : 5 : 5 : 5 | 4 | NPRU-(FPCT-1)*FPSi (i>0) NPRU-3* floor(NPRU*5/24) | floor(NPRU*5/24) |
| 7 | 0:1:1:0 | 2 | 0 | NPRU/2 for i=1,2 0 for i=3 |
| 8 | 1:1:1:0 | 3 | NPRU – 2*floor(NPRU/3) | floor(NPRU/3) for i=1,2 0 for i=3 |
| 79-15 | Reserved |  |  |  |

| - Mapping between DFPC and frequency partitioning for 10 MHz |  |  |  |  |
| --- | --- | --- | --- | --- |
| DFPC | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| 0 | 1 : 0 : 0 : 0 | 1 | NPRU | 0 |
| 1 | 0 : 1 : 1 : 1 | 3 | 0 | NPRU * 1/3 |
| 2 | 1 : 1 : 1 : 1 | 4 | NPRU * 1/4 | NPRU * 1/4 |
| 3 | 3 : 1 : 1 : 1 | 4 | NPRU * 1/2 | NPRU * 1/6 |
| 4 | 5 : 1 : 1 : 1 | 4 | NPRU * 5/8 | NPRU * 1/8 |
| 5 | 9 : 5 : 5: 5 | 4 | NPRU * 3/8 | NPRU * 5/24 |
| 6 | 0:1:1:0 | 2 | 0 | NPRU * 1/2 for i=1,2 0 for i=3 |
| 7 | 1:1:1:0 | 3 | NPRU * 1/3 | NPRU * 1/3 for i=1,2 0 for i=3 |

| - Mapping between DFPC and frequency partitioning for 1024 FFT size10 MHz>BW>5 MHz |  |  |  |  |
| --- | --- | --- | --- | --- |
| DFPC | Freq. Partitioning (Appro.) (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| 0 | 1 : 0 : 0 : 0 | 1 | NPRU | 0 |
| 1 | 0 : 1 : 1 : 1 | 3 | 0 | FPS1 =NPRU – 2*floor(NPRU/3) FPS2 = floor(NPRU/3) FPS3 = floor(NPRU/3) |
| 2 | 1 : 1 : 1 : 1 | 4 | NPRU-(FPCT-1)*FPSi (i>0) NPRU-3* floor(NPRU/4) | floor(NPRU/4) |
| 3 | 3 : 1 : 1 : 1 | 4 | NPRU-(FPCT-1)*FPSi (i>0) NPRU-3* floor(NPRU/6) | floor(NPRU/6) |
| 4 | 5 : 1 : 1 : 1 | 4 | NPRU-(FPCT-1)*FPSi (i>0) NPRU-3* floor(NPRU/8) | floor(NPRU/8) |
| 5 | 9 : 5 : 5 : 5 | 4 | NPRU-(FPCT-1)*FPSi (i>0) NPRU-3* floor(NPRU*5/24) | floor(NPRU*5/24) |
| 6 | 0:1:1:0 | 2 | 0 | NPRU/2 for i=1,2 0 for i=3 |
| 7 | 1:1:1:0 | 3 | NPRU – 2*floor(NPRU/3) | floor(NPRU/3) for i=1,2 0 for i=3 |
| 6-7 | Reserved |  |  |  |

| - Mapping between DFPC and frequency partitioning for 512 FFT size5 Mhz |  |  |  |  |
| --- | --- | --- | --- | --- |
| DFPC | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| 0 | 1 : 0 : 0 : 0 | 1 | NPRU | 0 |
| 1 | 0 : 1 : 1 : 1 | 3 | 0 | NPRU * 1/3 |
| 2 | 1 : 1 : 1 : 1 | 4 | NPRU * 1/4 | NPRU * 1/4 |
| 3 | 3 : 1 : 1 : 1 | 4 | NPRU * 1/2 | NPRU * 1/6 |
| 4 | 9 : 5 : 5 : 5 | 4 | NPRU * 3/8 | NPRU * 5/24 |
| 5 | 0 : 1 : 1 : 0 | 2 | 0 | NPRU/2 for i=1,2 0 for i=3 |
| 6 | 1 : 1 : 1 : 0 | 3 | NPRU*1/3 | NPRU/3 for i=1,2 0 for i=3 |
| 5-7 | Reserved |  |  |  |

-----------------------------------------------------End of the Text Proposal ---------------------------------------------------

---------------------------------------------------- Proposed Text #2 ------------------------------------------------

Make the following changes to Table 870 through Table 874 in Section 16.3.8.2.3 on Page 323 beginning Line 12

| Table 870 - Mapping between UFPC and frequency partitioning for 20 MHz |  |  |  |  |
| --- | --- | --- | --- | --- |
| UFPC | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPSi (i=0) | FPSi (i>0) |
| 0 | 1 : 0 : 0 : 0 | 1 | NPRU | 0 |
| 1 | 0 : 1 : 1 : 1 | 3 | 0 | NPRU * 1/3 |
| 2 | 1 : 1 : 1 : 1 | 4 | NPRU * 1/4 | NPRU * 1/4 |
| 3 | 3 : 1 : 1 : 1 | 4 | NPRU * 1/2 | NPRU * 1/6 |
| 4 | 5 : 1 : 1 : 1 | 4 | NPRU * 5/8 | NPRU * 1/8 |
| 5 | 9 : 1 : 1 : 1 | 4 | NPRU * 9/12 | NPRU * 1/12 |
| 6 | 9 : 5 : 5 : 5 | 4 | NPRU * 3/8 | NPRU * 5/24 |
| 7 | 0:1:1:0 | 2 | 0 | NPRU * 1/2 for i=1,2 0 for i=3 |
| 8 | 1:1:1:0 | 3 | NPRU * 1/3 | NPRU * 1/3 for i=1,2 0 for i=3 |
| 9-15 | Reserved |  |  |  |

| Table 871 - Mapping between UFPC and frequency partitioning for 2048 FFT size20 MHz>BW>10 MHz |  |  |  |  |
| --- | --- | --- | --- | --- |
| UFPC | Freq. Partitioning (Appro.) (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| 0 | 1 : 0 : 0 : 0 | 1 | NPRU | 0 |
| 1 | 0 : 1 : 1 : 1 | 3 | 0 | FPS1 =NPRU – 2*floor(NPRU/3) FPS2 = floor(NPRU/3) FPS3 = floor(NPRU/3) |
| 2 | 1 : 1 : 1 : 1 | 4 | NPRU-(FPCT-1)*FPSi (i>0) NPRU-3* floor(NPRU/4) | floor(NPRU/4) |
| 3 | 3 : 1 : 1 : 1 | 4 | NPRU-(FPCT-1)*FPSi (i>0) NPRU-3* floor(NPRU/6) | floor(NPRU/6) |
| 4 | 5 : 1 : 1 : 1 | 4 | NPRU-(FPCT-1)*FPSi (i>0) NPRU-3* floor(NPRU/8) | floor(NPRU/8) |
| 5 | 9 : 1 : 1 : 1 | 4 | NPRU-(FPCT-1)*FPSi (i>0) NPRU-3* floor(NPRU/12) | floor(NPRU/12) |
| 6 | 9 : 5 : 5 : 5 | 4 | NPRU-(FPCT-1)*FPSi (i>0) NPRU-3* floor(NPRU*5/24) | floor(NPRU*5/24) |
| 7 | 0:1:1:0 | 2 | 0 | NPRU/2 for i=1,2 0 for i=3 |
| 8 | 1:1:1:0 | 3 | NPRU – 2*floor(NPRU/3) | floor(NPRU/3) for i=1,2 0 for i=3 |
| 79-15 | Reserved |  |  |  |

| Table 872 - Mapping between UFPC and frequency partitioning for 10 MHz |  |  |  |  |
| --- | --- | --- | --- | --- |
| UFPC | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| 0 | 1 : 0 : 0 : 0 | 1 | NPRU | 0 |
| 1 | 0 : 1 : 1 : 1 | 3 | 0 | NPRU * 1/3 |
| 2 | 1 : 1 : 1 : 1 | 4 | NPRU * 1/4 | NPRU * 1/4 |
| 3 | 3 : 1 : 1 : 1 | 4 | NPRU * 1/2 | NPRU * 1/6 |
| 4 | 5 : 1 : 1 : 1 | 4 | NPRU * 5/8 | NPRU * 1/8 |
| 5 | 9 : 5 : 5: 5 | 4 | NPRU * 3/8 | NPRU * 5/24 |
| 6 | 0:1:1:0 | 2 | 0 | NPRU * 1/2 for i=1,2 0 for i=3 |
| 7 | 1:1:1:0 | 3 | NPRU * 1/3 | NPRU * 1/3 for i=1,2 0 for i=3 |

| Table 873 - Mapping between UFPC and frequency partitioning for 1024 FFT size10 MHz>BW>5 MHz |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| UFPC | Freq. Partitioning (Appro.) (FP0:FP1:FP2:FP3) | FPCT |  | FPS0 |  | FPSi (i>0) |  |
| 0 | 1 : 0 : 0 : 0 | 1 |  | NPRU |  | 0 |  |
| 1 | 0 : 1 : 1 : 1 | 3 |  | 0 |  | FPS1 =NPRU – 2*floor(NPRU/3) FPS2 = floor(NPRU/3) FPS3 = floor(NPRU/3) |  |
| 2 | 1 : 1 : 1 : 1 | 4 |  | NPRU-(FPCT-1)*FPSi (i>0) NPRU-3* floor(NPRU/4) |  | floor(NPRU/4) |  |
| 3 | 3 : 1 : 1 : 1 | 4 |  | NPRU-(FPCT-1)*FPSi (i>0) NPRU-3* floor(NPRU/6) |  | floor(NPRU/6) |  |
| 4 | 5 : 1 : 1 : 1 | 4 |  | NPRU-(FPCT-1)*FPSi (i>0) NPRU-3* floor(NPRU/8) |  | floor(NPRU/8) |  |
| 5 | 9 : 5 : 5 : 5 | 4 |  | NPRU-(FPCT-1)*FPSi (i>0) NPRU-3* floor(NPRU*5/24) |  | floor(NPRU*5/24) |  |
| 6 | 0:1:1:0 | 2 |  | 0 |  | NPRU/2 for i=1,2 0 for i=3 |  |
| 7 | 1:1:1:0 | 3 |  | NPRU – 2*floor(NPRU/3) |  | floor(NPRU/3) for i=1,2 0 for i=3 |  |
| 6-7 | Reserved |  |  |  |  |  |  |
| Table 874 - Mapping between UFPC and frequency partitioning for 512 FFT size5 Mhz |  |  |  |  |  |  |  |
| UFPC | Freq. Partitioning (FP0:FP1:FP2:FP3) |  | FPCT |  | FPSi (i=0) FPS0 | FPSi (i>0) |  |
| 0 | 1 : 0 : 0 : 0 |  | 1 |  | NPRU | 0 |  |
| 1 | 0 : 1 : 1 : 1 |  | 3 |  | 0 | NPRU * 1/3 |  |
| 2 | 1 : 1 : 1 : 1 |  | 4 |  | NPRU * 1/4 | NPRU * 1/4 |  |
| 3 | 3 : 1 : 1 : 1 |  | 4 |  | NPRU * 1/2 | NPRU * 1/6 |  |
| 4 | 9 : 5 : 5 : 5 |  | 4 |  | NPRU * 3/8 | NPRU * 5/24 |  |
| 5 | 0 : 1 : 1 : 0 |  | 2 |  | 0 | NPRU/2 for i=1,2 0 for i=3 |  |
| 6 | 1 : 1 : 1 : 0 |  | 3 |  | NPRU*1/3 | NPRU/3 for i=1,2 0 for i=3 |  |
| 5-7 | Reserved |  |  |  |  |  |  |

-----------------------------------------------------End of the Text Proposal ---------------------------------------------------