| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | # Proposed Text for Section 16.9.3.1 E-MBS Configuration Indicators |  |
| Date Submitted | 2009-01-14 |  |
| Source(s) | Kaushik Josiam, Qi Wu, Taeyoung Kim, Seonghyeon Chae, Hai Wang, Jaejeong Shim Samsung Electronics Eunkyung Kim ETRI Chun-Yuan Chiu, Chun-Yen Wang, Chang-Lan Tsai, Yan-Xiu Zheng, and Frank Ren ITRI Kaiying Lv, Sun Bo ZTE Yongjoo Tcha KT | Email : kjosiam@sta.samsung.com qi.wu@samsung.com ty33.kim@samsung.com seonghyeon.chae@samsung.com wang.hai@samsung.com brian.shim@samsung.com ekkim@etri.re.kr |
| Re: | Contribution in support of a comment on P802.16m/D3 currently under LB#30b |  |
| Abstract | This contribution updates the text and format for E-MBS Configuration Message |  |
| Purpose | For review and adoption by IEEE 802.16m TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Text for Section 16.9.3.1 E-MBS Configuration Indicators

Kaushik Josiam, Qi Wu, Taeyoung Kim, Seonghyeon Chae, Hai Wang, Jaejeong Shim

Samsung Electronics

Eunkyung Kim

ETRI

Chun-Yuan Chiu, Chun-Yen Wang, Chang-Lan Tsai, Yan-Xiu Zheng, and Frank Ren

ITRI

Kaiying Lv, Sun Bo

ZTE

Yongjoo Tcha

KT

---------------------------------------------------- Proposed Text #1 ------------------------------------------------

Make the following changes to Section 16.9.3.1 in Page 643 beginning Line 36 in P802.16m/D3.

16.9.3.1. E-MBS Configuration Indicators

The E-MBS configuration information necessary for E-MBS operation include E-MBS-Zone_IDs of serving and neighboring ABSs, E-MBS MAP time offset (frequency offset), E-MBS MAP resource allocation, E-MBS MAP ISizeOffset, E-MBS MAP MIMO Mode and MSTID and FID mappings between serving E-MBS Zone and neighboring E-MBS Zone for the same content. AAI_E-MBS-CFG shall be advertised at a predetermined frame offset in a superframe in the superframe before the beginning of the largest MSI (16 superframes).

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI-_E-MBS-CFG_Message_Format() { |  |  |
| Num_E-MBS_Zones | 3 | Number of E-MBS Zones with which the ABS is associated. |
| MAC Control Message Type | 8 | AAI_E-MBS-CFG |
| E-MBS_CFG_LIFETIME (m) | 4 | Indicates the duration of E-MBS_CFG_LIFETIME for which the E-MBS configuration information of the zone do not change. Duration of E-MBS_CFG_LIFETIME: 16(m+1) superframes |
| E-MBS_SUBFRAME_INDICATOR | 3 | Number of AAI subframes reserved for E-MBS in the DL zone within a frame 0b000: Last 1 subframe of the DL zone 0b001: Last 2 subframe s of the DL zone 0b010: Last 3 subframe s of the DL zone 0b011: Last 4 subframe s of the DL zone 0b100: Last 5 subframes of the DL zone 0b101: Last 6 subframes of the DL zone 0b110: Last 7 subframes of the DL zone 0b111:Last 8 subframes of the DL zone |
| Zone_Allocation Bit-MAP | variable | Indicates the zone configuration. The size is as below. 20 bits for 20 MHz 9 bits for 10MHz 3 bits for 5 MHz |
| ZF | 1 | Zone Flag Bit. Indicates the use of the last zone. 0b0: Unicast 0b1: E-MBS |
| for (i = 0; i< Num_E-MBS_Zones; i++) { |  |  |
| E-MBS_Zone_ID | 7 | The EMBS_Zone_ID to which this EMBS MAP applies. |
| MSI Length (NMSI) | 2 | The length of an MSI in units of the number of superframes 0b00: 2 superframe, 40ms (NMSI = 2) 0b01: 4 superframe, 80ms (NMSI = 4) 0b10: 8 superframe, 160ms (NMSI = 8) 0b11: 16 superframe, 320ms (NMSI = 16) |
| E-MBS MAP Resource Index | [TBD] 11 | Resource index includes location and allocation size. |
| E-MBS MAP ISizeOffset | 5 |  |
| MSTID and FID Mappings List inclusion | 1 | Indicates whether MSTID and FID Mapping list is included or not 0b0: No Zone boundary or Zone boundary but no inter-zone service continuity in neighbor ABS 0b1: Zone boundary and inter-zone service continuity |
| if (MSTID and FID Mappings List inclusion ==1) { |  |  |
| MSTID and FID Mappings List | Variable | MSTID and FID Mappings List between serving and neighbor E-MBS Zones |
| } |  |  |
| } |  |  |
| } |  |  |

Zone_Allocation Bit-MAP: Zone_Allocation Bit-MAP consists of sub-band indices reserved for all E-MBS zones the BS belongs to. The Zone Allocation Bit-MAP in the AAI-E-MBS-CFG message identifies the use of the resource comprising a set of contiguous subbands in a DL AAI subframe. The Bit-MAP determines the size (S) of each E-MBS zone in number of contiguous SLRUs in frequency domain within the subframe, the index (L) of each E-MBS zone from where the allocated zone begins and the total number (Num_E-MBS_Zones) of allocated zones.

The Zone Allocation Bit-MAP is constructed using the following rules:

- If the use of the resource is changed between one sub-band and the next sub-band, the bit ‘1’ is set in the location the same as the index of the sub-band in the Bit-MAP.

- Otherwise, the bit ‘0’ is set in the Bit-MAP.

For a given system bandwidth with maximum number of subbands, DSACmax = 21, 10, 4 for 20, 10 and 5 MHz respectively, the size of the Zone Allocation Bit-MAP is (DSACmax -1). Let the total number of the bit ‘1’ in the Bit-MAP be denoted as K and the index in which the bit ‘1’ in the Bit-MAP is located be denoted as Ji, where i is the E-MBS zone index which is from 1 to Num_E-MBS_Zones. J0 is fixed to 0.

Num_E-MBS_Zones  =  K + 1	if  ZF=0b1							(xx1)

K	if  ZF=0b0

Li  =  Ji-1 +1											(xx2)

Si  =  Ji – Ji-1			1 ≤ i ≤ K								(xx3)

DSACmax – JK		    i = K+1

For example, consider the case where the Zone Allocation Bit-MAP field = 000010010. The system bandwidth is 10MHz and DSACmax is 10. This is illustrated pictorially below in Figure xxx. In this case, ZF=0b0 because the last zone is the unicast zone. Num_E-MBS_Zones = 2, Li = {1, 6}, Si = {5, 3} from K = 2, Ji = {0, 5, 8}. Hence, the number of zones is 2. Each index of two zones is 1, 6 respectively and each size of two zones is 5, 2 respectively.

Figure xxx – Zone Allocation Bit-MAP for 10MHz

E-MBS MAP Resource Index: Resource Index indicating the starting SLRUE-MBS index and size of a single allocation spanning contiguous SLRUE-MBSs of the E-MBS MAP.

For each neighbor E-MBS_Zone, the MSTID and FID Mappings List between serving and neighbor E-MBS Zones shall be included if the serving and neighbor ABSs belong to different E-MBS Zones which provide service continuity.

The mappings list includes the followings:

Num_Neighbor_E-MBS_Zones (3 bits)

AAI_NBR-ADV Change Count (3 bits)

for (i = 0; i< Num_Neighbor_E-MBS_Zones; i++) {

Neighbor_E-MBS_Zone_ID (7 bits)

Physical Carrier Index (6 bits)

Number_Neighbor_ABS (8 bits)

for (j = 0; j< Number_Neighbor_ABS; j++) {

Index of BS (8 bits)

}

Num_MSTID_FID Mappings (4 bits)

for (k = 0; k< Num_MSTID_FID Mappings; k++) {

Current_MSTID and FID (k),

New_MSTID and FID (k),

}

}

If a Current_MSTID and FID is not available in the E-MBS Zone of the neighbor ABS, the mapping for the Current_MSTID and FID shall not be included in the mapping list.

-----------------------------------------------------End of the Text Proposal ---------------------------------------------------