| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for section 16.3.5.6 Downlink physical structure for E-MBS support |  |
| Date Submitted | 2009-12-30 |  |
| Source(s) | Kaushik Josiam, Taeyoung Kim, Seonghyeon Chae, Hai Wang, Qi Wu, Jaejeong Shim Samsung Electronics | Email : kjosiam@sta.samsung.com qi.wu@samsung.com ty33.kim@samsung.com seonghyeon.chae@samsung.com wang.hai@samsung.com brian.shim@samsung.com |
| Re: | Contribution in support of a comment on P802.16m/D3 currently under LB#30b |  |
| Abstract | This contribution defines the text for downlink physical structure for E-MBS support |  |
| Purpose | For review and adoption by IEEE 802.16m TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text for section 16.3.5.6 Downlink physical structure for E-MBS support

Kaushik Josiam, Taeyoung Kim, Seonghyeon Chae, Hai Wang, Qi Wu, Jaejeong Shim

Samsung Electronics

---------------------------------------------------- Proposed Text  ----------------------------------------------------------

- Downlink physical structure for E-MBS support

In a carrier that is not dedicated to E-MBS traffic, it may be required to multiplex E-MBS and unicast traffic. When multiplexed with unicast data, the E-MBS traffic is FDMed with the unicast traffic in the downlink subframes. More specifically, the E-MBS traffic is transmitted using subbands assigned in frequency partition FP0. The number of subbands logical resource unit (SLRU) used for E-MBS data, denoted by KSB,E-MBS, SLRUE-MBS, is indicated in the E-MBS_SUBBAND_INDICATOR Zone Allocation BitMAP in AAI-E-MBS_CFG message. This set of sub-bands form the E-MBS region in the downlink subframes. The E-MBS_SUBFRAME_INDICATOR indicates which sub-frames in a frame carry E-MBS traffic of all E-MBS Zones on that carrier. The E-MBS_SUBBAND_INDICATOR Zone Allocation BitMAP and E-MBS_SUBFRAME_INDICATOR are valid for the set of super-frames over which the AAI-E-MBS_CFG indicators are valid. For MBSFN support, KSB,E-MBS subbands are assigned from the lowest SLRU in frequency partition FP0. If the KSB,E-MBS is larger than the number of subbands in FP0, i.e., KSB,E-MBS > KSB,FP0, then, the remaining (KSB,E-MBS - KSB,FP0) subbands are assigned equitably to the three partitions FPi (i > 0) one by one starting from the lowest SLRU of the each FP.  When FPCT = 3, KSB,E-MBS subbands are assigned equitably from FPi (i > 0) one by one starting from the lowest SLRU of the lowest FP.

The mapping between SLRUE-MBS index and the PRU index shall be done as follows:

If the 0-th frequency partition, FP0 is available to use, the sub-bands for E-MBS shall be assigned only from the sub-bands in FP0. In case that FP0 is available to use, the SLRUE-MBSs are indexed as

SLRUE-MBS[k] = SLRUFP0[k], for 0  k < N1·KSB,E-MBS

In case that FP0 is not available to use (e.g., reuse-3 only), the number of PRUs for E-MBS in each frequency partition, FPi (i>0) is derived as

LEMBS,FPm=N1⋅KSB,E−MBSFPCTm=1,...,FPCT−1N1⋅KSB,E−MBS−FPCT−1⋅KSB,E−MBSFPCTm=FPCT

The total number of subbands for E-MBS over all partitions may be calculated as

YE−MBS=m=1FPCTLEMBS,FPmN1

For each frequency partition i (i > 0), the total number of PRUs for E-MBS up-to and including that partition may be calculated as

Xi=m=1iLEMBS,FPm, 1≤i≤FPCT

Then, the SLRUE-MBSs are indexed as

SLRUE-MBS[k] = SLRUFPi[k], for Xi-1  k < Xi with 1  i < FPCT, with 0  k < N1·YE-MBS and X0=0.

The subbands are indexed as

SBE-MBS[m]=All SLRUE-MBS[k] with indices k such that kN1=m,  with 0≤m<YE−MBS

The mapping from the SLRUFPi to the PRU indices is as specified in 16.3.5.3.5.

-----------------------------------------------------End of the Text Proposal ---------------------------------------------------