| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Section 16.9.3.3 E-MBS Scheduling Interval (MSI) |  |
| Date Submitted | 2010-01-30 |  |
| Source(s) | Kaushik Josiam, Taeyoung Kim, Seonghyeon Chae, Hai Wang, Qi Wu, Jaejeong Shim Samsung Electronics | Email : kjosiam@sta.samsung.com qi.wu@samsung.com ty33.kim@samsung.com seonghyeon.chae@samsung.com wang.hai@samsung.com brian.shim@samsung.com |
| Re: | Contribution in support of a comment on P802.16m/D3 currently under LB#30b |  |
| Abstract | This contribution defines the text for E-MBS Scheduling Interval (MSI) |  |
| Purpose | For review and adoption by IEEE 802.16m TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Section 16.9.3.3 E-MBS Scheduling Interval (MSI)

Kaushik Josiam, Taeyoung Kim, Seonghyeon Chae, Hai Wang, Qi Wu, Jaejeong Shim

Samsung Electronics

---------------------------------------------------- Proposed Text  ----------------------------------------------------------

Make the following changes in section 16.9.3.3 beginning at line 51, Page 644 in IEEE P802.16m/D3

      - E-MBS Scheduling Interval (MSI)

For each E-MBS Zone there is an E-MBS Scheduling Interval (MSI), which refers to a number of successive super-frames for which the access network may schedule traffic for the streams associated with the MBS Zone prior to the start of the interval. The MSI can span several super-frames and the length of this interval, denoted by NMSI, depends on the particular use case of E-MBS. The MSI can be, NMSI = 2, 4, 8 and 16 superframes long.  The E-MBS_MAP message addresses the mapping of E-MBS data associated with an E-MBS Zone for an entire MSI.  The MBS MAP message is structured such that it may be used to efficiently define multiple transmission instances for a given stream within an MSI. The MSI for a particular E-MBS zone is transmitted in the AAI-E-MBS-CFG message. Using the superframe number, Nsuperframe from SFH, and NMSI from AAI-E-MBS-CFG message, the AMS computes the beginning of the MSI as follows:

The MSI begins at the superframe when its Nsuperframe meets the following condition.

Nsuperframe  modulo NMSI  == 0

An AMS decodes only the E-MBS data bursts associated with user selected content.  The AMS wakes up in each MSI in order to check whether there are E-MBS data bursts to be decoded.

-----------------------------------------------------End of the Text Proposal ---------------------------------------------------