| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal for Single BS precoding with Multi-BS Coordination on Section 16.5.1.2.1 |  |
| Date Submitted | 2009-12-30 |  |
| Source(s) | Jiann-An Tsai, Jerry Pi, David Mazzarese Samsung Electronics Co., Ltd. | jtsai@sta.samsung.com |
| Re: | Contribution in support of a comment on LB30B Section 16.5.1.2.1 Single BS precoding with Multi-BS Coordination |  |
| Abstract | The contribution proposes the text of transformation codebook to be included in the 802.16m AWD (P802.16m/D3). |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m AWD (P802.16m/D3). |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AWD Text Proposal for Single BS precoding with Multi-BS Coordination on Section 16.5.1.2.1

Jiann-An Tsai, Jerry Pi, David Mazzarese

Samsung Electronics Co., Ltd.

- Introduction

The contribution proposes the text of Single BS precoding with Multi-BS Coordination to be included in the 802.16m AWD ([4]). The proposed text is developed so that it can be readily combined with IEEE P802.16 Rev2/D8 ([1]), it is compliant to the 802.16m SRD ([2])).

- References

[1] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Dec. 2008.

[2] IEEE 802.16m-07/002r7, “IEEE 802.16m System Requirements Document”

[3] IEEE 802.16m-08/003r8, “IEEE 802.16m System Description Document”

[4] IEEE 802.16m/D3, “802.16m Amendment Working Document (AWD)”

- Text proposal for inclusion in the 802.16m AWD/D3

-------------------------------  Text Start  ---------------------------------------------------------------

Page 608, line 42, section 16.5.1.2.1

With codebook-based feedback, PMI coordination can be applied as either PMI recommendation or PMI restriction.

If ICT (interference coordination type) is set to 0b00 in Feedback Polling A-MAP IE, then the AMS finds the PMI which acts as the strongest interference for the neighboring cell in the frequency resource unit indicated by TRU (target resource unit) indicated in Feedback Polling A-MAP IE as the reference PMI.

If ICT (interference coordination type) is set to 0b01 in Feedback Polling A-MAP IE, then the AMS finds the PMI which acts as the weakest interference for the neighboring cell in the frequency resource unit indicated by TRU (target resource unit) indicated in Feedback Polling A-MAP IE as the reference PMI.

Page 608, line 62, section 16.5.1.2.1

- Once an AMS receives a Feedback Polling A-MAP IE, it shall send AAI_MultiBS_MIMO_FBK periodically with the requested information. This information may include a PMI (the reference PMI) or a set of PMIs, Temp_BSID (diversity member ID) and additional measurements.

For reporting a set of PMIs, the following procedure shall be performed at the AMS after determining the reference  PMI wk:

Page 609, line 21-26, section 16.5.1.2.1

The AMS determines the size of the subset of PMIs to be jointly recommended (or restricted), based on two fixed correlation levels determined by n1 and n2 in Equation (287). The AMS indicates the selection of n1 or n2 via PMI_coordination_subset in a feedback header. The selection of n1 or n2 identify the PMI subset to be either recommended or restricted. The value of PMI_coordination_subset is specified in Table 937.

-------------------------------  Text End  -----------------------------------------------------------------