| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to Advanced Preamble design for supporting tone dropping (15.3.6.1) |  |
| Date Submitted | 2010-01-11 |  |
| Source(s) | Sung-Eun Park, Chiwoo Lim, Jaeweon Cho, Jaehee Cho, Heewon Kang, Hokyu Choi Samsung Electronics Co., Ltd. Yang-Seok Choi Intel Corporation Jin Sam Kwak LG Electronics | se.park@samsung.com |
| Re: | Change Request on 802.16m Draft Amendment, IEEE P802.16m/D3 Section: 15.3.6.1 Advanced Preamble |  |
| Abstract | The contribution proposes the text changes to Advanced Preamble design of the 802.16m draft amendment, IEEE P802.16m/D3. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to Advanced Preamble design 
for supporting tone dropping (15.3.6.1)

Sung-Eun Park, Chiwoo Lim, Jaeweon Cho, Jaehee Cho, Heewon Kang, Hokyu Choi
Samsung Electronics Co., Ltd.

Yang-Seok Choi

Intel Corporation

Jin Sam Kwak

LG Electronics

- Introduction

The contribution proposes the text of Advanced Preamble with tone dropping support to be included in the 802.16m amendment. The Advanced Preamble in the current draft amendment [1] only has symbol structure to support tone dropping.  In this contribution, antenna configuration, block cover sequences and corresponding boosting level will be provided.

- Proposal for Advanced Preamble for supporting tone dropping

Assumption

1) Irregular nominal channel bandwidth in 5~20 MHz uses the same subcarrier spacing of 10MHz or 20MHz.

2) Irregular nominal channel bandwidth in 5~10 MHz uses 1024-FFT, and Irregular nominal channel bandwidth in 10~20MHz uses 2048-FFT.

3) SA-Preamble occupies the available bandwidth as large as possible, while maintaining the scalable structure.

4) The loading amount on each transmit antenna should be balanced as uniformly as possible in SA-Preamble

Advanced Preamble design concept

For regular nominal channel bandwidth, the SA-Preamble has a scalable structure as shown in Figure 1. That is, the 5MHz sequence is reused for 10 and 20MHz bandwidth by repetition of 8 sub-blocks {A, B, C, D, E, F, G, and H}. Each sub-block consists of 18 QPSK symbols deployed over 54 sub-carriers.

Figure 1─SA-Preamble structure

For irregular nominal channel bandwidth, the SA-Preamble is obtained by dropping the farthest sub-blocks of the regular bandwidth from DC subcarrier on both sides.

For support of irregular nominal channel bandwidth by tone dropping, the dropping unit of SA-Preamble is two sub-blocks. The granularity of nominal channel bandwidth is 1.25MHz. Table 1 shows the allocation of sequence sub-blocks for SA-Preamble according to the different range of available nominal channel bandwidth where Nsi is the total number of sub-blocks in irregular nominal channel bandwidth.

Table 1─Allocation of SA-Preamble sequence sub-blocks for tone dropping support

| Irregular Nominal Channel Bandwidth Range, x (MHz) | Nsi | Allocation of sequence sub-blocks |
| --- | --- | --- |
| 5 < x < 6.25 | 8 | ABCD EFGH |
| 6.25 ≤ x < 7.5 | 10 | H ABCD EFGH A |
| 7.5 ≤ x < 8.75 | 12 | GH ABCD EFGH AB |
| 8.75 ≤ x < 10 | 14 | FGH ABCD EFGH ABC |
| 10 < x < 11.25 | 16 | EFGH ABCD EFGH ABCD |
| 11.25 ≤ x < 12.5 | 18 | D EFGH ABCD EFGH ABCD E |
| 12.5 ≤ x < 13.75 | 20 | CD EFGH ABCD EFGH ABCD EF |
| 13.75 ≤ x < 15 | 22 | BCD EFGH ABCD EFGH ABCD EFG |
| 15 ≤ x < 16.25 | 24 | ABCD EFGH ABCD EFGH ABCD EFGH |
| 16.25 ≤ x < 17.5 | 26 | H ABCD EFGH ABCD EFGH ABCD EFGH A |
| 17.5 ≤ x < 18.75 | 28 | GH ABCD EFGH ABCD EFGH ABCD EFGH AB |
| 18.75 ≤ x < 20 | 30 | FGH ABCD EFGH ABCD EFGH ABCD EFGH ABC |

When the antenna configuration of the reference channel bandwidth is applied to the irregular channel bandwidth, the loading on each transmit antenna is not balanced. Figure 2 shows the antenna configuration of 4Tx, 11.25~12.5MHz bandwidth, for instance. The different loading on the transmit antennas cause different peak power per antennas. So it is required that the loading on antennas should be balanced to achieve maximum power boosting gain.

Figure 2─Allocation of SA-Preamble on 4Tx antennas at 11.25 ≤ x < 12.5

In case of 5 < x < 6.25, the antenna configuration of 512-FFT size for the regular channel bandwidth 5MHz is used. In case of 10 < x < 11.25, the antenna configuration of 1024-FFT size for the regular channel bandwidth 10MHz is used. Except for these two cases, the algorithm to assign the SA-Preamble sub-blocks to multiple transmit antennas in the irregular system bandwidth can be described as follows. Let Nst,k be the number of sub-blocks for kth antenna, where k=0, 1, …, Nt -1. Then Nst,k is defined by Equation (xx) and Equation (yy).

,								(xx)

where

.										(yy)

Table 2 shows the value of Nst,k for each Nsi and Nt.

Table 2─Number of sub-blocks for multiple transmit antenna in irregular system bandwidth

| Nsi | Nt | Nst,0 | Nst,1 | Nst,2 | Nst,3 | Nst,4 | Nst,5 | Nst,6 | Nst,7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 1 | 8 |  |  |  |  |  |  |  |
|  | 2 | 4 | 4 |  |  |  |  |  |  |
|  | 4 | 2 | 2 | 2 | 2 |  |  |  |  |
|  | 8 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 10 | 1 | 10 |  |  |  |  |  |  |  |
|  | 2 | 5 | 5 |  |  |  |  |  |  |
|  | 4 | 2 | 3 | 3 | 2 |  |  |  |  |
|  | 8 | 1 | 1 | 1 | 2 | 2 | 1 | 1 | 1 |
| 12 | 1 | 12 |  |  |  |  |  |  |  |
|  | 2 | 6 | 6 |  |  |  |  |  |  |
|  | 4 | 3 | 3 | 3 | 3 |  |  |  |  |
|  | 8 | 1 | 1 | 2 | 2 | 2 | 2 | 1 | 1 |
| 14 | 1 | 14 |  |  |  |  |  |  |  |
|  | 2 | 7 | 7 |  |  |  |  |  |  |
|  | 4 | 3 | 4 | 4 | 3 |  |  |  |  |
|  | 8 | 1 | 2 | 2 | 2 | 2 | 2 | 2 | 1 |
| 16 | 1 | 16 |  |  |  |  |  |  |  |
|  | 2 | 8 | 8 |  |  |  |  |  |  |
|  | 4 | 4 | 4 | 4 | 4 |  |  |  |  |
|  | 8 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 18 | 1 | 18 |  |  |  |  |  |  |  |
|  | 2 | 9 | 9 |  |  |  |  |  |  |
|  | 4 | 4 | 5 | 5 | 4 |  |  |  |  |
|  | 8 | 2 | 2 | 2 | 3 | 3 | 2 | 2 | 2 |
| 20 | 1 | 20 |  |  |  |  |  |  |  |
|  | 2 | 10 | 10 |  |  |  |  |  |  |
|  | 4 | 5 | 5 | 5 | 5 |  |  |  |  |
|  | 8 | 2 | 2 | 3 | 3 | 3 | 3 | 2 | 2 |
| 22 | 1 | 22 |  |  |  |  |  |  |  |
|  | 2 | 11 | 11 |  |  |  |  |  |  |
|  | 4 | 5 | 6 | 6 | 5 |  |  |  |  |
|  | 8 | 2 | 3 | 3 | 3 | 3 | 3 | 3 | 2 |
| 24 | 1 | 24 |  |  |  |  |  |  |  |
|  | 2 | 12 | 12 |  |  |  |  |  |  |
|  | 4 | 6 | 6 | 6 | 6 |  |  |  |  |
|  | 8 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| 6 | 1 | 26 |  |  |  |  |  |  |  |
|  | 2 | 13 | 13 |  |  |  |  |  |  |
|  | 4 | 6 | 7 | 7 | 6 |  |  |  |  |
|  | 8 | 3 | 3 | 3 | 4 | 4 | 3 | 3 | 3 |
| 28 | 1 | 28 |  |  |  |  |  |  |  |
|  | 2 | 14 | 14 |  |  |  |  |  |  |
|  | 4 | 7 | 7 | 7 | 7 |  |  |  |  |
|  | 8 | 3 | 3 | 4 | 4 | 4 | 4 | 3 | 3 |
| 30 | 1 | 30 |  |  |  |  |  |  |  |
|  | 2 | 15 | 15 |  |  |  |  |  |  |
|  | 4 | 7 | 8 | 8 | 7 |  |  |  |  |
|  | 8 | 3 | 4 | 4 | 4 | 4 | 4 | 4 | 3 |

The sub-block position for the kth antenna is , where t = 0, 1, …, , and .

Figure 3 shows the proposed SA-Preamble allocation at 11.25 ≤ x < 12.5 as an example.

Figure 3─Proposed allocation of SA-Preamble at 11.25 ≤ x < 12.5

In addition, the block cover sequence (BCS) is introduced to reduce PAPR of SA-Preamble. The BCS is optimized for arbitrary number of transmit antennas and any bandwidths. Table 2 shows proposed BCSs base on the sub-block dropping design. In Table 2, BWmin denotes the minimum value (MHz) of Irregular Nominal Channel Bandwidth Range as shown in Table 1. BCS for the range 5 < x < 6.25 is same as BCS of 512-FFT size for the regular channel bandwidth 5MHz, and BCS for the range 10 < x < 11.25 is same as BCS of 1024-FFT size for the regular channel bandwidth 10MHz as shown in Table 792.

Table 2—SA-Preamble block cover sequence based on sub-block dropping

| (BWmin, number of antennas)\Segment ID | 0 | 1 | 2 |
| --- | --- | --- | --- |
| (6.25,1) | 0AA0 | 0FF8 | 0008 |
| (6.25,2) | 0F68 | 0650 | 0458 |
| (6.25,4) | 0300 | 0300 | 0908 |
| (6.25,8) | 0140 | 0000 | 0100 |
| (7.5,1) | 000C | 1008 | 0990 |
| (7.5,2) | 0C24 | 1558 | 0F58 |
| (7.5,4) | 1B08 | 030C | 1904 |
| (7.5,8) | 0140 | 0400 | 0510 |
| (8.75,1) | 0C10 | 229A | 2554 |
| (8.75,2) | 335A | 146C | 3C10 |
| (8.75,4) | 1320 | 0252 | 2406 |
| (8.75,8) | 0140 | 0404 | 1514 |
| (11.25, 1) | F7FDCEF3 | 29D51936 | 6B59CC03 |
| (11.25, 2) | 005A0F80 | 00000000 | 00000000 |
| (11.25, 4) | 00AF6A80 | 00E65280 | 00A45900 |
| (11.25, 8) | 00230000 | 00A30200 | 00090800 |
| (12.5, 1) | AC1AD967 | 602F7D20 | 96771160 |
| (12.5, 2) | 000F7000 | 01FF0000 | 00000040 |
| (12.5, 4) | 00AF6A80 | 00E65040 | 00C45900 |
| (12.5, 8) | 015B0940 | 00030C40 | 01190540 |
| (13.75, 1) | 4F56FD74 | C0F7EEAD | 55459EDB |
| (13.75, 2) | 012C1BA0 | 030177A0 | 02266200 |
| (13.75, 4) | 024C2400 | 03155920 | 01CF59A0 |
| (13.75, 8) | 009B0800 | 02830D20 | 02190420 |
| (15, 1) | DAD99B4F | 96771160 | CCF40660 |
| (15, 2) | 00CC5AB0 | 015411D0 | 020E0050 |
| (15, 4) | 038C2470 | 02D559C0 | 048F58D0 |
| (15, 8) | 009B0820 | 02830D10 | 06190420 |
| (16.25, 1) | 492F4A63 | 3785F431 | AFCB95FD |
| (16.25, 2) | 065A5AD0 | 06543808 | 080F4C50 |
| (16.25, 4) | 0D335A70 | 01146C70 | 053C10D8 |
| (16.25, 8) | 09932018 | 0D025248 | 00240640 |
| (17.5, 1) | DD37B03D | 81418C78 | A47BAAA9 |
| (17.5, 2) | 10C36614 | 1B553644 | 0F9A5524 |
| (17.5, 4) | 19B35A4C | 13146C44 | 03BC1030 |
| (17.5, 8) | 1223234C | 09525248 | 1B04042C |
| (18.75, 1) | 56849127 | 7EBE7387 | 4B958D3A |
| (18.75, 2) | 2BA55AD4 | 029C6D58 | 13590072 |
| (18.75, 4) | 3D73734C | 0F30306C | 2B00000E |
| (18.75, 8) | 05232312 | 21525272 | 04040414 |

- References

[1] IEEE P802.16m/D3, “Draft Amendment to IEEE Standard for Local and Metropolitan Area Networks- Part 16: Air Interface for Broadband Wireless Access Systems,” December 2009.

- Proposed changes for 802.16m draft amendment

---------------------------------------------------  Text Start  ---------------------------------------------------

[Remedy: Insert the following text under the Table 793 in page356]

In case of 5 < x < 6.25, the antenna configuration of 512-FFT size for the regular channel bandwidth 5MHz is used. In case of 10 < x < 11.25, the antenna configuration of 1024-FFT size for the regular channel bandwidth 10MHz is used. Except for these two cases, the algorithm to assign the SA-Preamble sub-blocks to multiple transmit antennas in the irregular system bandwidth is described as follows. Let Nst,k be the number of sub-blocks for kth antenna, where k=0, 1, …, Nt -1. Then Nst,k is defined by Equation (xx) and Equation (yy).

,						(xx)

where

.								(yy)

The sub-block position for the kth antenna is , where t = 0, 1, …, , and .

Figure zzz shows the proposed SA-Preamble allocation at 11.25 ≤ x < 12.5 as an example.

Figure zzz─Proposed allocation of SA-Preamble at 11.25 ≤ x < 12.5

At each time frame containing SA-Preamble, the transmit structure are rotated across the transmit antennas. The rotating direction is same as the reference channel bandwidth.

The block cover sequence shall be applied to each sub-block. Each bit {0, 1} of block cover sequence shown in Table yyy as a hexadecimal format is mapped to real number {+1, -1}, and then multiplied to all the subcarriers in the sub-block structure corresponding to the tone dropping design. In Table yyy, BWmin denotes the minimum value (MHz) of Irregular Nominal Channel Bandwidth Range as shown in Table 793. BCS for the range 5 < x < 6.25 is same as BCS of 512-FFT size for the regular channel bandwidth 5MHz, and BCS for the range 10 < x < 11.25 is same as BCS of 1024-FFT size for the regular channel bandwidth 10MHz as shown in Table 792.

Table yyy—SA-Preamble block cover sequence based on sub-block dropping

| (BWmin, number of antennas)\Segment ID | 0 | 1 | 2 |
| --- | --- | --- | --- |
| (6.25,1) | 0AA0 | 0FF8 | 0008 |
| (6.25,2) | 0F68 | 0650 | 0458 |
| (6.25,4) | 0300 | 0300 | 0908 |
| (6.25,8) | 0140 | 0000 | 0100 |
| (7.5,1) | 000C | 1008 | 0990 |
| (7.5,2) | 0C24 | 1558 | 0F58 |
| (7.5,4) | 1B08 | 030C | 1904 |
| (7.5,8) | 0140 | 0400 | 0510 |
| (8.75,1) | 0C10 | 229A | 2554 |
| (8.75,2) | 335A | 146C | 3C10 |
| (8.75,4) | 1320 | 0252 | 2406 |
| (8.75,8) | 0140 | 0404 | 1514 |
| (11.25, 1) | F7FDCEF3 | 29D51936 | 6B59CC03 |
| (11.25, 2) | 005A0F80 | 00000000 | 00000000 |
| (11.25, 4) | 00AF6A80 | 00E65280 | 00A45900 |
| (11.25, 8) | 00230000 | 00A30200 | 00090800 |
| (12.5, 1) | AC1AD967 | 602F7D20 | 96771160 |
| (12.5, 2) | 000F7000 | 01FF0000 | 00000040 |
| (12.5, 4) | 00AF6A80 | 00E65040 | 00C45900 |
| (12.5, 8) | 015B0940 | 00030C40 | 01190540 |
| (13.75, 1) | 4F56FD74 | C0F7EEAD | 55459EDB |
| (13.75, 2) | 012C1BA0 | 030177A0 | 02266200 |
| (13.75, 4) | 024C2400 | 03155920 | 01CF59A0 |
| (13.75, 8) | 009B0800 | 02830D20 | 02190420 |
| (15, 1) | DAD99B4F | 96771160 | CCF40660 |
| (15, 2) | 00CC5AB0 | 015411D0 | 020E0050 |
| (15, 4) | 038C2470 | 02D559C0 | 048F58D0 |
| (15, 8) | 009B0820 | 02830D10 | 06190420 |
| (16.25, 1) | 492F4A63 | 3785F431 | AFCB95FD |
| (16.25, 2) | 065A5AD0 | 06543808 | 080F4C50 |
| (16.25, 4) | 0D335A70 | 01146C70 | 053C10D8 |
| (16.25, 8) | 09932018 | 0D025248 | 00240640 |
| (17.5, 1) | DD37B03D | 81418C78 | A47BAAA9 |
| (17.5, 2) | 10C36614 | 1B553644 | 0F9A5524 |
| (17.5, 4) | 19B35A4C | 13146C44 | 03BC1030 |
| (17.5, 8) | 1223234C | 09525248 | 1B04042C |
| (18.75, 1) | 56849127 | 7EBE7387 | 4B958D3A |
| (18.75, 2) | 2BA55AD4 | 029C6D58 | 13590072 |
| (18.75, 4) | 3D73734C | 0F30306C | 2B00000E |
| (18.75, 8) | 05232312 | 21525272 | 04040414 |

---------------------------------------------------  Text End  ---------------------------------------------------