| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Identifications of the Joint Processing ABSs in IEEE P802.16m/D3 (16.5.1.1) |  |
| Date Submitted | 2009-12-30 |  |
| Source(s) | Ping-Heng Kuo, Yu-Tao Hsieh, Pang-An Ting ITRI | Voice: +886-3-5918371 E-mail: pinghengkuo@itri.org.tw |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirc #30a P802.16m/D3 : 16.5.1.1 |  |
| Abstract | This contribution proposes the method of transferring collaborative ABS IDs to the corresponding AMS in Multi-BS operations. |  |
| Purpose | To be discussed and adopted by TGm for the IEEE P802.16m Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Identifications of the Joint Processing ABSs

in IEEE P802.16m/D3 (16.5.1.1)

Ping-Heng Kuo, Yu-Tao Hsieh, Pang-An Ting

ITRI

# Introduction

To enable multi-BS joint processing, the ABS shall inform the corresponding AMS regarding the IDs of collaborative ABSs. How the IDs are sent is still TBD in [1]. The size of a cell ID is at least 24 bits and it could be heavy burden to transfer such information, especially when the number of collaborative ABSs is large.

Based on the fact that the AMS can obtain the information of its neighboring ABSs through AAI_NBR-ADV message as described in Section 16.2.3.12 of [1], the ABS could simply transfer the identifications of collaborative ABSs using BitMap format. In other words, the ABS sends a bitmap with the size of M bits where M is the total number of ABS that has been included in AAI_NBR-ADV message (as defined in Section 16.2.3.12 of [1]). When the ith element of such bitmap is 1, it indicates that the ith neighboring ABS in AAI_NBR-ADV message will be involved in the joint processing.

# Reference

[1] IEEE P802.16m/D3, “Part 16: Air Interface for Broadband Wireless Access Systems: Advanced Air Interface,” December 2009.

- Text proposal for inclusion in the 802.16m amendment

----------------------------------------------------------------------------------------------------------------------------------------------------------------

Black text: text in the subclause 16.5.1.1 of P802.16m/D3

Red Strike through Text: Deleted

Blue text: new text

[Bracketed Italic text]: Informative

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

16.5.1.1 DL/UL Signaling

[insert the following text at the end of 16.5.1.1 (right below Table 935)]

Uplink sounding can be used to support Co-MIMO and CL-MD.

In a multi-BS joint processing mode (i.e. CL-MD mode or Co-MIMO), an ABS shall inform the corresponding AMS of the set of the ABSs participating in joint processing. The ABS shall send a bitmap with M bits, where M denotes the number of neighboring ABSs that has been indicated in AAI_NBR-ADV (as described in Section 16.2.3.12). The ordering of neighboring ABS in the bitmap shall also follow the ABS index given in AAI_NBR-ADV. If ith element of the bitmap is set to 1, then the neighboring ABS with index i is involved in joint processing. The IDs of the joint processing ABSs should be transmitted from the serving ABS to the AMS by using the following message format shown in Table 936.

Table 936 – Parameters to be transferred from serving ABSs to AMSs

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| N_Preferred_C_ABSs | 3 | N_preferred_C_ABSs denotes the number of preferred cooperating ABSs excluding the serving ABS, which is expressed as a 3-bit binary number. For example, N_preferred_C_ABSs=0b001 means that the serving ABS is going to cooperate with one neighbor ABS for the AMSs. |
| Preferred_C_ABS IDs Preferred_C_ABSs | [TBD] BitMAP: M bits | Note: IDs of the preferred cooperating ABSs The value of M is defined in Section 16.2.3.12 (Table 684) |

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------