| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Frame Configuration for Femto ABS (16.4.11) |  |
| Date Submitted | 2009-12-30 |  |
| Source(s) | Ming-Hung Tao, Ying-Chuan Hsiao, Yung-Han Chen ITRI YihShen Chen MediaTek | Voice: +886-3-5914629 E-mail: MHTao@itri.org.tw yihshen.chen@mediatek.com |
| Re: | Category: LB30b / Area: Chapter 16.4.11 (Interference Avoidance and Interference Mitigation) |  |
| Abstract | This contribution proposes changes to the Section 16.4.11 of IEEE P802.16m/D3 |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Frame Configuration for Femto ABS

Ming-Hung Tao, Ying-Chuan Hsiao, Yung-Han Chen

ITRI

YihShen Chen

MediaTek

- Introduction

In current D3, the interference between Femto and/or macro can be mitigated by static or semi-static radio resource reservation and resource sharing using FDM and/or TDM manner and/or DL power control. To achieve TDM resource reservation, it is necessary for femto ABSs to provide frame configuration information in its broadcast channel, i.e., Superframe Header (SFH). In addition, the frame configuration in current S-SFH SP2 provides legacy support function, which is not necessary for femto ABSs. Therefore, we think it is necessary to modify the current frame configuration in SP2 for the femto ABS case.

- Femto Interference Mitigation Using Resource Reservation

Each Femto ABS should be able to scan the Macro ABS to acquire system information and perform synchronization. After synchronizing with Macro ABS, the Femto ABS should also scan the neighbor Femto ABSs to acquire their FFR partition and Frame configuration information and selects a proper FFR setting and a proper frame configuration to minimize the interference. The following is an example scenario illustrating how the interference mitigation can be done by using FDM/TDM resource reservation.

fBS3

fBS1

fBS2

Macrocell

Frequency f1 used by Macro ABS to serve outdoors MSs

Frequency f2 used by fBS1, fBS2, fBS3 with different frame configurations

Mitigated Interference zone

Macro ABS

…

B

A

SFH

fBS4

FA1

f2

f3

Frequency f3, used by fBS4 to mitigate interference other fBSs

Mitigated Interference by synchronizing every DL/UL transmission

Different profiles and frequencies are broadcast by the macro BS

…

C

B

A

C

…

B

A

SFH

C

B

A

C

…

B

A

SFH

C

B

A

C

…

B

A

SFH

B

A

C

C

…

B

A

SFH

C

B

A

C

FA2

Fig.1: Illustration of interference mitigation with different frame configurations.

Both the FFR partition and Frame configuration information can be obtained from the broadcast S-SFH content. However, it should be noticed that the Frame configuration index and DFPC in S-SFH SP2 cannot refer to Tables 771-773 and 777-781 directly, because those tables are particularly designed for Marco ABS only. Regarding to the Femto ABS case, another tables which exclude WirelessMAN OFDMA support and include subframe-and-subband-based resource reservation should be defined. The detailed design for those tables is shown in the next section.

- Proposed Text for IEEE P802.16m/D3

[Accept the following modifications marked in blue]

============================ Start of Proposed Text ==================================

#### 16.4 Support for Femto ABS

#### 16.4.11 Interference Avoidance and Interference Mitigation

[Note: Please insert the following subsection under Section 16.4.11 (Insert the following subsection after Line 11, page 604)]

#### 16.4.11.1 Set of frame configurations for Femto ABS

To configure “Frame configuration index” in S-SFH SP2, the Femto ABS shall refer to Table Y, Y+1, Y+2 for the mapping between the index value and the corresponding frame configurations.

Table Y – Femto Frame Configuration and Indexing (5/10/20MHz channel bandwidth)

| - No. | - BW | - CP | - Frame configuration index | - Duplex | D:U | Subframe provision |  |  | Subframe Type |  |  |  |  |  |  |  | TTG /RTG (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | - DL - offset | - DL length | - UL - length | #0 | #1 | #2 | #3 | #4 | #5 | #6 | #7 |  |
| 1 | 5/10/20 | 1/16 | 0 | TDD | 6:2 | N/A | N/A | N/A | DL Type1 | DL Type2 | DL Type1 | DL Type1 | DL Type1 | DL Type1 | UL Type1 | UL Type2 | 82.853 / 60 |
| 2 | 5/10/20 | 1/16 | 1 | TDD | 6:2 | N/A | N/A | N/A | DL Type1 | DL Type2 | DL Type1 | Not used | Not used | - Not used | - UL Type1 | - Not used | - 82.853 / 60 |
| - 3 | - 5/10/20 | - 1/16 | - 2 | - TDD | - 6:2 | N/A | N/A | N/A | - Not used | - Not used | - Not used | - DL Type1 | - DL Type1 | - DL Type1 | - Not used | - UL Type2 | - 82.853 / 60 |
| - 4 | - 5/10/20 | - 1/16 | - 3 | - TDD | - 5:3 | N/A | N/A | N/A | - DL Type1 | - DL Type2 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type2 | - 82.853 / 60 |
| - 5 | - 5/10/20 | - 1/16 | - 4 | - TDD | - 5:3 | N/A | N/A | N/A | - DL Type1 | - DL Type2 | - Not used | - Not used | - Not used | - UL Type1 | - Not used | - Not used | - 82.853 / 60 |
| - 6 | - 5/10/20 | - 1/16 | - 5 | - TDD | - 5:3 | N/A | N/A | N/A | - Not used | - Not used | - DL Type1 | - Not used | - Not used | - Not used | - UL Type1 | - Not used | - 82.853 / 60 |
| - 7 | - 5/10/20 | - 1/16 | - 6 | - TDD | - 5:3 | N/A | N/A | N/A | - Not used | - Not used | - Not used | - DL Type1 | - DL Type1 | - Not used | - Not used | - UL Type2 | - 82.853 / 60 |
| - 8 | - 5/10/20 | - 1/16 | - 7 | - TDD | - 4:4 | N/A | N/A | N/A | - DL Type1 | - DL Type2 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type1 | - UL Type2 | - 82.853 / 60 |
| - 9 | - 5/10/20 | - 1/16 | - 8 | - TDD | - 4:4 | N/A | N/A | N/A | - DL Type1 | - Not used | - Not used | - Not used | - UL Type1 | - Not used | - Not used | - Not used | - 82.853 / 60 |
| - 10 | - 5/10/20 | - 1/16 | - 9 | - TDD | - 4:4 | N/A | N/A | N/A | - Not used | - DL Type2 | - Not used | - Not used | - Not used | - UL Type1 | - Not used | - Not used | - 82.853 / 60 |
| - 11 | - 5/10/20 | - 1/16 | - 10 | - TDD | - 4:4 | N/A | N/A | N/A | - Not used | - Not used | - DL Type1 | - Not used | - Not used | - Not used | - UL Type1 | - Not used | - 82.853 / 60 |
| - 12 | - 5/10/20 | - 1/16 | - 11 | - TDD | - 4:4 | N/A | N/A | N/A | - Not used | - Not used | - Not used | - DL Type1 | - Not used | - Not used | - Not used | - UL Type2 | - 82.853 / 60 |
| - 13 | - 5/10/20 | - 1/16 | - 12 | - TDD | - 3:5 | N/A | N/A | N/A | - DL Type1 | - DL Type2 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type1 | - UL Type1 | - UL Type2 | - 82.853 / 60 |
| - 14 | - 5/10/20 | - 1/16 | - 13 | - TDD | - 3:5 | N/A | N/A | N/A | - DL Type1 | - Not used | - Not used | - UL Type1 | - UL Type1 | - Not used | - Not used | - Not used | - 82.853 / 60 |
| - 15 | - 5/10/20 | - 1/16 | - 14 | - TDD | - 3:5 | N/A | N/A | N/A | - Not used | - DL Type2 | - Not used | - Not used | - Not used | - UL Type1 | - Not used | - Not used | - 82.853 / 60 |
| - 16 | - 5/10/20 | - 1/16 | - 15 | - TDD | - 3:5 | N/A | N/A | N/A | - Not used | - Not used | - DL Type1 | - Not used | - Not used | - Not used | - UL Type1 | - UL Type2 | - 82.853 / 60 |
| - 17 | - 5/10/20 | - 1/16 | - 16 | - FDD | - N/C | N/A | N/A | N/A | - D/U Type1 | - D/U Type2 | - D/U Type1 | - D/U Type1 | - D/U Type2 | - D/U Type1 | - D/U Type1 | - D/U Type2 | - N/A |
| - 18 | - 5/10/20 | - 1/16 | - 17 | - FDD | - N/C | N/A | N/A | N/A | - D/U Type1 | - D/U Type2 | - Not used | - Not used | - Not used | - Not used | - Not used | - Not used | - N/A |
| - 19 | - 5/10/20 | - 1/16 | - 18 | - FDD | - N/C | N/A | N/A | N/A | - Not used | - Not used | - D/U Type1 | - D/U Type1 | - Not used | - Not used | - Not used | - Not used | - N/A |
| - 20 | - 5/10/20 | - 1/16 | - 19 | - FDD | - N/C | N/A | N/A | N/A | - Not used | - Not used | - Not used | - Not used | - D/U Type2 | - D/U Type1 | - Not used | - Not used | - N/A |
| - 21 | - 5/10/20 | - 1/16 | - 20 | - FDD | - N/C | N/A | N/A | N/A | - Not used | - Not used | - Not used | - Not used | - Not used | - Not used | - D/U Type1 | - D/U Type2 | - N/A |
| - 22 | - 5/10/20 | - 1/8 | - 0 | - TDD | - 6:2 | N/A | N/A | N/A | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type3 | - UL Type1 | - UL Type1 | - 105.714 - / 60 |
| - 23 | - 5/10/20 | - 1/8 | - 1 | - TDD | - 6:2 | N/A | N/A | N/A | - DL Type1 | - DL Type1 | - DL Type1 | - Not used | - Not used | - Not used | - UL Type1 | - Not used | - 105.714 - / 60 |
| - 24 | - 5/10/20 | - 1/8 | - 2 | - TDD | - 6:2 | N/A | N/A | N/A | - Not used | - Not used | - Not used | - DL Type1 | - DL Type1 | - DL Type3 | - Not used | - UL Type1 | - 105.714 - / 60 |
| - 25 | - 5/10/20 | - 1/8 | - 3 | - TDD | - 5:3 | N/A | N/A | N/A | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type3 | - UL Type1 | - UL Type1 | - UL Type1 | - 105.714 - / 60 |
| - 26 | - 5/10/20 | - 1/8 | - 4 | - TDD | - 5:3 | N/A | N/A | N/A | - DL Type1 | - DL Type1 | - Not used | - Not used | - Not used | - UL Type1 | - Not used | - Not used | - 105.714 - / 60 |
| - 27 | - 5/10/20 | - 1/8 | - 5 | - TDD | - 5:3 | N/A | N/A | N/A | - Not used | - Not used | - DL Type1 | - Not used | - Not used | - Not used | - UL Type1 | - Not used | - 105.714 - / 60 |
| - 28 | - 5/10/20 | - 1/8 | - 6 | - TDD | - 5:3 | N/A | N/A | N/A | - Not used | - Not used | - Not used | - DL Type1 | - DL Type3 | - Not used | - Not used | - UL Type1 | - 105.714 - / 60 |
| - 29 | - 5/10/20 | - 1/8 | - 7 | - TDD | - 4:4 | N/A | N/A | N/A | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type3 | - UL Type1 | - UL Type1 | - UL Type1 | - UL Type1 | - 105.714 - / 60 |
| - 30 | - 5/10/20 | - 1/8 | - 8 | - TDD | - 4:4 | N/A | N/A | N/A | - DL Type1 | - Not used | - Not used | - Not used | - UL Type1 | - Not used | - Not used | - Not used | - 105.714 - / 60 |
| - 31 | - 5/10/20 | - 1/8 | - 9 | - TDD | - 4:4 | N/A | N/A | N/A | - Not used | - DL Type1 | - Not used | - Not used | - Not used | - UL Type1 | - Not used | - Not used | - 105.714 - / 60 |
| - 32 | - 5/10/20 | - 1/8 | - 10 | - TDD | - 4:4 | N/A | N/A | N/A | - Not used | - Not used | - DL Type1 | - Not used | - Not used | - Not used | - UL Type1 | - Not used | - 105.714 - / 60 |
| - 33 | - 5/10/20 | - 1/8 | - 11 | - TDD | - 4:4 | N/A | N/A | N/A | - Not used | - Not used | - Not used | - DL Type3 | - Not used | - Not used | - Not used | - UL Type1 | - 105.714 - / 60 |
| - 34 | - 5/10/20 | - 1/8 | - 12 | - TDD | - 3:5 | N/A | N/A | N/A | - DL Type1 | - DL Type1 | - DL Type3 | - UL Type1 | - UL Type1 | - UL Type1 | - UL Type1 | - UL Type1 | - 105.714 - / 60 |
| - 35 | - 5/10/20 | - 1/8 | - 13 | - TDD | - 3:5 | N/A | N/A | N/A | - DL Type1 | - Not used | - Not used | - UL Type1 | - UL Type1 | - Not used | - Not used | - Not used | - 105.714 - / 60 |
| - 36 | - 5/10/20 | - 1/8 | - 14 | - TDD | - 3:5 | N/A | N/A | N/A | - Not used | - DL Type1 | - Not used | - Not used | - Not used | - UL Type1 | - UL Type1 | - Not used | - 105.714 - / 60 |
| - 37 | - 5/10/20 | - 1/8 | - 15 | - TDD | - 3:5 | N/A | N/A | N/A | - Not used | - Not used | - DL Type3 | - Not used | - Not used | - Not used | - UL Type1 | - UL Type1 | - 105.714 - / 60 |
| - 38 | - 5/10/20 | - 1/8 | - 16 | - FDD | - N/C | N/A | N/A | N/A | - D/U Type1 | - D/U Type1 | - D/U Type1 | - D/U Type1 | - D/U Type1 | - D/U Type1 | - D/U Type1 | - D/U Type1 | - N/A |
| - 39 | - 5/10/20 | - 1/8 | - 17 | - FDD | - N/C | N/A | N/A | N/A | - D/U Type1 | - D/U Type1 | - Not used | - Not used | - Not used | - Not used | - Not used | - Not used | - N/A |
| - 40 | - 5/10/20 | - 1/8 | - 18 | - FDD | - N/C | N/A | N/A | N/A | - Not used | - Not used | - D/U Type1 | - D/U Type1 | - Not used | - Not used | - Not used | - Not used | - N/A |
| - 41 | - 5/10/20 | - 1/8 | - 19 | - FDD | - N/C | N/A | N/A | N/A | - Not used | - Not used | - Not used | - Not used | - D/U Type1 | - D/U Type1 | - Not used | - Not used | - N/A |
| - 42 | - 5/10/20 | - 1/8 | - 20 | - FDD | - N/C | N/A | N/A | N/A | - Not used | - Not used | - Not used | - Not used | - Not used | - Not used | - D/U Type1 | - D/U Type1 | - N/A |
| - 43 | - 5/10/20 | - 1/8 | - 21 | - TDD | - 5:3 | 3 | 2 | 3 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - Not used | - Not used | - Not used | - 105.714 - / 60 |
| - 44 | - 5/10/20 | - 1/8 | - 22 | - TDD | - 5:3 | 3 | 2 | 1 | - DL Type1 | - DL Type1 | - Not used | - Not used | - UL Type1 | - Not used | - Not used | - Not used | - 105.714 - / 60 |
| - 45 | - 5/10/20 | - 1/8 | - 23 | - TDD | - 5:3 | 2 | 3 | 3 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type1 | - Not used | - Not used | - 105.714 - / 60 |
| - 46 | - 5/10/20 | - 1/8 | - 24 | - TDD | - 5:3 | 2 | 3 | 2 | - DL Type1 | - DL Type1 | - DL Type1 | - Not used | - UL Type1 | - UL Type1 | - Not used | - Not used | - 105.714 - / 60 |
| - 47 | - 5/10/20 | - 1/8 | - 25 | - TDD | - 5:3 | 2 | 2 | 3 | - DL Type1 | - DL Type1 | - Not used | - UL Type1 | - UL Type1 | - UL Type1 | - Not used | - Not used | - 105.714 - / 60 |
| - 48 | - 5/10/20 | - 1/8 | - 26 | - TDD | - 5:3 | 2 | 2 | 1 | - DL Type1 | - DL Type1 | - Not used | - Not used | - Not used | - UL Type1 | - Not used | - Not used | - 105.714 - / 60 |
| - 49 | - 5/10/20 | - 1/8 | - 27 | - TDD | - 5:3 | 1 | 4 | 3 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type1 | - Not used | - 105.714 - / 60 |
| - 50 | - 5/10/20 | - 1/8 | - 28 | - TDD | - 5:3 | 1 | 4 | 2 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - Not used | - UL Type1 | - UL Type1 | - Not used | - 105.714 - / 60 |
| - 51 | - 5/10/20 | - 1/8 | - 29 | - TDD | - 5:3 | 1 | 3 | 3 | - DL Type1 | - DL Type1 | - DL Type1 | - Not used | - Not used | - UL Type1 | - UL Type1 | - Not used | - 105.714 - / 60 |
| - 52 | - 5/10/20 | - 1/8 | - 30 | - TDD | - 5:3 | 1 | 3 | 2 | - DL Type1 | - DL Type1 | - DL Type1 | - Not used | - UL Type1 | - UL Type1 | - UL Type1 | - Not used | - 105.714 - / 60 |
| - 53 | - 5/10/20 | - 1/8 | - 31 | - TDD | - 5:3 | 1 | 2 | 3 | - DL Type1 | - DL Type1 | - Not used | - Not used | - UL Type1 | - UL Type1 | - UL Type1 | - Not used | - 105.714 - / 60 |
| - 54 | - 5/10/20 | - 1/8 | - 32 | - TDD | - 5:3 | 1 | 2 | 1 | - DL Type1 | - DL Type1 | - Not used | - Not used | - Not used | - Not used | - UL Type1 | - Not used | - 105.714 - / 60 |
| - 55 | - 5/10/20 | - 1/8 | - 33 | - TDD | - 6:2 | N/A | N/A | N/A | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type3 | - DL Type3 | - UL Type1 | - UL Type1 | - 208.571 / 60 |
| - 56 | - 5/10/20 | - 1/8 | - 34 | - TDD | - 6:2 | N/A | N/A | N/A | - DL Type1 | - DL Type1 | - DL Type1 | - Not used | - Not used | - Not used | - UL Type1 | - Not used | - 208.571 / 60 |
| - 57 | - 5/10/20 | - 1/8 | - 35 | - TDD | - 6:2 | N/A | N/A | N/A | - Not used | - Not used | - Not used | - DL Type1 | - DL Type3 | - DL Type3 | - Not used | - UL Type1 | - 208.571 / 60 |
| - 58 | - 5/10/20 | - 1/8 | - 36 | - TDD | - 5:3 | N/A | N/A | N/A | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type3 | - DL Type3 | - UL Type1 | - UL Type1 | - UL Type1 | - 208.571 / 60 |
| - 59 | - 5/10/20 | - 1/8 | - 37 | - TDD | - 5:3 | N/A | N/A | N/A | - DL Type1 | - DL Type1 | - Not used | - Not used | - Not used | - UL Type1 | - Not used | - Not used | - 208.571 / 60 |
| - 60 | - 5/10/20 | - 1/8 | - 38 | - TDD | - 5:3 | N/A | N/A | N/A | - Not used | - Not used | - DL Type1 | - Not used | - Not used | - Not used | - UL Type1 | - Not used | - 208.571 / 60 |
| - 61 | - 5/10/20 | - 1/8 | - 39 | - TDD | - 5:3 | N/A | N/A | N/A | - Not used | - Not used | - Not used | - DL Type3 | - DL Type3 | - Not used | - Not used | - UL Type1 | - 208.571 / 60 |
| - 62 | - 5/10/20 | - 1/8 | - 40 | - TDD | - 5:3 | N/A | N/A | N/A | - DL Type1 | - DL Type1 | - DL Type3 | - DL Type3 | - DL Type3 | - UL Type1 | - UL Type1 | - UL Type1 | - 311.428 / 60 |
| - 63 | - 5/10/20 | - 1/8 | - 41 | - TDD | - 5:3 | N/A | N/A | N/A | - DL Type1 | - DL Type1 | - Not used | - Not used | - Not used | - UL Type1 | - Not used | - Not used | - 311.428 / 60 |
| - 64 | - 5/10/20 | - 1/8 | - 42 | - TDD | - 5:3 | N/A | N/A | N/A | - Not used | - Not used | - DL Type3 | - Not used | - Not used | - Not used | - UL Type1 | - Not used | - 311.428 / 60 |
| - 65 | - 5/10/20 | - 1/8 | - 43 | - TDD | - 5:3 | N/A | N/A | N/A | - Not used | - Not used | - Not used | - DL Type3 | - DL Type3 | - Not used | - Not used | - UL Type1 | - 311.428 / 60 |
| - 66 | - 5/10/20 | - 1/4 | - 0 | - TDD | - 5:2 | N/A | N/A | N/A | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 |  | - 139.988 - / 60 |
| - 67 | - 5/10/20 | - 1/4 | - 1 | - TDD | - 5:2 | N/A | N/A | N/A | - DL Type1 | - DL Type1 | - Not used | - Not used | - Not used | - UL Type1 | - Not used |  | - 139.988 - / 60 |
| - 68 | - 5/10/20 | - 1/4 | - 2 | - TDD | - 5:2 | N/A | N/A | N/A | - Not used | - Not used | - DL Type1 | - DL Type1 | - DL Type1 | - Not used | - UL Type1 |  | - 139.988 - / 60 |
| - 69 | - 5/10/20 | - 1/4 | - 3 | - TDD | - 4:3 | N/A | N/A | N/A | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type1 |  | - 139.988 - / 60 |
| - 70 | - 5/10/20 | - 1/4 | - 4 | - TDD | - 4:3 | N/A | N/A | N/A | - DL Type1 | - Not used | - Not used | - Not used | - UL Type1 | - Not used | - Not used |  | - 139.988 - / 60 |
| - 71 | - 5/10/20 | - 1/4 | - 5 | - TDD | - 4:3 | N/A | N/A | N/A | - Not used | - DL Type1 | - Not used | - Not used | - Not used | - UL Type1 | - Not used |  | - 139.988 - / 60 |
| - 72 | - 5/10/20 | - 1/4 | - 6 | - TDD | - 4:3 | N/A | N/A | N/A | - Not used | - Not used | - DL Type1 | - DL Type1 | - Not used | - Not used | - UL Type1 |  | - 139.988 - / 60 |
| - 73 | - 5/10/20 | - 1/4 | - 7 | - TDD | - 3:4 | N/A | N/A | N/A | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type1 | - UL Type1 |  | - 139.988 - / 60 |
| - 74 | - 5/10/20 | - 1/4 | - 8 | - TDD | - 3:4 | N/A | N/A | N/A | - DL Type1 | - Not used | - Not used | - UL Type1 | - UL Type1 | - Not used | - Not used |  | - 139.988 - / 60 |
| - 75 | - 5/10/20 | - 1/4 | - 9 | - TDD | - 3:4 | N/A | N/A | N/A | - Not used | - DL Type1 | - Not used | - Not used | - Not used | - UL Type1 | - Not used |  | - 139.988 - / 60 |
| - 76 | - 5/10/20 | - 1/4 | - 10 | - TDD | - 3:4 | N/A | N/A | N/A | - Not used | - Not used | - DL Type1 | - Not used | - Not used | - Not used | - UL Type1 |  | - 139.988 - / 60 |
| - 77 | - 5/10/20 | - 1/4 | - 11 | - FDD | - N/A | N/A | N/A | N/A | - D/U Type1 | - D/U Type1 | - D/U Type1 | - D/U Type2 | - D/U Type1 | - D/U Type1 | - D/U Type1 |  | - N/A |
| - 78 | - 5/10/20 | - 1/4 | - 12 | - FDD | - N/A | N/A | N/A | N/A | - D/U Type1 | - D/U Type1 | - Not used | - Not used | - Not used | - Not used | - Not used |  | - N/A |
| - 79 | - 5/10/20 | - 1/4 | - 13 | - FDD | - N/A | N/A | N/A | N/A | - Not used | - Not used | - D/U Type1 | - D/U Type2 | - Not used | - Not used | - Not used |  | - N/A |
| - 80 | - 5/10/20 | - 1/4 | - 14 | - FDD | - N/A | N/A | N/A | N/A | - Not used | - Not used | - Not used | - Not used | - D/U Type1 | - D/U Type1 | - Not used |  | - N/A |
| - 81 | - 5/10/20 | - 1/4 | - 15 | - FDD | - N/A | N/A | N/A | N/A | - Not used | - Not used | - Not used | - Not used | - Not used | - Not used | - D/U Type1 |  | - N/A |

Table Y+1 – Femto Frame Configuration and Indexing (8.75MHz channel bandwidth).

| - No. | - BW | - CP | - Frame configuration index | - Duplex | - D:U | - Subframe provision |  |  | - Subframe Type |  |  |  |  |  |  |  | - TTG /RTG - (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | - DL - offset | - DL length | - UL - length | - #0 | - #1 | - #2 | - #3 | - #4 | - #5 | - #6 | - #7 |  |
| - 1 | - 8.75 | - 1/16 | - 0 | - TDD | - 5:2 | N/A | N/A | N/A | - DL Type1 | - DL Type2 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type2 |  | - 138.4 - / 74.4 |
| - 2 | - 8.75 | - 1/16 | - 1 | - TDD | - 5:2 | N/A | N/A | N/A | - DL Type1 | - DL Type2 | - Not used | - Not used | - Not used | - UL Type1 | - Not used |  | - 138.4 - / 74.4 |
| - 3 | - 8.75 | - 1/16 | - 2 | - TDD | - 5:2 | N/A | N/A | N/A | - Not used | - Not used | - DL Type1 | - DL Type1 | - DL Type1 | - Not used | - UL Type2 |  | - 138.4 - / 74.4 |
| - 4 | - 8.75 | - 1/16 | - 3 | - TDD | - 4:3 | N/A | N/A | N/A | - DL Type1 | - DL Type2 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type2 |  | - 138.4 - / 74.4 |
| - 5 | - 8.75 | - 1/16 | - 4 | - TDD | - 4:3 | N/A | N/A | N/A | - DL Type1 | - Not used | - Not used | - Not used | - UL Type1 | - Not used | - Not used |  | - 138.4 - / 74.4 |
| - 6 | - 8.75 | - 1/16 | - 5 | - TDD | - 4:3 | N/A | N/A | N/A | - Not used | - DL Type2 | - Not used | - Not used | - Not used | - UL Type1 | - Not used |  | - 138.4 - / 74.4 |
| - 7 | - 8.75 | - 1/16 | - 6 | - TDD | - 4:3 | N/A | N/A | N/A | - Not used | - Not used | - DL Type1 | - DL Type1 | - Not used | - Not used | - UL Type2 |  | - 138.4 - / 74.4 |
| - 8 | - 8.75 | - 1/16 | - 7 | - TDD | - 3:4 | N/A | N/A | N/A | - DL Type1 | - DL Type2 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type1 | - UL Type2 |  | - 138.4 - / 74.4 |
| - 9 | - 8.75 | - 1/16 | - 8 | - TDD | - 3:4 | N/A | N/A | N/A | - DL Type1 | - Not used | - Not used | - UL Type1 | - UL Type1 | - Not used | - Not used |  | - 138.4 - / 74.4 |
| - 10 | - 8.75 | - 1/16 | - 9 | - TDD | - 3:4 | N/A | N/A | N/A | - Not used | - DL Type2 | - Not used | - Not used | - Not used | - UL Type1 | - Not used |  | - 138.4 - / 74.4 |
| - 11 | - 8.75 | - 1/16 | - 10 | - TDD | - 3:4 | N/A | N/A | N/A | - Not used | - Not used | - DL Type1 | - Not used | - Not used | - Not used | - UL Type2 |  | - 138.4 - / 74.4 |
| - 12 | - 8.75 | - 1/16 | - 11 | - FDD | - N/A | N/A | N/A | N/A | - D/U Type1 | - D/U Type2 | - D/U Type1 | - D/U Type2 | - D/U Type1 | - D/U Type1 | - D/U Type2 |  | - N/A |
| - 13 | - 8.75 | - 1/16 | - 12 | - FDD | - N/A | N/A | N/A | N/A | - D/U Type1 | - D/U Type2 | - Not used | - Not used | - Not used | - Not used | - Not used |  | - N/A |
| - 14 | - 8.75 | - 1/16 | - 13 | - FDD | - N/A | N/A | N/A | N/A | - Not used | - Not used | - D/U Type1 | - D/U Type2 | - Not used | - Not used | - Not used |  | - N/A |
| - 15 | - 8.75 | - 1/16 | - 14 | - FDD | - N/A | N/A | N/A | N/A | - Not used | - Not used | - Not used | - Not used | - D/U Type1 | - D/U Type1 | - Not used |  | - N/A |
| - 16 | - 8.75 | - 1/16 | - 15 | - FDD | - N/A | N/A | N/A | N/A | - Not used | - Not used | - Not used | - Not used | - Not used | - Not used | - D/U Type2 |  | - N/A |
| - 17 | - 8.75 | - 1/8 | - 0 | - TDD | - 5:2 | N/A | N/A | N/A | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 |  | - 87.2 - / 74.4 |
| - 18 | - 8.75 | - 1/8 | - 1 | - TDD | - 5:2 | N/A | N/A | N/A | - DL Type1 | - DL Type1 | - Not used | - Not used | - Not used | - UL Type1 | - Not used |  | - 87.2 - / 74.4 |
| - 19 | - 8.75 | - 1/8 | - 2 | - TDD | - 5:2 | N/A | N/A | N/A | - Not used | - Not used | - DL Type1 | - DL Type1 | - DL Type1 | - Not used | - UL Type1 |  | - 87.2 - / 74.4 |
| - 20 | - 8.75 | - 1/8 | - 3 | - TDD | - 4:3 | N/A | N/A | N/A | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type1 |  | - 87.2 - / 74.4 |
| - 21 | - 8.75 | - 1/8 | - 4 | - TDD | - 4:3 | N/A | N/A | N/A | - DL Type1 | - Not used | - Not used | - Not used | - UL Type1 | - Not used | - Not used |  | - 87.2 - / 74.4 |
| - 22 | - 8.75 | - 1/8 | - 5 | - TDD | - 4:3 | N/A | N/A | N/A | - Not used | - DL Type1 | - Not used | - Not used | - Not used | - UL Type1 | - Not used |  | - 87.2 - / 74.4 |
| - 23 | - 8.75 | - 1/8 | - 6 | - TDD | - 4:3 | N/A | N/A | N/A | - Not used | - Not used | - DL Type1 | - DL Type1 | - Not used | - Not used | - UL Type1 |  | - 87.2 - / 74.4 |
| - 24 | - 8.75 | - 1/8 | - 7 | - TDD | - 3:4 | N/A | N/A | N/A | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type1 | - UL Type1 |  | - 87.2 - / 74.4 |
| - 25 | - 8.75 | - 1/8 | - 8 | - TDD | - 3:4 | N/A | N/A | N/A | - DL Type1 | - Not used | - Not used | - UL Type1 | - UL Type1 | - Not used | - Not used |  | - 87.2 - / 74.4 |
| - 26 | - 8.75 | - 1/8 | - 9 | - TDD | - 3:4 | N/A | N/A | N/A | - Not used | - DL Type1 | - Not used | - Not used | - Not used | - UL Type1 | - Not used |  | - 87.2 - / 74.4 |
| - 27 | - 8.75 | - 1/8 | - 10 | - TDD | - 3:4 | N/A | N/A | N/A | - Not used | - Not used | - DL Type1 | - Not used | - Not used | - Not used | - UL Type1 |  | - 87.2 - / 74.4 |
| - 28 | - 8.75 | - 1/8 | - 11 | - FDD | - N/A | N/A | N/A | N/A | - D/U Type1 | - D/U Type1 | - D/U Type1 | - D/U Type2 | - D/U Type1 | - D/U Type1 | - D/U Type1 |  | - N/A |
| - 29 | - 8.75 | - 1/8 | - 12 | - FDD | - N/A | N/A | N/A | N/A | - D/U Type1 | - D/U Type1 | - Not used | - Not used | - Not used | - Not used | - Not used |  | - N/A |
| - 30 | - 8.75 | - 1/8 | - 13 | - FDD | - N/A | N/A | N/A | N/A | - Not used | - Not used | - D/U Type1 | - D/U Type2 | - Not used | - Not used | - Not used |  | - N/A |
| - 31 | - 8.75 | - 1/8 | - 14 | - FDD | - N/A | N/A | N/A | N/A | - Not used | - Not used | - Not used | - Not used | - D/U Type1 | - D/U Type1 | - Not used |  | - N/A |
| - 32 | - 8.75 | - 1/8 | - 15 | - FDD | - N/A | N/A | N/A | N/A | - Not used | - Not used | - Not used | - Not used | - Not used | - Not used | - D/U Type1 |  | - N/A |
| - 33 | - 8.75 | - 1/8 | - 16 | - TDD | - 5:2 | 3 | 2 | 2 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type4 | - Not used | - Not used | - Not used |  | - 87.2 - / 74.4 |
| - 34 | - 8.75 | - 1/8 | - 17 | - TDD | - 5:2 | 3 | 2 | 1 | - DL Type1 | - DL Type1 | - Not used | - UL Type1 | - Not used | - Not used | - Not used |  | - 87.2 - / 74.4 |
| - 35 | - 8.75 | - 1/8 | - 18 | - TDD | - 5:2 | 2 | 3 | 2 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type4 | - Not used | - Not used |  | - 87.2 - / 74.4 |
| - 36 | - 8.75 | - 1/8 | - 19 | - TDD | - 5:2 | 2 | 3 | 1 | - DL Type1 | - DL Type1 | - DL Type1 | - Not used | - UL Type1 | - Not used | - Not used |  | - 87.2 - / 74.4 |
| - 37 | - 8.75 | - 1/8 | - 20 | - TDD | - 5:2 | 2 | 2 | 2 | - DL Type1 | - DL Type1 | - Not used | - UL Type1 | - UL Type4 | - Not used | - Not used |  | - 87.2 - / 74.4 |
| - 38 | - 8.75 | - 1/8 | - 21 | - TDD | - 5:2 | 2 | 2 | 1 | - DL Type1 | - DL Type1 | - Not used | - Not used | - UL Type1 | - Not used | - Not used |  | - 87.2 - / 74.4 |
| - 39 | - 8.75 | - 1/8 | - 22 | - TDD | - 5:2 | 2 | 2 | 1 | - DL Type1 | - DL Type1 | - Not used | - Not used | - UL Type1 | - Not used | - Not used |  | - 87.2 - / 74.4 |
| - 40 | - 8.75 | - 1/8 | - 23 | - TDD | - 5:2 | 1 | 4 | 2 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type4 | - Not used |  | - 87.2 - / 74.4 |
| - 41 | - 8.75 | - 1/8 | - 24 | - TDD | - 5:2 | 1 | 4 | 1 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - Not used | - UL Type1 | - Not used |  | - 87.2 - / 74.4 |
| - 42 | - 8.75 | - 1/8 | - 25 | - TDD | - 5:2 | 1 | 3 | 2 | - DL Type1 | - DL Type1 | - DL Type1 | - Not used | - UL Type1 | - UL Type4 | - Not used |  | - 87.2 - / 74.4 |
| - 43 | - 8.75 | - 1/8 | - 26 | - TDD | - 5:2 | 1 | 3 | 1 | - DL Type1 | - DL Type1 | - DL Type1 | - Not used | - Not used | - UL Type1 | - Not used |  | - 87.2 - / 74.4 |
| - 44 | - 8.75 | - 1/8 | - 27 | - TDD | - 5:2 | 1 | 2 | 2 | - DL Type1 | - DL Type1 | - Not used | - Not used | - UL Type1 | - UL Type4 | - Not used |  | - 87.2 - / 74.4 |
| - 45 | - 8.75 | - 1/8 | - 28 | - TDD | - 5:2 | 1 | 2 | 1 | - DL Type1 | - DL Type1 | - Not used | - Not used | - Not used | - UL Type1 | - Not used |  | - 87.2 - / 74.4 |
| - 46 | - 8.75 | - 1/4 | - 0 | - TDD | - 4:2 | N/A | N/A | N/A | - DL Type1 | - DL Type2 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type2 |  |  | - (TBD) |
| - 47 | - 8.75 | - 1/4 | - 1 | - TDD | - 4:2 | N/A | N/A | N/A | - DL Type1 | - DL Type2 | - Not used | - Not used | - UL Type1 | - Not used |  |  | - (TBD) |
| - 48 | - 8.75 | - 1/4 | - 2 | - TDD | - 4:2 | N/A | N/A | N/A | - Not used | - Not used | - DL Type1 | - DL Type1 | - Not used | - UL Type2 |  |  | - (TBD) |
| - 49 | - 8.75 | - 1/4 | - 3 | - TDD | - 3:3 | N/A | N/A | N/A | - DL Type1 | - DL Type2 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type2 |  |  | - (TBD) |
| - 50 | - 8.75 | - 1/4 | - 4 | - TDD | - 3:3 | N/A | N/A | N/A | - DL Type1 | - Not used | - Not used | - UL Type1 | - Not used | - Not used |  |  | - (TBD) |
| - 51 | - 8.75 | - 1/4 | - 5 | - TDD | - 3:3 | N/A | N/A | N/A | - Not used | - DL Type2 | - Not used | - Not used | - UL Type1 | - Not used |  |  | - (TBD) |
| - 52 | - 8.75 | - 1/4 | - 6 | - TDD | - 3:3 | N/A | N/A | N/A | - Not used | - Not used | - DL Type1 | - Not used | - Not used | - UL Type2 |  |  | - (TBD) |
| - 53 | - 8.75 | - 1/4 | - 7 | - TDD | - 2:4 | N/A | N/A | N/A | - DL Type1 | - DL Type2 | - UL Type1 | - UL Type1 | - UL Type1 | - UL Type2 |  |  | - (TBD) |
| - 54 | - 8.75 | - 1/4 | - 8 | - TDD | - 2:4 | N/A | N/A | N/A | - DL Type1 | - Not used | - UL Type1 | - UL Type1 | - Not used | - Not used |  |  | - (TBD) |
| - 55 | - 8.75 | - 1/4 | - 9 | - TDD | - 2:4 | N/A | N/A | N/A | - Not used | - DL Type2 | - Not used | - Not used | - UL Type1 | - UL Type2 |  |  | - (TBD) |
| - 56 | - 8.75 | - 1/4 | - 10 | - FDD | - N/A | N/A | N/A | N/A | - D/U Type1 | - D/U Type2 | - D/U Type1 | - D/U Type2 | - D/U Type1 | - D/U Type2 |  |  | - N/A |
| - 57 | - 8.75 | - 1/4 | - 11 | - FDD | - N/A | N/A | N/A | N/A | - D/U Type1 | - D/U Type2 | - Not used | - Not used | - Not used | - Not used |  |  | - N/A |
| - 58 | - 8.75 | - 1/4 | - 12 | - FDD | - N/A | N/A | N/A | N/A | - Not used | - Not used | - D/U Type1 | - D/U Type2 | - Not used | - Not used |  |  | - N/A |
| - 59 | - 8.75 | - 1/4 | - 13 | - FDD | - N/A | N/A | N/A | N/A | - Not used | - Not used | - Not used | - Not used | - D/U Type1 | - D/U Type2 |  |  | - N/A |

Table Y+2 – Femto Frame Configuration and Indexing (7MHz channel bandwidth).

| - No. | - BW | - CP | - Frame configuration index | - Duplex | - D:U | - Subframe provision |  |  | - Subframe Type |  |  |  |  |  |  |  | - TTG /RTG - (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | - DL - offset | - DL length | - UL - length | - #0 | - #1 | - #2 | - #3 | - #4 | - #5 | - #6 | - #7 |  |
| - 1 | - 7 | - 1/16 | - 0 | - TDD | - 4:2 | N/A | N/A | N/A | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type3 | - UL Type1 | - UL Type1 |  |  | - 180 - / 60 |
| - 2 | - 7 | - 1/16 | - 1 | - TDD | - 4:2 | N/A | N/A | N/A | - DL Type1 | - DL Type1 | - Not used | - Not used | - UL Type1 | - Not used |  |  | - 180 - / 60 |
| - 3 | - 7 | - 1/16 | - 2 | - TDD | - 4:2 | N/A | N/A | N/A | - Not used | - Not used | - DL Type1 | - DL Type3 | - Not used | - UL Type1 |  |  | - 180 - / 60 |
| - 4 | - 7 | - 1/16 | - 3 | - TDD | - 3:3 | N/A | N/A | N/A | - DL Type1 | - DL Type1 | - DL Type3 | - UL Type1 | - UL Type1 | - UL Type1 |  |  | - 180 - / 60 |
| - 5 | - 7 | - 1/16 | - 4 | - TDD | - 3:3 | N/A | N/A | N/A | - DL Type1 | - Not used | - Not used | - UL Type1 | - Not used | - Not used |  |  | - 180 - / 60 |
| - 6 | - 7 | - 1/16 | - 5 | - TDD | - 3:3 | N/A | N/A | N/A | - Not used | - DL Type1 | - Not used | - Not used | - UL Type1 | - Not used |  |  | - 180 - / 60 |
| - 7 | - 7 | - 1/16 | - 6 | - TDD | - 3:3 | N/A | N/A | N/A | - Not used | - Not used | - DL Type3 | - Not used | - Not used | - UL Type1 |  |  | - 180 - / 60 |
| - 8 | - 7 | - 1/16 | - 7 | - TDD | - 2:4 | N/A | N/A | N/A | - DL Type1 | - DL Type3 | - UL Type1 | - UL Type1 | - UL Type1 | - UL Type1 |  |  | - 180 - / 60 |
| - 9 | - 7 | - 1/16 | - 8 | - TDD | - 2:4 | N/A | N/A | N/A | - DL Type1 | - Not used | - UL Type1 | - UL Type1 | - Not used | - Not used |  |  | - 180 - / 60 |
| - 10 | - 7 | - 1/16 | - 9 | - TDD | - 2:4 | N/A | N/A | N/A | - Not used | - DL Type3 | - Not used | - Not used | - UL Type1 | - UL Type1 |  |  | - 180 - / 60 |
| - 11 | - 7 | - 1/16 | - 10 | - FDD | - N/A | N/A | N/A | N/A | - D/U Type1 | - D/U Type1 | - D/U Type1 | - D/U Type1 | - D/U Type1 | - D/U Type1 |  |  | - N/A |
| - 12 | - 7 | - 1/16 | - 11 | - FDD | - N/A | N/A | N/A | N/A | - D/U Type1 | - D/U Type1 | - Not used | - Not used | - Not used | - Not used |  |  | - N/A |
| - 13 | - 7 | - 1/16 | - 12 | - FDD | - N/A | N/A | N/A | N/A | - Not used | - Not used | - D/U Type1 | - D/U Type1 | - Not used | - Not used |  |  | - N/A |
| - 14 | - 7 | - 1/16 | - 13 | - FDD | - N/A | N/A | N/A | N/A | - Not used | - Not used | - Not used | - Not used | - D/U Type1 | - D/U Type1 |  |  | - N/A |
| - 15 | - 7 | - 1/8 | 0 | TDD | 3:2 | N/A | N/A | N/A | DL Type1 | DL Type2 | DL Type2 | UL Type1 | UL Type2 |  |  |  | 188 / 60 |
| - 16 | - 7 | - 1/8 | 1 | TDD | 3:2 | N/A | N/A | N/A | DL Type1 | - Not used | - Not used | UL Type1 | - Not used |  |  |  | 188 / 60 |
| - 17 | - 7 | - 1/8 | 2 | TDD | 3:2 | N/A | N/A | N/A | - Not used | DL Type2 | DL Type2 | - Not used | UL Type2 |  |  |  | 188 / 60 |
| - 18 | - 7 | - 1/8 | 3 | TDD | 2:3 | N/A | N/A | N/A | DL Type1 | DL Type2 | UL Type1 | UL Type2 | UL Type2 |  |  |  | 188 / 60 |
| - 19 | - 7 | - 1/8 | 4 | TDD | 2:3 | N/A | N/A | N/A | DL Type1 | - Not used | UL Type1 | UL Type2 | - Not used |  |  |  | 188 / 60 |
| - 20 | - 7 | - 1/8 | 5 | TDD | 2:3 | N/A | N/A | N/A | - Not used | DL Type2 | - Not used | - Not used | UL Type2 |  |  |  | 188 / 60 |
| - 21 | - 7 | - 1/8 | 6 | FDD | N/A | N/A | N/A | N/A | D/U Type1 | D/U Type2 | D/U Type2 | D/U Type2 | D/U Type2 |  |  |  | N/A |
| - 22 | - 7 | - 1/8 | 7 | FDD | N/A | N/A | N/A | N/A | D/U Type1 | D/U Type2 | - Not used | - Not used | - Not used |  |  |  | N/A |
| - 23 | - 7 | - 1/8 | 8 | FDD | N/A | N/A | N/A | N/A | - Not used | - Not used | D/U Type2 | D/U Type2 | - Not used |  |  |  | N/A |
| - 24 | - 7 | - 1/8 | 9 | FDD | N/A | N/A | N/A | N/A | - Not used | - Not used | - Not used | - Not used | D/U Type2 |  |  |  | N/A |
| - 25 | - 7 | - 1/8 | 10 | TDD | 3:2 | 2 | 2 | 2 | DL Type1 | DL Type2 | UL Type1 | UL Type1 | Not used |  |  |  | 188 / 60 |
| - 26 | - 7 | - 1/8 | 11 | TDD | 3:2 | 2 | 2 | 1 | DL Type1 | DL Type2 | Not used | UL Type1 | Not used |  |  |  | 188 / 60 |
| - 27 | - 7 | - 1/4 | - 0 | - TDD | - 3:2 | N/A | N/A | N/A | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 |  |  |  | - 140 - / 60 |
| - 28 | - 7 | - 1/4 | - 1 | - TDD | - 3:2 | N/A | N/A | N/A | - DL Type1 | - Not used | - Not used | - UL Type1 | - Not used |  |  |  | - 140 - / 60 |
| - 29 | - 7 | - 1/4 | - 2 | - TDD | - 3:2 | N/A | N/A | N/A | - Not used | - DL Type1 | - DL Type1 | - Not used | - UL Type1 |  |  |  | - 140 - / 60 |
| - 30 | - 7 | - 1/4 | - 3 | - TDD | - 2:3 | N/A | N/A | N/A | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type1 |  |  |  | - 140 - / 60 |
| - 31 | - 7 | - 1/4 | - 4 | - TDD | - 2:3 | N/A | N/A | N/A | - DL Type1 | - Not used | - UL Type1 | - UL Type1 | - Not used |  |  |  | - 140 - / 60 |
| - 32 | - 7 | - 1/4 | - 5 | - TDD | - 2:3 | N/A | N/A | N/A | - Not used | - DL Type1 | - Not used | - Not used | - UL Type1 |  |  |  | - 140 - / 60 |
| - 33 | - 7 | - 1/4 | - 6 | - FDD | - N/A | N/A | N/A | N/A | - D/U Type1 | - D/U Type1 | - D/U Type2 | - D/U Type1 | - D/U Type1 |  |  |  | - N/A |
| - 34 | - 7 | - 1/4 | - 7 | - FDD | - N/A | N/A | N/A | N/A | - D/U Type1 | - D/U Type1 | - Not used | - Not used | - Not used |  |  |  | - N/A |
| - 35 | - 7 | - 1/4 | - 8 | - FDD | - N/A | N/A | N/A | N/A | - Not used | - Not used | - D/U Type2 | - D/U Type1 | - Not used |  |  |  | - N/A |
| - 36 | - 7 | - 1/4 | - 9 | - FDD | - N/A | N/A | N/A | N/A | - Not used | - Not used | - Not used | - Not used | - D/U Type1 |  |  |  | - N/A |

#### 16.4.11.2 Set of DFPC for Femto ABS

To configure DFPC in S-SFH SP2, the Femto ABS shall refer to Table Y+3 to indicate the starting PRU number of the subframe configured by the Frame Configuration Index.

Table Y+3 – Mapping between DFPC and Starting PRU number for Femto

| DFPC | Starting PRU number |
| --- | --- |
| 0 | 0 |
| 1 | NPRU * 1/4 |
| 2 | NPRU * 1/2 |
| 3 | NPRU * 3/4 |
| 4-7 (for 3-bit DFPC) or 4-15 (for 4-bit DFPC) | Reserved |

============================== End of Proposed Text ================================

References

- IEEE P802.16m/D3. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, Dec 2009.

- IEEE 802.16m-08/0034r2. IEEE 802.16m System Description Document, Sep 2009.

- IEEE 802.16m-07/002r9. IEEE 802.16m System Requirements Document, Sep 2009.