| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AMS Identifier in the DCR Mode (16.2.18) |  |
| Date Submitted | 2009-12-30 |  |
| Source(s) | Ming-Hung Tao, Ying-Chuan Hsiao ITRI | Voice: +886-3-5914629 E-mail: MHTao@itri.org.tw |
| Re: | Category: LB30b / Area: Chapter 16.2.18 (Deregistration with content retention mode) |  |
| Abstract | This contribution proposes changes to the Section 16.2.18 of IEEE P802.16m/D3 |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AMS Identifier in the DCR Mode (16.2.18)

Ming-Hung Tao, Ying-Chuan Hsiao

ITRI

- Introduction

In current D3 [1], the description for the network entity which manages the DCR mode operation is not clear. If the network entity means paging controller, we can reuse the idle mode identification mechanism (DID, paging offset, paging cycle) for the DCR mode operation. However, if the network entity is a different entity rather than the paging controller, we need to define a new AMS identifier for the DCR operation to avoid ID assignment/exchange complexity.

In case of reusing the idle mode identifier as the DCR mode identifier, 10-bits DID is not sufficient to identify all DCR mode AMSs in the network. Therefore we suggest assigning the DCR mode AMS with the paging cycle equal to 0, and combining DID with paging offset as the identifier for the DCR mode AMS. In this case, we will have sufficient bits (>14 bits) to identify DCR mode AMSs.

In case of redefining a new identifier for the DCR mode operation, since the new defined identifier should be large enough to identify all DCR mode AMSs, we suggest using at least 14 bits for this new identifier.

- Proposed Text for IEEE P802.16m/D3

================================= Remedy 2 =======================================

[Modify the paragraph from lines 23 on page 268 of P802.16m/D3 as follows.]

16.2.18 Deregistration with content retention (DCR) mode

…

While the idle mode retention timer is valid, the network retains AMS’s information which is used to expedite AMS’s network re-entry. Retention identifier (RID) Deregistration identifier (DID) is assigned to AMS during DCR mode initiation from connected state or idle mode. with paging cycle and paging offset value of 0. RID DID together with the Authenticator ID of the network entity that assigns/retains the context is used to uniquely identify the DCR mode AMSs.

[Modify the paragraph from lines 38 on page 268 of P802.16m/D3 as follows.]

16.2.18.1 DCR initiation in connected state

…

In the AAI-DREG-CMD, DID RID shall be assigned which is used for network re-entry from DCR mode to identify the AMS.

…

[Modify the paragraph from lines 53 on page 268 of P802.16m/D3 as follows.]

16.2.18.2 DCR mode initiation from idle mode

…

Upon successful DCR mode change request, the network shall initiate DCR mode operation by retaining AMS’s information until the idle mode retention timer is valid. At the time of DCR mode change, paging cycle, paging offset, paging controller values shall be updated to be 0 for the AMS and the RID and the Authenticator ID of the network entity that assigns/retains the context shall be used to uniquely identify DCR mode AMS.

[Modify the paragraph from lines 11 on page 269 of P802.16m/D3 as follows.]

- 16.2.18.4 Network reentry from DCR mode

For the network reentry from DCR mode, the AMS shall initiate network reentry with the ABS by sending an AAI_RNG-REQ message with ranging purpose code of 0x06 and the RID and the Authenticator ID of the network entity that assigns/retains the context. Rest of reentry procedure shall be performed same as network reentry from idle mode described in section 16.2.17.5.

[Insert the following fields to Table-674 from line 32 on page 46 of P802.16m/D3.]

Table 674—Parameters for AAI_ RNG-REQ

| Name | Value | Usage |
| --- | --- | --- |
| Authenticator ID | Anchor authenticator ID which maintains the AMS’s context. | It shall be included when the AMS is attempting to perform DCR mode extension or network reentry from DCR mode/coverage loss. |
| Retention Identifier (RID) | The ID together with ID of the network entity that assigns/retains the context is used to uniquely identify the DCR mode AMSs. | It shall be included when the AMS is attempting to perform DCR mode extension or network reentry from DCR mode. |

[Insert the following fields to Table-675 from line 32 on page 48 of P802.16m/D3.]

Table 675—parameters for AAI_ RNG-RSP

| Name | Value | Usage |
| --- | --- | --- |
| Retention Identifier (RID) | The ID together with ID of the network entity that assigns/retains the context is used to uniquely identify the DCR mode AMSs. | It shall be included when the AMS is attempting to enter DCR mode from idle mode. |

[Modify the paragraph from lines 2 on page 83 of P802.16m/D3 as follows.]

- 16.2.3.22 AAI_DREG-RSP message

When the action code is set to 0x08, the following parameters shall be included in AAI_DREG-RSP message.

Idle Mode Retain Information

Retention identifier (RID) Deregistration identifier (DID)

============================== End of Remedy 2 ================================

References

- IEEE P802.16m/D3. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, Dec 2009.

- IEEE 802.16m-08/0034r2. IEEE 802.16m System Description Document, Sep 2009.

- IEEE 802.16m-07/002r9. IEEE 802.16m System Requirements Document, Sep 2009.