| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed modifications to the uplink feedback formats (16.3.9) |  |
| Date Submitted | 2009-12-30 |  |
| Source(s) | David Mazzarese, Sangheon Kim, Hokyu Choi, Heewon Kang Samsung Electronics | d.mazzarese@samsung.com |
| Re: | P802.16m/D3 LB30b |  |
| Abstract | This contribution proposes change to the feedback formats for uplink fast feedback channels |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 3) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed modifications to the uplink feedback formats (16.3.9)

David Mazzarese, Sangheon Kim, Hokyu Choi, Heewon Kang

Samsung Electronics Co., Ltd.

- Introduction

This contribution proposes changes to the feedback formats:

- The number of bits for encoding the stream index is specified.

- The feedback formats used when the short period equals the long period are specified for each MFM.

- Feedback formats for MFM 1 outside the OL region are deleted, as the support of MFM 1 outside the OL region is removed by this proposal.

- Constraints on CS (Codebook_subset) parameter are added to ensure interoperability between ABS and AMS for MFM3 and MFM6 with feedback format 3, in order to avoid exceeding 24 bits in SFBCH.

- The meaning of Base PMI in the feedback formats for MFM 3 and 6 with differential codebook is not clear since it is never defined. This is a PMI from the base codebook or base codebook subset, depending on CS (Codebook_subset) in Feedback Allocation A-MAP IE. It cannot be from the differential codebook.

- Remove the constraint of STC rate 1 with MFM 4 in case of single-user wideband beamforming.

- Remove the constraint of STC rate 1 or 2 in MFM 2 with best-1 subband feedback in PFBCH

Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and struck through and existing AWD text is shown in black.

- References

[1] DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D3, Dec 2009.

# Proposed AWD text

<Modify the section 16.3.9.3.1.5 on line 6, page 550 as shown >

----------------------------------------- Start of the Proposed Amendment Text --------------------------------------------

# 16.3.9.3.1.5 Feedback formats

Feedback formats define the information content carried by the fast feedback channels. The format of the content of the PFBCH is determined by the PFBCH encoding type. The SFBCH payload information bits may carry subbands index (Best_subbands_index), STC rate, wideband STC rate, PFBCH indicator, subband CQI , subband avg_CQI, differential CQI, stream index, wideband PMI, subband PMI, base PMI and differential PMI. The SFBCH payload information bits a0a1...a11N-1 are defined in the order specified in Tables 887 904 to 895 910.

STC rate and wideband STC rate shall be encoded with 1, 2 or 3 bits when the ABS has 2, 4 or 8 transmit antennas, respectively.

Subband CQI and subband avg_CQI shall be encoded with 4 bits corresponding to the nominal MCS of Table 886.

Differential CQI shall be encoded with 2 bits indicating values {-1, 0, +1, +2}.

Wideband PMI, subband PMI and base PMI shall be encoded with 3, 4 or 6 bits according to the codebook size.

Differential PMI shall be encoded with 2, 4 or 4 bits when the ABS has 2, 4 or 8 transmit antennas, respectively.

Stream index shall be encoded with ceil(log2(MaxMt)) bits.

Best_subbands_index shall be encoded withbits, whereis the combination operation.

The total number of subbands YSB available for feedback is shown in equation (268 ):

( 268)

Where LSB,FPmLSB-CRU,FPm is given in Equation (188) section 15.3.5.3 the number of allocated subband CRUs for FPm.

# Feedback format for MFM 0,1,4,7

Feedback formats for MFM 0, 1, 4, and 7 are listed in Table 902. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report according to long term feedback period in feedback allocation A-MAP IE. The long period report shall start by puncturing the first short period report if MFM = 4 or 7, or by puncturing the 2q-th short period report if MFM = 0 or 1, where q is defined in the feedback allocation A-MAP IE. When q=0, only the short period reports shall be sent. If MFM = 4 or 7 and q=0, the ABS should allocate the feedback of the transmit correlation matrix using feedback polling A-MAP IE before allocating MFM 4 or 7 to the AMS.

The wideband CQI is one average CQI corresponding to the MIMO mode signaled by the combination MFM and STC rate, with averaging over the whole band. In the case where the number of DL frequency partitions FPCT > 1, the wideband CQI is one average CQI over the corresponding frequency partition.

When MFM=4 or 7 and q>0, AMS should calculate CQI assuming that ABS will calculate the beamforming matrix utilizing the latest PMI which is fed back to the ABS. When MFM=4 or 7 and q=0, AMS should calculate CQI assuming that ABS will calculate the beamforming matrix utilizing the latest successfully received transmit correlation matrix which is fed back to the ABS.

Table 902 shows feedback formats for MIMO feedback mode 0, 1, 4, and 7 when Measurement Method Indication = 0b0 in Feedback Allocation A-MAP IE (operation outside the open-loop region).

Table 902 - Feedback formats for MIMO feedback mode 0, 1, 4, and 7 (outside OL region)

| MFM | FBCH | Number of reports | Report period | Feedback Fields | Description/Notes |
| --- | --- | --- | --- | --- | --- |
| 0 | PFBCH | 2 | Short | Wideband CQI and STC rate | Joint encoding of CQI and STC rate Encoding type 0 |
|  |  |  | Long | Wideband CQI and STC rate | Joint encoding of CQI and STC rate Encoding type 0 Long term FPI for FFR No long report when q=0 |
| 1 | PFBCH | 2 | Short | Wideband CQI and STC rate | Joint encoding of CQI and STC rate Encoding type 0 |
|  |  |  | Long | Wideband CQI and STC rate | Joint encoding of CQI and STC rate Encoding type 0 Long term FPI for FFR |
| 4 | PFBCH | 2 | Short | Wideband CQI and STC rate | STC rate = 1 Encoding type 0 |
|  |  |  | Long | Wideband PMI | PMI for rank 1 Encoding type 2 No long report when q=0 |
| 7 | PFBCH | 2 | Short | Wideband CQI | STC rate = 1 Encoding type 0 |
|  |  |  | Long | Wideband PMI | PMI for rank 1 Encoding type 2 No long report when q=0 |

Table 903 shows Feedback formats for MIMO feedback mode 0 and 1 when Measurement Method Indication = 0b1 in Feedback Allocation A-MAP IE (operation inside the open-loop region). The ABS shall not assign a feedback channel on the short periods.

Table 903 - Feedback formats for MIMO feedback mode 0, 1 (inside OL region)

| MFM | FBCH | Number of reports | Report period | Feedback Fields | Description/Notes |
| --- | --- | --- | --- | --- | --- |
| 0 | PFBCH | 1 | Long | Wideband CQI and STC rate | Joint encoding of CQI and STC rate Encoding type 0 |
| 1 | PFBCH | 1 | Long | Wideband CQI | CQI for STC rate = 1/2 Encoding type 3 |

# Feedback format for MFM 2

The detailed format is listed in Table 889. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report according to long term feedback period in feedback allocation A-MAP IE. The long period report shall start by puncturing the first short period report. If the PFBCH indicator in SFBCH is set to '1', PFBCH is transmitted by puncturing SFBCH in the next feedback opportunity regardless of short-term and long-term feedback using encoding type 0. When q=0, only the long period reports shall be sent. q=0 shall not be set if Measurement Method Indication = 0b1.

The CQI of subband m shall be computed as follows: (Subband m CQI index) = (Subband avg CQI index) + (Subband m differential CQI), using the latest available reports of Subband avg CQI and Subband differential CQI. Subband avg CQI index is an average measure of the CQI over the M reported subbands. The possible differential CQI values are {-1, 0, +1, +2}. The AMS shall ensure that the reported differential CQI will produce a value of subband m CQI in the range of 0 to 15.

Table 904 shows Feedback formats for MIMO feedback mode 2 when Measurement Method Indication = 0b0 in Feedback Allocation A-MAP IE (operation outside the open-loop region).

Table 904 - Feedback formats for MFM 2 (outside OL Region)

| Feedback Format | FBCH | Number of reports | Report period | Feedback Fields | Description/Notes |
| --- | --- | --- | --- | --- | --- |
| 0 (M = 1) | PFBCH | 2 | Short | Subband CQI and STC rate (rate=1 and 2) | Joint encoding of CQI and STC rate with PFBCH encoding Type 0. No short report when q=0 |
|  |  |  | Long | Best_subbands_index | PFBCH encoding Type 1 |
| 1 (M = 1) | SFBCH | 1 | Long | Best_subbands_index Subband CQI STC rate PFBCH indicator | Support of STC rate 1 to 8 |
| 2 (M=min{5,YSB}) | SFBCH | 2 | Short | Subband avg CQI for (m=1:M){ differential CQI } | No short report when q=0 |
|  |  |  | Long | Best_subbands_index Wideband STC rate PFBCH indicator | Support of STC rate 1 to 8 |
| 3 (M=min{10,YSB}) | SFBCH | 2 | Short | Subband avg CQI for (m=1:M){ differential CQI } | No short report when q=0 |
|  |  |  | Long | Best_subbands_index Wideband STC rate PFBCH indicator | Support of STC rate 1 to 8 |

Table 905 shows Feedback formats for MIMO feedback mode 2 when Measurement Method Indication = 0b1 in Feedback Allocation A-MAP IE (operation inside the open-loop region).

Table 905 - Feedback formats for MFM 2 (inside OL Region)

| Feedback Format | FBCH | Number of reports | Report period | Feedback Fields | Description/Notes |
| --- | --- | --- | --- | --- | --- |
| 0 (M = 1) | PFBCH | 2 | Short | Subband CQI for STC rate=1/2 | Joint encoding of CQI with STC rate =1/2 PFBCH encoding Type 3 |
|  |  |  | Long | Best_subbands_index | PFBCH encoding Type 1 |
| 1 (M = 1) | SFBCH | 1 | Long | Best_subbands_index Subband CQI PFBCH indicator | STC rate = ½ |
| 2 (M=min{5,YSB}) | SFBCH | 2 | Short | Subband avg CQI for (m=1:M){ differential CQI } | STC rate = ½ |
|  |  |  | Long | Best_subbands_index PFBCH indicator |  |
| 3 (M=min{10,YSB}) | SFBCH | 2 | Short | Subband avg CQI for (m=1:M){ differential CQI } |  |
|  |  |  | Long | Best_subbands_index PFBCH indicator |  |

# Feedback format for MFM 3

The detailed format is listed in Table 891. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report according to long term feedback period in feedback allocation A-MAP IE. The long period report shall start by puncturing the first short period report. For feedback format 0 with 3 reports using the PFBCH, Subband PMI shall be transmitted first after every report of Best_subbands_index. Subband CQI and subband PMI are then transmitted alternately and Best_subbands_index is transmitted in the long term period. If the PFBCH indicator in SFBCH is set to '1', PFBCH is transmitted using encoding type 0 by puncturing SFBCH in the next feedback opportunity regardless of short-term and long-term feedback. When q=0, only the long period reports shall be sent. q=0 shall not be set with the differential feedback mode.

The CQI of subband m shall be computed as follows: (Subband m CQI index) = (Subband avg CQI index) + (Subband m differential CQI), using the latest available reports of Subband avg CQI and Subband differential CQI. Subband avg CQI index is an average measure of the CQI over the M reported subbands. The possible differential CQI values are {-1, 0, +1, +2}. The AMS shall ensure that the reported differential CQI will produce a value of subband m CQI in the range of 0 to 15.

If the ABS has 4 transmit antennas and assigns MFM3 with feedback format 3, then it shall set Codebook_subset = 0b1 except if Codebook_mode indicates the differential codebook-based feedback mode.

The detailed format for MIMO feedback mode 3 with differential codebook is in Table 907. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE. For M > 1, two long term reports will puncture two short term reports continuously according to long term feedback period in feedback allocation A-MAP IE. For M = 1, the long term report will puncture one short term report according to long term feedback period in feedback allocation A-MAP IE.

Table 906 - Feedback formats for MFM 3

| Feedback Format | FBCH | Number of reports | Report period | Feedback Fields | Description/Notes |
| --- | --- | --- | --- | --- | --- |
| 0 (M = 1) | PFBCH | 3 | Short | Subband CQI | PFBCH encoding Type 0 No short report when q=0 |
|  |  |  | Short | Subband PMI | PFBCH encoding Type 2 STC rate = 1 No short report when q=0 |
|  |  |  | Long | Best_subbands_index | PFBCH encoding Type 1 |
| 1 (M = 2) | SFBCH | 2 | Short | for (m=1:M){ Subband differential CQI Subband PMI } | No short report when q=0 |
|  |  |  | Long | Best_subbands_index Wideband STC rate Subband avg CQI PFBCH indicator |  |
| 2 (M = 3) | SFBCH | 2 | Short | for (m=1:M){ Subband differential CQI Subband PMI } | No short report when q=0 |
|  |  |  | Long | Best_subbands_index Wideband STC rate Subband avg CQI PFBCH indicator |  |
| 3 (M = 4) | SFBCH | 2 | Short | for (m=1:M){ Subband differential CQI Subband PMI } | No short report when q=0 |
|  |  |  | Long | Best_subbands_index Wideband STC rate Subband avg CQI PFBCH indicator |  |

Table 907 - Feedback formats for MFM 3 for differential codebook

| Feedback Format | FBCH | Number of Reports | Feedback Fields |  | Description/Notes |
| --- | --- | --- | --- | --- | --- |
| 0 (M =1) | SFBCH | 2 | Short | Subband CQI Differential PMI Padding | Padding 1 bit: If Nt=2 0 bit: Otherwise |
|  |  |  | Long | Best_subbands_index STC rate Base PMI PFBCH indicator | PMI for reset. Base PMI is a PMI from the base codebook or the base codebook subset. |
| 1 (M=2) | SFBCH | 3 | Short | for (m=1:M){ Subband differential CQI Subband Differential PMI } |  |
|  |  |  | Long | Best_subbands_index subband avg_CQI PFBCH indicator |  |
|  |  |  | Long | for (m=1:M){ Base PMI } Wideband STC rate | PMI for reset. Base PMI is a PMI from the base codebook or the base codebook subset. |
| 2 (M =3) | SFBCH | 3 | Short | for (m=1:M){ Subband differential CQI Subband Differential PMI } |  |
|  |  |  | Long | Best_subbands_index Wideband STC rate subband avg_CQI PFBCH indicator |  |
|  |  |  | Long | for (m=1:M){ Base PMI } | PMI for reset. Base PMI is a PMI from the base codebook or the base codebook subset. |
| 3 (M =4) | SFBCH | 3 | Short | for (m=1:M){ Subband differential CQI Subband Differential PMI } |  |
|  |  |  | Long | Best_subbands_index Wideband STC rate subband avg_CQI PFBCH indicator |  |
|  |  |  | Long | for (m=1:M){ Base PMI } | PMI for reset. Base PMI is a PMI from the base codebook or the base codebook subset. |

# Feedback format for MFM 5

The detailed format is listed in Table 893. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report according to long term feedback period in feedback allocation A-MAP IE. The long period report shall start by puncturing the first short period report. If the PFBCH indicator in SFBCH is set to '1', PFBCH is transmitted by puncturing SFBCH in the next feedback opportunity regardless of short-term and long-term feedback using encoding type 0. When q=0, only the long period reports shall be sent. q=0 shall not be set if Measurement Method Indication = 0b1.

The CQI of subband m shall be computed as follows: (Subband m CQI index) = (Subband avg CQI index) + (Subband m differential CQI), using the latest available reports of Subband avg CQI and Subband differential CQI. Subband avg CQI index is an average measure of the CQI over the M reported subbands. The possible differential CQI values are {-1, 0, +1, +2}. The AMS shall ensure that the reported differential CQI will produce a value of subband m CQI in the range of 0 to 15.

The feedback formats for MIMO feedback mode 5 are the same for operation inside or outside the open-loop region.

Table 908 - Feedback formats for MFM 5

| Feedback Format | FBCH | Number of Reports | Report period | Feedback Fields | Description/Notes |
| --- | --- | --- | --- | --- | --- |
| 0 (M =1) | SFBCH | 1 | Long | Best_subbands_index Subband CQI Stream Index PFBCH indicator |  |
| 1 (M=2) | SFBCH | 2 | Short | Subband avg_CQI for (m=1:M){ Subband differential CQI Stream Index } | No short report when q=0 |
|  |  |  | Long | Best_subbands_index PFBCH indicator |  |
| 2 (M =3) | SFBCH | 2 | Short | Subband avg_CQI for (m=1:M){ Subband differential CQI Stream Index } | No short report when q=0 |
|  |  |  | Long | Best_subbands_index PFBCH indicator |  |
| 3 (M =5) | SFBCH | 3 | Short | Subband avg_CQI for (m=1:M){ Subband differential CQI Stream Index } | No short report when q=0 |
|  |  |  | Long | Best_subbands_index PFBCH indicator |  |

# Feedback format for MFM 6

The detailed format is listed in Table 894. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report according to long term feedback period in feedback allocation A-MAP IE. The long period report shall start by puncturing the first short period report. For feedback format 0 with 3 reports using the PFBCH, Subband PMI shall be transmitted first after every report of Best_subbands_index. Subband CQI and subband PMI are then transmitted alternately and Best_subbands_index is transmitted in the long term period. If the PFBCH indicator in SFBCH is set to '1', PFBCH is transmitted using encoding type 0 by puncturing SFBCH in the next feedback opportunity regardless of short-term and long-term feedback. When q=0, only the long period reports shall be sent. q=0 shall not be set with the differential feedback mode.

The CQI of subband m shall be computed as follows: (Subband m CQI index) = (Subband avg CQI index) + (Subband m differential CQI), using the latest available reports of Subband avg CQI and Subband differential CQI. Subband avg CQI index is an average measure of the CQI over the M reported subbands. The possible differential CQI values are {-1, 0, +1, +2}. The AMS shall ensure that the reported differential CQI will produce a value of subband m CQI in the range of 0 to 15.

If the ABS has 4 transmit antennas and assigns MFM6 with feedback format 3, then it shall set Codebook_subset = 0b1 except if Codebook_mode indicates the differential codebook-based feedback mode.

Table 909 - Feedback formats for MFM 6

| Feedback Format | FBCH | Number of reports | Report period | Feedback Fields | Description/Notes |
| --- | --- | --- | --- | --- | --- |
| 0 (M = 1) | PFBCH | 3 | Short | Subband CQI | PFBCH encoding Type 0 No short report when q=0 |
|  |  |  | Short | Subband PMI | PFBCH encoding Type 2 No short report when q=0 |
|  |  |  | Long | Best_subbands_index | PFBCH encoding Type 1 |
| 1 (M = 2) | SFBCH | 2 | Short | for (m=1:M){ Subband differential CQI Subband PMI } | No short report when q=0 |
|  |  |  | Long | Best_subbands_index Subband avg CQI PFBCH indicator |  |
| 2 (M = 3) | SFBCH | 2 | Short | for (m=1:M){ Subband differential CQI Subband PMI } | No short report when q=0 |
|  |  |  | Long | Best_subbands_index Subband avg CQI PFBCH indicator |  |
| 3 (M = 4) | SFBCH | 2 | Short | for (m=1:M){ Subband differential CQI Subband PMI } | No short report when q=0 |
|  |  |  | Long | Best_subbands_index Subband avg CQI PFBCH indicator for (m=1:M){ Subband differential CQI Subband PMI } |  |

The detailed format for MIMO feedback mode 6 with differential codebook is in Table 910. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE. For M>1, two long term reports will puncture two short term reports continuously according to long term feedback period in feedback allocation A-MAP IE. For M=1, the long term report will puncture one short term report according to long term feedback period in feedback allocation A-MAP IE.

Table 910 - Feedback formats for MFM 6 for differential codebook

| Feedback Format | FBCH | Number of Reports | Feedback Fields |  | Description/Notes |
| --- | --- | --- | --- | --- | --- |
| 0 (M =1) | SFBCH | 2 | Short | Subband CQI Differential PMI Padding | Padding 1 bit: If Nt=2 0 bit: Otherwise |
|  |  |  | Long | Subband index Base PMI PFBCH indicator | PMI for reset. Base PMI is a PMI from the base codebook or the base codebook subset. |
| 1 (M=2) | SFBCH | 3 | Short | for (m=1:M){ Subband differential CQI Subband Differential PMI } |  |
|  |  |  | Long | Best_subbands_index subband avg_CQI PFBCH indicator |  |
|  |  |  | Long | for (m=1:M){ Base PMI } | PMI for reset. Base PMI is a PMI from the base codebook or the base codebook subset. |
| 2 (M =3) | SFBCH | 3 | Short | for (m=1:M){ Subband differential CQI Subband Differential PMI } |  |
|  |  |  | Long | Best_subbands_index subband avg_CQI PFBCH indicator |  |
|  |  |  | Long | for (m=1:M){ Base PMI } | PMI for reset. Base PMI is a PMI from the base codebook or the base codebook subset. |
| 3 (M =4) | SFBCH | 3 | Short | for (m=1:M){ Subband differential CQI Subband Differential PMI } |  |
|  |  |  | Long | Best_subbands_index subband avg_CQI PFBCH indicator |  |
|  |  |  | Long | for (m=1:M){ Base PMI } | PMI for reset. Base PMI is a PMI from the base codebook or the base codebook subset. |

------------------------------------------- End of the Proposed Amendment Text --------------------------------------------