| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed modifications to the MIMO feedback extended header (16.2.2) |  |
| Date Submitted | 2009-12-30 |  |
| Source(s) | David Mazzarese, Sangheon Kim, Hokyu Choi, Heewon Kang Samsung Electronics | d.mazzarese@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | P802.16m/D3 LB30b |  |
| Abstract | This contribution proposes modifications to the MIMO feedback extended header |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 3) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed modifications to the MIMO feedback extended header (16.2.2)

David Mazzarese, Hokyu Choi
Samsung Electronics Co., Ltd.

# Introduction

This contribution proposes modifications to the MIMO feedback extended header section 16.2.2.2.7, as accepted by comment A0169 with contribution C80216m-09/2497r1 in November 2009. Additionally, we enable the feedback of wideband STC rate for single-user wideband beamforming in NLRU.

- References

[1] DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D3, Dec 2009.

# Text proposal for inclusion in the 802.16m amendment

<< Modify section “16.2.2.2.7 Correlation matrix feedback extended header (MFEH)” on page 35 line 33 >>

-------------------------------  Start of Text Proposal  ---------------------------------------------------

16.2.2.2.7 MIMO feedback extended header (MFEH)

This MIMO feedback extended header format is defined in Table 670. This header is used by AMS as a response to a Feedback Polling A-MAP IE requesting only the wideband information for MFM 0, 1, 4 or 7, or only the subband information for the best subband for MFM 2, 3, 5 or 6. Variables MaxMt, Codebook_subset, long period q and Measurement Method Indication are indicated in the Feedback Polling A-MAP IE. Nt is the number of transmit antennas at the ABS, indicated in S-SFH SP3. The number of subbands YSB is described in section 16.3.9.3.1.5.

Table 670 –MFEH Format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| MFEH() { |  |  |
| Type | 4 | MFEH type |
| MFM | 3 |  |
| If (MFM == 0){ |  |  |
| Wideband CQI | 4 |  |
| Wideband STC rate | variable | If Measurement Method Indication=0b0: MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits If Measurement Method Indication=0b1: 1 bit This field shall be omitted if MaxMt =1. |
| } |  |  |
| If (MFM == 1){ |  |  |
| Wideband CQI | 4 |  |
| Wideband STC rate | variable | If Measurement Method Indication=0b0: MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits This field shall be omitted if Measurement Method Indication=0b1 or if MaxMt =1. |
| } |  |  |
| If (MFM == 4){ |  |  |
| Wideband CQI | 4 |  |
| Wideband STC rate | variable | MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits This field shall be omitted if MaxMt =1. |
| Wideband PMI | variable | Nt =2: 3 bits Nt =4 and Codebook_subset =0b0: 6 bits Nt =4 and Codebook_subset =0b1: 4 bits Nt =8: 4 bits |
| } |  |  |
| If (MFM == 7){ |  |  |
| Wideband CQI | 4 |  |
| Wideband PMI | variable | Nt =2: 3 bits Nt =4 and Codebook_subset =0b0: 6 bits Nt =4 and Codebook_subset =0b1: 4 bits Nt =8: 4 bits |
| } |  |  |
| If (MFM == 2){ |  |  |
| Subband index | variable | Ceil(log2(YSB)) bits used for binary representation of logical subband index |
| Subband CQI | 4 |  |
| Subband STC rate | variable | If Measurement Method Indication=0b0: MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits MaxMt > 4: 3 bits This field shall be omitted if Measurement Method Indication=0b1 or if MaxMt =1. |
| } |  |  |
| If (MFM == 3){ |  |  |
| Subband index | variable | Ceil(log2(YSB)) bits used for binary representation of logical subband index |
| Subband CQI | 4 |  |
| Subband STC rate | variable | MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits MaxMt > 4: 3 bits This field shall be omitted if MaxMt =1. |
| Subband PMI | variable | Nt = 2: 3 bits Nt = 4 and Codebook_subset = 0b0: 6 bits Nt = 4 and Codebook_subset = 0b1: 4 bits Nt = 8: 4 bits |
| } |  |  |
| If (MFM == 5){ |  |  |
| Subband index | variable | Ceil(log2(YSB)) bits used for binary representation of logical subband index |
| Subband CQI | 4 |  |
| Subband stream index | variable | If Measurement Method Indication=0b0: MaxMt = 2: 1 bit MaxMt = 3 or 4: 2 bits If Measurement Method Indication=0b1: 1 bit |
| } |  |  |
| If (MFM == 6){ |  |  |
| Subband index | variable | Ceil(log2(YSB)) bits used for binary representation of logical subband index |
| Subband CQI | 4 |  |
| Subband PMI | variable | Nt =2: 3 bits Nt =4 and Codebook_subset =0b0: 6 bits Nt =4 and Codebook_subset =0b1: 4 bits Nt =8: 4 bits |
| } |  |  |
| } |  |  |

Table 670 –MFEH Format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| MFEH() { |  |  |
| Type | 4 | MFEH type |
| MFM | 3 |  |
| If (MFM == 0 or 1){ |  |  |
| Wideband CQI | 4 |  |
| Wideband STC rate | 1 or 2 | Depending on MaxMt |
| } |  |  |
| If (MFM == 4 or 7){ |  |  |
| Wideband CQI | 4 |  |
| Wideband PMI | 3 or 6 | Depending on Nt and CS |
| } |  |  |
| If (MFM == 2){ |  |  |
| Subband index | 3 to 5 | Depending on bandwidth |
| Subband CQI | 4 |  |
| Subband STC rate | 1 to 3 | Depending on MaxMt |
| } |  |  |
| If (MFM == 3){ |  |  |
| Subband index | 3 to 5 | Depending on bandwidth |
| Subband CQI | 4 |  |
| Subband STC rate | 1 to 3 | Depending on MaxMt |
| Subband PMI | 3 or 6 | Depending on Nt and CS |
| } |  |  |
| If (MFM == 5){ |  |  |
| Subband index | 3 to 5 | Depending on bandwidth |
| Subband CQI | 4 |  |
| Subband stream index | 1 to 3 | Depending on MaxMt |
| } |  |  |
| If (MFM == 6){ |  |  |
| Subband index | 3 to 5 | Depending on bandwidth |
| Subband CQI | 4 |  |
| Subband PMI | 3 or 6 | Depending on Nt and CS |
| } |  |  |
| } |  |  |

-------------------------------  End of Text Proposal  ---------------------------------------------------