| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed modifications to the MIMO feedback extended header (16.2.2) |  |
| Date Submitted | 2009-12-30 |  |
| Source(s) | David Mazzarese, Sangheon Kim, Hokyu Choi, Heewon Kang Samsung Electronics | d.mazzarese@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | P802.16m/D3 LB30b |  |
| Abstract | This contribution proposes modifications to the MIMO feedback extended header |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 3) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed modifications to the MIMO feedback extended header (16.2.2)

David Mazzarese, Hokyu Choi
Samsung Electronics Co., Ltd.

# Introduction

This contribution proposes modifications to the MIMO feedback extended header section 16.2.2.2.7, as accepted by comment A0169 with contribution C80216m-09/2497r1 in November 2009. Additionally, we enable the feedback of wideband STC rate for single-user wideband beamforming in NLRU with MFM4, and we remove the feedback of STC rate for single-user rank-1 open-loop precoding in NLRU outside OL Region. Finally, we fix the MFEH size to 3 bytes including padding, to achieve a burst size of 6 bytes for the AGMH+MFEH as a stand-alone packet.

- References

[1] DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D3, Dec 2009.

# Text proposal for inclusion in the 802.16m amendment

<< Modify section “16.2.2.2.7 Correlation matrix feedback extended header (MFEH)” on page 35 line 33 >>

-------------------------------  Start of Text Proposal  ---------------------------------------------------

16.2.2.2.7 MIMO feedback extended header (MFEH)

This MIMO feedback extended header format is defined in Table 670. This header is used by AMS as a response to a Feedback Polling A-MAP IE requesting only the wideband information for MFM 0, 1, 4 or 7, or only the subband information for the best subband for MFM 2, 3, 5 or 6. Variables MaxMt, Codebook_subset, long period q and Measurement Method Indication are indicated in the Feedback Polling A-MAP IE. Nt is the number of transmit antennas at the ABS, indicated in S-SFH SP3. The number of subbands YSB is described in section 16.3.9.3.1.5. The MFEH shall be padded by zeros for byte alignment to 3 bytes.

The decimal value of wideband STC rate is encoded with 2 or 3 bits in binary representation, but its allowed values depend on Measurement Method Indication and MaxMt. The decimal value of subband and wideband PMI is encoded with 4 or 6 bits in binary representation, but its allowed values depend on Nt and Codebook_subset. The decimal value of subband stream index is always encoded with 2 bits, but its allowed values depend on Measurement Method Indication and MaxMt. The decimal value of logical subband index is always encoded with 5 bits, but its allowed values are limited to the interval [0, YSB-1].

Table 670 –MFEH Format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| MFEH() { |  |  |
| Type | 4 | MFEH type |
| MFM | 3 |  |
| If (MFM == 0){ |  |  |
| Wideband CQI | 4 |  |
| Wideband STC rate | 3 |  |
| Zero padding | 10 |  |
| } |  |  |
| If (MFM == 1){ |  |  |
| Wideband CQI | 4 |  |
| Zero padding | 13 |  |
| } |  |  |
| If (MFM == 2){ |  |  |
| Subband index | 5 |  |
| Subband CQI | 4 |  |
| Subband STC rate | 3 |  |
| Zero padding | 5 |  |
| } |  |  |
| If (MFM == 3 and Nt == 2 or 4){ |  |  |
| Subband index | 5 |  |
| Subband CQI | 4 |  |
| Subband STC rate | 2 |  |
| Subband PMI | 6 |  |
| } |  |  |
| If (MFM == 3 and Nt == 8){ |  |  |
| Subband index | 5 |  |
| Subband CQI | 4 |  |
| Subband STC rate | 3 |  |
| Subband PMI | 4 |  |
| Zero padding | 1 |  |
| } |  |  |
| If (MFM == 4){ |  |  |
| Wideband CQI | 4 |  |
| Wideband STC rate | 3 |  |
| Wideband PMI | 6 |  |
| Zero padding | 4 |  |
| } |  |  |
| If (MFM == 5){ |  |  |
| Subband index | 5 |  |
| Subband CQI | 4 |  |
| Subband stream index | 2 |  |
| Zero padding | 6 |  |
| } |  |  |
| If (MFM == 6){ |  |  |
| Subband index | 5 |  |
| Subband CQI | 4 |  |
| Subband PMI | 6 |  |
| Zero padding | 2 |  |
| } |  |  |
| If (MFM == 7){ |  |  |
| Wideband CQI | 4 |  |
| Wideband PMI | 6 |  |
| Zero padding | 7 |  |
| } |  |  |
| } |  |  |

Table 670 –MFEH Format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| MFEH() { |  |  |
| Type | 4 | MFEH type |
| MFM | 3 |  |
| If (MFM == 0 or 1){ |  |  |
| Wideband CQI | 4 |  |
| Wideband STC rate | 1 or 2 | Depending on MaxMt |
| } |  |  |
| If (MFM == 4 or 7){ |  |  |
| Wideband CQI | 4 |  |
| Wideband PMI | 3 or 6 | Depending on Nt and CS |
| } |  |  |
| If (MFM == 2){ |  |  |
| Subband index | 3 to 5 | Depending on bandwidth |
| Subband CQI | 4 |  |
| Subband STC rate | 1 to 3 | Depending on MaxMt |
| } |  |  |
| If (MFM == 3){ |  |  |
| Subband index | 3 to 5 | Depending on bandwidth |
| Subband CQI | 4 |  |
| Subband STC rate | 1 to 3 | Depending on MaxMt |
| Subband PMI | 3 or 6 | Depending on Nt and CS |
| } |  |  |
| If (MFM == 5){ |  |  |
| Subband index | 3 to 5 | Depending on bandwidth |
| Subband CQI | 4 |  |
| Subband stream index | 1 to 3 | Depending on MaxMt |
| } |  |  |
| If (MFM == 6){ |  |  |
| Subband index | 3 to 5 | Depending on bandwidth |
| Subband CQI | 4 |  |
| Subband PMI | 3 or 6 | Depending on Nt and CS |
| } |  |  |
| } |  |  |

-------------------------------  End of Text Proposal  ---------------------------------------------------