| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to the feedback polling A-MAP IE (16.3.6) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | David Mazzarese, Hokyu Choi Samsung Electronics | d.mazzarese@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | P802.16m/D2 LB30b |  |
| Abstract | Modifications are proposed for the feedback polling A-MAP IE for allocation and deallocation procedure |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 3) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed changes to the feedback polling A-MAP IE (16.3.6)

David Mazzarese, Hokyu Choi

Samsung Electronics Co., Ltd.

# Introduction

Modifications are proposed for the feedback polling A-MAP IE in order to efficiently allocate and de-allocate feedback. The polling IE allocates some periodic feedback with or without a dedicated uplink resource. A dedicated uplink resource can be effectively de-allocated by indicating an allocation duration d=0 along with the resource index. But there is no procedure to de-allocate feedback that was not allocated with a dedicated uplink resource. When the allocation duration d=0 is specified without a resource index, the AMS does not know how to interpret the de-allocation. One interpretation could be that all the previously assigned feedback is de-allocated by the ABS. But this is inefficient, since the ABS would have to re-allocate part of the feedback that it did not wish to de-allocate.

For solving this problem, we propose to include a polling_allocation_index which uniquely identifies feedback allocations and de-allocation that do not use a dedicated uplink resource. 3 bits are reserved for this index, with a constraint to allow up to 3 concurrent feedback allocations without dedicated uplink resources. This feature can be used, for example, to concurrently allocate feedback for the transmit correlation matrix, for the wideband CQI feedback for MFM 0, and for subband feedback for MFM 6, enabling operation with the codebook transformation mode in subband operation while still providing robust wideband CQI feedback for DLRU allocations.

The proposed de-allocation procedure allows to de-allocate a single, two or all allocations with a single indication. De-allocation with the allocation of a currently used index is also allowed, conditioned on the allocation of an HARQ feedback channel for confirmation by the AMS. This last feature avoids sending a de-allocation IE and then an allocation IE, especially for cases where all indexes are used and the ABS wishes to reuse one of them for another purpose.

Additionally, a long TTI allocation should also be feasible to extend the coverage of MIMO feedback in MAC message or extended header. A long period should be defined along with the short period to allow for the joint allocation of the feedback for the correlation matrix and the PMI and CQI. The TBD for the parameter CM is resolved by defining a rule according to the long period (q) and its usage is clarified.

- References

[1] DRAFT amendment to IEEE standard for local and metropolitan area networks, P80216m/D2, Oct 2009.

# Text proposal for inclusion in the 802.16m amendment

<Modify section “16.3.6.5.2.4.11 Feedback Polling A-MAP IE” on line 1 page 437 as follows>

-------------------------------  Start of Text Proposal ---------------------------------------------------

16.3.6.5.2.4.11 Feedback Polling A-MAP IE

The information element shown in Table 827 is used by the ABS to schedule MIMO feedback transmission by the AMS. The AMS sends the MIMO feedback using a MAC control message or an extended header, depending on the requested feedback content.

When the Dedicated UL Allocation bit is set to 0b1, a dedicated UL allocation shall be included in this IE. The dedicated UL allocation shall be used by the AMS to transmit feedback at the designated feedback transmission frame defined by this IE. When the Dedicated UL Allocation bit is set to 0b0, no dedicated UL allocation shall be included. Instead, at the designated transmission frame defined by this IE, the AMS shall compose the feedback and the ABS shall include a UL allocation for the transmission using UL Basic Assignment A-MAP IE or UL Subband Assignment A-MAP IE.

The MS shall send the feedback in an extended header when the following feedback information is requested:

- wideband information for MIMO feedback modes 0, 1, 4 and 7

- subband information for 1 subband for MIMO feedback modes 2, 3, 5 and 6

The MS shall send the feedback in a MAC control message when the following feedback information is requested:

- subband information for more than one subband for MIMO feedback modes 2, 3, 5 and 6

- multi-BS feedback

The coefficients of the quantized transmit correlation matrix should be feedback in Correlation Matrix Feedback Extended Header (CMFEH) when no AAI_SingleBS_MIMO_FBK message is sent in the same packet if the ABS is equipped with 2 or 4 transmit antennas. Otherwise, the coefficients of the quantized transmit correlation matrix should be feedback in AAI_SingleBS_MIMO_FBK message. The coefficients of the quantized transmit correlation matrix shall be feedback in AAI_SingleBS_MIMO_FBK message if the ABS is equipped with 8 transmit antennas.

In case of feedback for MIMO feedback modes 0, 1, 4 or 7, the MS shall feedback the CQI for FP0 if FPCT equals 1 or 4FPS0>0, or for FPk if FPCT equals 2 or 3FPS0=0, where FPk is determined by k = floor(IDCell/256) )+1 for FPCT=3 or k = floor(IDcell/384)+1 for FPCT=2.

Table 827 - Feedback Polling A-MAP IE

| Syntax | Size in bits | Notes |
| --- | --- | --- |
| A-MAP IE Type { | 4 | Feedback_Polling_IE |
| Allocation Duration (d) | 3 | The allocation is valid for 2(d–1) superframes starting from the superframe defined by allocation relevance. If d == 0b000, the pre-scheduled feedback header transmission is released. If d == 0b111, the pre-scheduled feedback header transmission shall be valid until the BS commands to release it. |
| If (d ==0b000){ |  | Feedback de-allocation |
| Dedicated UL allocation | 1 | 0b0: the feedback identified by Polling Allocation Index is de-allocated. 0b1: the feedback identified by a dedicated UL resource is de-allocated. |
| If (Dedicated UL allocation ==0b1){ |  |  |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| } else { |  |  |
| Polling_deallocation_index | 3 | Unique index referring to feedback allocations previously assigned with a Feedback Polling A-MAP IE. 0b000: all feedback previously assigned by Feedback Polling A-MAP IE is de-allocated, including all allocations with dedicated uplink resources 0b001: feedback assigned with index 0b01 is de-allocated 0b010: feedback assigned with index 0b10 is de-allocated 0b011: feedback assigned with index 0b11 is de-allocated 0b100: feedback assigned with indexes 0b01 and 0b10 is de-allocated 0b101: feedback assigned with indexes 0b01 and 0b11 is de-allocated 0b110: feedback assigned with indexes 0b10 and 0b11 is de-allocated 0b111: feedback assigned with indexes 0b01, 0b10, 0b11 is de-allocated |
| } |  |  |
| HFA | 3 | HARQ feedback channel allocation for Feedback Channel De-allocation confirmation HARQ feedback channel allocation to acknowledge the successful detection of this IE. |
| } else if (d != 0b000){ |  | Feedback allocation |
| Dedicated UL allocation | 1 | 0b0: No dedicated UL resource is allocated. BS shall provide UL allocation for the MIMO feedback IE transmission through UL A-MAP IE at each designated transmitting frame defined by this IE. 0b1: Dedicated UL resource is included in this IE |
| If (Dedicated UL allocation ==0b1){ |  |  |
| ISizeOffset | 5 | Offset used to compute burst size index |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| MEF | 1 | MIMO encoder format for uplink feedback transmission Non-adaptive precoding shall be used at the AMS. 0b0: SFBC 0b1: VE with Mt = 2 if Nt > 1, or VE with Mt = 1 if Nt = 1 Nt is the number of transmit antennas at the AMS. |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: AAI subframe (default) 0b1: 4 UL AAI subframes for FDD or all UL AAI subframes for TDD If number of DL AAI subframes, D, is less than number of UL AAI subframes, U, Long TTI Indicator = 0b1 |
| } else { |  |  |
| ACK Allocation Flag | 1 |  |
| If (ACK Allocation Flag == 0b1){ |  |  |
| HFA | 3 | HARQ feedback channel allocation to acknowledge the successful detection of this IE. |
| } |  |  |
| Polling_allocation_index | 2 | Unique index assigned to this feedback allocation. Allowed values are 0b01, 0b10 and 0b11. This newly assigned Polling_Allocation_Index shall be different than a currently assigned Polling_Allocation_Index if ACK Allocation Flag == 0b0. |
| } |  |  |
| Period (p) | 3 | Transmit feedback header every 4p frame. The first report shall start at the next frame. |
| MIMO_feedback_IE_type | 1 | - 0b0: feedback for single-BS MIMO operation - 0b1: feedback for multi-BS MIMO operation |
| If (MIMO_feedback_IE_type == 0b0){ |  | - Single-BS MIMO feedback request |
| Transmit_Correlation_Matrix | 1 | - 0b0: feedback of the transmit correlation matrix is indicated by CM - 0b1: feedback of the quantized BS transmit correlation matrix only - Transmit correlation matrix shall be feedback if CM = 0b1 |
| - MFM | - 3 | - MIMO Feedback Mode for which the AMS shall transmit feedback. |
| Period | 4 | 16 combination of short feedback period p (frames) and long feedback period q (superframes). The first report shall start at the next frame with the short period feedback. The long period feedback shall also be included in the first report if it is assigned. 0b0000: p = 1, q = 0 0b0001: p = 2, q = 0 0b0010: p = 4, q = 0 0b0011: p = 1, q = 1 0b0100: p = 2, q = 1 0b0101: p = 1, q = 2 0b0110: p = 2, q = 2 0b0111: p = 4, q = 2 0b1000: p = 1, q = 4 0b1001: p = 2, q = 4 0b1010: p = 4, q = 4 0b1011: p = 0, q = 1 0b1100: p = 0, q = 2 0b1101: p = 0, q = 4 0b1110: reserved 0b1111: reserved |
| If (Transmit_Correlation_Matrix == 0b0){ |  | ABS requests AMS to feedback CQI And CSI for a specific MFM. |
| MaxMt | Variable 1 or 2 | Variable number of bits - depends on number of transmit antenna Nt Nt at the ABS - If Nt=2 (Any MFM): - 0b0: 1 - 0b1: 2 - If Nt =4 (Any MFM): - 0b00: 1 - 0b01: 2 - 0b10: 3 - 0b11: 4 - If Nt =8 (SU-MIMO MFM 0, 1, 2, 3, 4): - 0b00: 1 - 0b01: 2 - 0b10: 4 - 0b11: 8 - If Nt =8 (MU-MIMO MFM 5, 6, 7): - 0b00: 1 - 0b01: 2 - 0b10: 3 - 0b11: 4 |
| - MFM | - 3 | - MIMO Feedback Mode for which the AMS shall transmit feedback. |
| If (MFM = 2, 3, 5, 6) { |  | - Feedback of CQI and CSI for localized resource units |
| Num_best_subbands | 2 | 0b00: report all subbands 0b01: 1 best subband 0b10: min{6, YSB} best subbands 0b11: min{12, YSB} best subbands 1< Num_best_subbands ≤ YSB |
| } |  |  |
| If (MFM == 3,4,6,7) { |  | Feedback of CQI and CSI for CL SU and MU MIMO |
| CM Codebook_mode | [TBD] 1 | [TBD] Codebook Feedback Mode and Codebook Coordination. This field shall be ignored if long period q > 0. 0b0: base mode with codebook coordination disabled 0b1: base mode with codebook coordination enabled |
| CS Codebook_subset | 1 | Codebook_subset shall be ignored if Nt ≠ 4. 0b0: report PMI from the base codebook or transformed base codebook 0b1: report PMI from the codebook subset or transformed codebook subset |
| } |  |  |
| If(MFM=0, 1, 2, 5){ |  | Indication of report for OL region |
| Measurement Method Indication | 1 | 0b0: Use the midamble for CQI measurements 0b1: Use pilots in OL region with MaxMt streams for CQI measurements |
| } |  |  |
| } |  |  |
| } |  |  |
| Else { |  | - Multi-BS MIMO feedback request |
| Period (p) | 3 | Transmit feedback header every 4×p frames. - The first report shall start at the next frame. |
| TRU | 2 | - Target RU indicating which RUs or which type of RU to work on for feedback 0b00: Latest best subbands reported for single BS MIMO 0b01: Whole bandwidth 0b10: FFR partition 0 0b11: boosted FFR partition |
| ICT | 2 | 0b00: PMI restriction for single-BS precoding; 0b01: PMI recommendation for single-BS precoding; 0b10: CL-MD for multi-BS precoding; 0b11: Co-MIMO for multi-BS precoding; |
| CS Codebook_subset | 1 | Codebook_subset shall be ignored if Nt ≠ 4. 0b0: report PMI from the base codebook 0b1: report PMI from the codebook subset |
| N_multiBS_reports | 3 | N_multiBS_reports indicates the number of reports. |
| If (ICT = 0b11) { |  |  |
| MaxUser | 2 | Maximum number of users supported in Co-MIMO in the same resource. 0b00: 2 users 0b01: 3 users 0b10: 4 users 0b11: reserved |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| Padding | variable | Padding to reach byte boundary |

ACK Allocation Flag: BS may set ACK Allocation Flag to 0b1 if Allocation Duration is d = 0b000. BS may set ACK Allocation Flag to 0b1 if Allocation Duration is not d = 0b000 and the resource index (resp. Polling_allocation_index) of the newly allocated FBCH is the same as the resource index (resp. Polling_allocation_index) of the deallocated feedback.

Polling_allocation_index: This field assigns a unique identifier associated with a feedback allocation, only if that allocation does not have a dedicated uplink resource index. Two concurrently assigned feedback allocations shall not have the same identifier. An AMS supports a maximum of 3 distinct concurrent feedback allocations without a dedicated uplink resource, including correlation matrix feedback extended header (CMFEH), MIMO feedback extended header (MFEH), AAI_SingleBS_MIMO_FBK and AAI_MultiBS_MIMO_FBK. The Polling_allocation_index shall not be set to a currently assigned Polling_allocation_index if ACK Allocation Flag == 0b0. If ACK Allocation Flag == 0b1 and the Polling_allocation_index is set to a currently assigned Polling_allocation_index, then the currently assigned feedback is de-allocated and replaced by the newly assigned feedback with the same Polling_allocation_index.

Polling_deallocation_index: When the Allocation Duration (d) is set to 0b000, this field is used to de-allocate one or more specific feedback allocations.

Period: The setting of short and long periods p and q shall obey the rules below when MIMO_feedback_IE_type == 0b0.

For MFM 0 or 1: when p = 0, q > 0, the wideband CQI and wideband STC rate shall be transmitted using long feedback period. When p > 0, q = 0, the wideband CQI and wideband STC rate shall be transmitted using short feedback period. p > 0 and q > 0 shall not be set at the same time with MFM 0 or 1.

For MFM 2 or 5: when p = 0, q > 0, the Best_subbands_index, subband CQI and Stream index shall be transmitted using long feedback period. When p > 0, q = 0, the Best_subbands_index, subband CQI and Stream index shall be transmitted using short feedback period. p > 0 and q > 0 shall not be set at the same time with MFM 2 or 5.

For MFM 3: when p > 0 and q = 0, the Best_subbands_index, STC_Rate, subband CQI and subband PMI from the base codebook shall be transmitted using short period. When p = 0 and q > 0, only the correlation matrix shall be transmitted using long feedback period. p > 0 and q > 0 shall not be set at the same time with MFM 3.

For MFM 6: when p > 0 and q > 0, the Best_subbands_index, STC_Rate, subband CQI and subband PMI from the transformed codebook shall be transmitted using short period, while the correlation matrix shall be transmitted using long period. When p = 0 and q > 0, only the correlation matrix shall be transmitted using long feedback period. When p > 0 and q = 0, the Best_subbands_index, STC_Rate, subband CQI and subband PMI from the base codebook shall be transmitted using short period.

For MFM 4: when p > 0 and q = 0, the wideband CQI, STC rate and PMI from the base codebook shall be transmitted using short feedback period. When p > 0 and q > 0, the wideband CQI and STC rate shall be transmitted using short feedback period, and the correlation matrix shall be transmitted using long feedback period. When p = 0 and q > 0, only the correlation matrix shall be transmitted using long feedback period.

For MFM 7: when p > 0 and q = 0, the wideband CQI and PMI from the base codebook shall be transmitted using short feedback period. When p > 0 and q > 0, the wideband CQI shall be transmitted using short feedback period, and the correlation matrix shall be transmitted using long feedback period. When p = 0 and q > 0, only the correlation matrix shall be transmitted using long feedback period.

-------------------------------  End of Text Proposal  ---------------------------------------------------