| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to the feedback polling A-MAP IE (16.3.6) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | David Mazzarese, Sangheon Kim, Hokyu Choi, Heewon Kang Samsung Electronics Jisoo Park, Namsuk Lee, Sookjin Lee, Yongseouk Choi, Nakwoon Sung ETRI | d.mazzarese@samsung.com jsp@etri.re.kr *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | P802.16m/D2 LB30b |  |
| Abstract | Modifications are proposed for the feedback polling A-MAP IE for allocation and deallocation procedure |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 3) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed changes to the feedback polling A-MAP IE (16.3.6)

David Mazzarese, Hokyu Choi

Samsung Electronics Co., Ltd.

# Introduction

Several changes are proposed for the Feedback Polling A-MAP IE:

- Correction of HFA field to 6 bits when there is no available 11-bit resource index

- Addition of ACID when the allocation indicated an explicit uplink resource index

- Retransmission rules for MIMO feedback in extended header or MAC message are specified

Due to the increased size of bit fields, the IE is split into 2 branches:

- With resource index, for uplink resource allocation or deallocation

  - Additionally, the feedback of the transmit correlation matrix and/or one MIMO feedback mode can be jointly allocated.

  - Additionally, the de-allocation of currently assigned MIMO Feedback Modes can be performed jointly with the de-allocation of an existing dedicated uplink resource.

  - UL HARQ feedback allocation is enabled to acknowledge the correct detection of the IE in case of de-allocation.

  - Long TTI allocation is enabled to extend the coverage of the MIMO feedback in extended header or MAC control message.

  - Uplink collaborative spatial multiplexing is enabled as instructed by the ABS

- Without resource index, for feedback content allocation of any MIMO feedback mode

  - Additionally, allocation of multiple MIMO feedback modes with a single IE is enabled by the usage of a MFM bitmap.

  - Additionally, de- allocation of multiple MIMO feedback modes with a single IE is enabled by a Polling_deallocation_bitmap.

  - Joint allocation of two feedback period (short and long) is enabled. When the long feedback period is present, the feedback of the transmit correlation matrix is assigned, and it automatically triggers the feedback of PMI from the transformed codebook with MFM3 or MFM6. This resolves the TBD of the parameter CM.

- References

[1] DRAFT amendment to IEEE standard for local and metropolitan area networks, P80216m/D2, Oct 2009.

# Text proposal for inclusion in the 802.16m amendment

<Modify section “16.3.6.5.2.4.11 Feedback Polling A-MAP IE” on line 1 page 437 as follows>

-------------------------------  Start of Text Proposal ---------------------------------------------------

16.3.6.5.2.4.11 Feedback Polling A-MAP IE

The information element shown in Table 827 is used by the ABS to schedule MIMO feedback transmission by the AMS. The AMS sends the MIMO feedback using a MAC control message or an extended header, depending on the requested feedback content.

When the Dedicated UL Allocation Polling_sub_type bit is set to 0b1 0b0, a dedicated UL allocation shall be is included in this IE. The dedicated UL allocation shall be used by the AMS to transmit feedback at the designated feedback transmission frame defined by this IE. When the Dedicated UL Allocation Polling_sub_type bit is set to 0b0 0b1, no dedicated UL allocation shall be is included. Instead, at the designated transmission frame defined by this IE, the AMS shall compose the feedback, and the ABS shall either include a UL allocation for the transmission using UL Basic Assignment A-MAP IE or UL Subband Assignment A-MAP IE, or the AMS shall transmit in a dedicated UL allocation assigned by a previous Feedback Polling A-MAP IE with feedback periods designating the same transmission frames.

The AMS shall send the feedback in an extended header when the following feedback information is requested:

- wideband information for MIMO feedback modes 0, 1, 4 and 7

- subband information for 1 subband for MIMO feedback modes 2, 3, 5 and 6

The AMS shall send the feedback in a MAC control message when the following feedback information is requested:

- subband information for more than one subband for MIMO feedback modes 2, 3, 5 and 6

- multi-BS feedback

The coefficients of the quantized transmit correlation matrix should be feedback in Correlation Matrix Feedback Extended Header (CMFEH) when no AAI_SingleBS_MIMO_FBK message is sent in the same packet if the ABS is equipped with 2 or 4 transmit antennas. Otherwise, the coefficients of the quantized transmit correlation matrix should be feedback in AAI_SingleBS_MIMO_FBK message. The coefficients of the quantized transmit correlation matrix shall be feedback in AAI_SingleBS_MIMO_FBK message if the ABS is equipped with 8 transmit antennas.

In case of feedback for MIMO feedback modes 0, 1 with Measurement Method Indication = 0b0, and MIMO feedback modes 4 or 7, the AMS shall feedback the CQI for FP0 if FPCT equals 1 or 4 FPS0>0, or for FPk if FPCT equals 2 or 3 FPS0=0, where FPk is determined by k = floor(IDCell/256) )+1 for FPCT=3 or k = floor(IDcell/384)+1 for FPCT=2.

Table 827 - Feedback Polling A-MAP IE

| Syntax | Size in bits | Notes |
| --- | --- | --- |
| Feedback_Polling_A-MAP_IE(){ |  |  |
| A-MAP IE Type | 4 | Feedback_Polling_IE |
| Allocation Duration (d) | 3 | The allocation is valid for 2(d–1) superframes starting from the superframe defined by allocation relevance. If d == 0b000, the pre-scheduled feedback header transmission is released. If d == 0b111, the pre-scheduled feedback header transmission shall be valid until the BS commands to release it. |
| If (d ==0b000){ |  | Feedback de-allocation |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| HFA | 3 | HARQ feedback channel allocation for Feedback Channel De-allocation confirmation |
| } else if (d != 0b000){ |  | Feedback allocation |
| Dedicated UL allocation | 1 | 0b0: No dedicated UL resource is allocated. BS shall provide UL allocation for the MIMO feedback IE transmission through UL A-MAP IE at each designated transmitting frame defined by this IE. 0b1: Dedicated UL resource is included in this IE |
| If (Dedicated UL allocation ==0b1){ |  |  |
| ISizeOffset | 5 | Offset used to compute burst size index |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| MEF | 1 | MIMO encoder format for uplink feedback transmission Non-adaptive precoding shall be used at the AMS. 0b0: SFBC 0b1: VE with Mt = 2 if Nt > 1, or VE with Mt = 1 if Nt = 1 Nt is the number of transmit antennas at the AMS. |
| } |  |  |
| Period (p) | 3 | - Transmit feedback every 4p frame. - The first report shall start at the next frame. |
| MIMO_feedback_IE_type | 1 | - 0b0: feedback for single-BS MIMO operation - 0b1: feedback for multi-BS MIMO operation |
| If (MIMO_feedback_IE_type == 0b0){ |  | - Single-BS MIMO feedback request |
| Transmit_Correlation_Matrix | 1 | - 0b0: feedback of the transmit correlation matrix is indicated by CM - 0b1: feedback of the quantized BS transmit correlation matrix only - Transmit correlation matrix shall be feedback if CM = 0b1 |
| If (Transmit_Correlation_Matrix == 0b0){ |  | ABS requests AMS to feedback CQI And CSI for a specific MFM. |
| MaxMt | Variable 1 or 2 | Variable number of bits - depends on number of transmit antenna Nt Nt at the ABS - If Nt=2 (Any MFM): - 0b0: 1 - 0b1: 2 - If Nt =4 (Any MFM): - 0b00: 1 - 0b01: 2 - 0b10: 3 - 0b11: 4 - If Nt =8 (SU-MIMO MFM 0, 1, 2, 3, 4): - 0b00: 1 - 0b01: 2 - 0b10: 4 - 0b11: 8 - If Nt =8 (MU-MIMO MFM 5, 6, 7): - 0b00: 1 - 0b01: 2 - 0b10: 3 - 0b11: 4 |
| - MFM | - 3 | - MIMO Feedback Mode for which the AMS shall transmit feedback. |
| If (MFM = 2, 3, 5, 6) { |  | - Feedback of CQI and CSI for localized resource units |
| Num_best_subbands | 2 | 0b00: report all subbands 0b01: 1 best subband 0b10: min{6, YSB} best subbands 0b11: min{12, YSB} best subbands 1< Num_best_subbands ≤ YSB |
| } |  |  |
| If (MFM == 3,4,6,7) { |  | Feedback of CQI and CSI for CL SU and MU MIMO |
| CM | [TBD] | [TBD] |
| CS | 1 | 0b0: report PMI from the base codebook 0b1: report PMI from the codebook subset |
| } |  |  |
| If(MFM=0, 1, 2, 5){ |  | Indication of report for OL region |
| Measurement Method Indication | 1 | 0b0: Use the midamble for CQI measurements 0b1: Use pilots in OL region with MaxMt streams for CQI measurements |
| } |  |  |
| } |  |  |
| } |  |  |
| Else { |  | - Multi-BS MIMO feedback request |
| TRU | 2 | - Target RU indicating which RUs or which type of RU to work on for feedback 0b00: Latest best subbands reported for single BS MIMO 0b01: Whole bandwidth 0b10: FFR partition 0 0b11: boosted FFR partition |
| ICT | 2 | 0b00: PMI restriction for single-BS precoding; 0b01: PMI recommendation for single-BS precoding; 0b10: CL-MD for multi-BS precoding; 0b11: Co-MIMO for multi-BS precoding; |
| CS | 1 | 0b0: report PMI from the base codebook 0b1: report PMI from the codebook subset |
| N_multiBS_reports | 3 | N_multiBS_reports indicates the number of reports. |
| If (ICT = 0b11) { |  |  |
| MaxUser | 2 | Maximum number of users supported in Co-MIMO in the same resource. 0b00: 2 users 0b01: 3 users 0b10: 4 users 0b11: reserved |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| Padding | variable | Padding to reach byte boundary |
| } |  |  |

* A 16 bit CRC is generated based on the contents of the Feedback Polling A-MAP IE. The CRC is masked by the Station ID

Table 827 - Feedback Polling A-MAP IE

| Syntax | Size in bits | Notes |
| --- | --- | --- |
| Feedback_Polling_A-MAP_IE(){ |  |  |
| A-MAP IE Type | 4 | Feedback Polling A-MAP IE |
| Polling_sub_type | 1 | 0b0: uplink resource allocation or de-allocation. 0b1: feedback mode allocation or de-allocation. |
| If (Polling_sub_type == 0b0){ |  |  |
| Allocation Duration (d) | 3 | The allocation is valid for 2(d–1) superframes starting from the superframe defined by allocation relevance. If d == 0b000, the pre-scheduled feedback header transmission is released. If d == 0b111, the pre-scheduled feedback header transmission shall be valid until the ABS commands to release it. |
| If (d ==0b000){ |  | Feedback de-allocation |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Polling_deallocation_bitmap | 3 |  |
| HFA | 3 | HARQ feedback channel allocation to acknowledge the successful detection of this IE. |
| } else if (d != 0b000){ |  | Feedback allocation |
| ISizeOffset | 5 | Offset used to compute burst size index |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| MEF | 2 | MIMO encoder format for uplink feedback transmission Non-adaptive precoding shall be used at the AMS. 0b00: SFBC 0b01: Vertical encoding with Mt = 2, or Mt =1 if AMS has 1 tranmsit antenna 0b10: CSM with TNS = 2, Mt =1, SI =1 0b11: CSM with TNS = 2, Mt =1, SI =2 |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: AAI subframe (default) 0b1: 4 UL AAI subframes for FDD or all UL AAI subframes for TDD If number of DL AAI subframes, D, is less than number of UL AAI subframes, U, Long TTI Indicator = 0b1 |
| HFA | 3 | Index for HARQ Feedback Allocation |
| ACID | 4 | HARQ channel identifier |
| MFM_allocation_index | 2 | 0b00: MFM 0 with Measurement Method Indication = 0 0b01: MFM 3 with all subbands 0b10: MFM 6 with all subbands - 0b11: MFM is defined in Feedback Polling A-MAP IE with Polling_sub-type = 0b1 |
| MaxMt | 1~2 |  |
| Period | 4 | Resource is allocated at frames designated by every short and long period. The short feedback period is p frames. The long feedback period is q superframes. The first allocation shall start two frames later. The AAI subframe index is defined as in 16.2.14.2.2 and the AAI frame index is given by i+2, where i is the index of the frame where the Feedback Polling A-MAP IE is transmitted. The feedback of MIMO feedback modes in MFM_allocation_index is allocated on the short period. The feedback of the transmit correlation matrix is allocated on the long period if q > 0. Short and long period reports shall start at the first allocation. When short and long period feedback reports coincide in the same frame, both short period feedback content and long period feedback content shall be sent in the same burst. 0b0000: p = 1, q = 0 0b0001: p = 2, q = 0 0b0010: p = 4, q = 0 0b0011: p = 8, q = 0 0b0100: p = 16, q = 0 0b0101: p = 1, q = 1 0b0110: p = 2, q = 1 0b0111: p = 1, q = 2 0b1000: p = 2, q = 2 0b1001: p = 4, q = 2 0b1010: p = 1, q = 4 0b1011: p = 2, q = 4 0b1100: p = 4, q = 4 0b1101: p = 0, q = 1 0b1110: p = 0, q = 2 0b1111: p = 0, q = 4 |
| } else { |  | - Polling_sub_type == 0b1 |
| Allocation Duration (d) | 3 | The allocation is valid for 2(d–1) superframes starting from the superframe defined by allocation relevance. If d == 0b000, the pre-scheduled feedback header transmission is released. If d == 0b111, the pre-scheduled feedback header transmission shall be valid until the BS commands to release it. |
| ACK Allocation Flag | 1 |  |
| If (ACK Allocation Flag == 0b1){ |  |  |
| HFA | 6 | HARQ feedback channel allocation to acknowledge the successful detection of this IE. |
| } |  |  |
| If (d == 0b000){ |  | Feedback de-allocation |
| Polling_deallocation_bitmap | 3 |  |
| } Else { |  | - Feedback allocation |
| MIMO_feedback_IE_type | 1 | - 0b0: feedback allocation for single-BS MIMO operation - 0b1: feedback allocation for multi-BS MIMO operation |
| If (MIMO_feedback_IE_type == 0b0){ |  | - Single-BS MIMO feedback request |
| MFM_bitmap | 8 | If a currently allocated MFM is indicated in the MFM_bitmap, it indicates a deallocation and reallocation of this MFM. ACK Allocation Flag shall be set to 0b1 in this case. |
| Period | 4 | 16 combination of short feedback period p (frames) and long feedback period q (superframes). The first report shall start two frames later. The AAI subframe index is defined as in 16.2.14.2.2 and the AAI frame index is given by i+2, where i is the index of the frame where the Feedback Polling A-MAP IE is transmitted. The short period feedback shall be included in the first report. The long period feedback shall also be included in the first report if it is assigned. When short and long period feedback reports coincide in the same frame, both short period feedback content and long period feedback content shall be sent in the same burst. 0b0000: p = 1, q = 0 0b0001: p = 2, q = 0 0b0010: p = 4, q = 0 0b0011: p = 8, q = 0 0b0100: p = 16, q = 0 0b0101: p = 1, q = 1 0b0110: p = 2, q = 1 0b0111: p = 1, q = 2 0b1000: p = 2, q = 2 0b1001: p = 4, q = 2 0b1010: p = 1, q = 4 0b1011: p = 2, q = 4 0b1100: p = 4, q = 4 0b1101: p = 0, q = 1 0b1110: p = 0, q = 2 0b1111: p = 0, q = 4 |
| If (LSB #0 in MFM_bitmap == 1){ |  | - MFM 0 |
| MaxMt | 1~2 |  |
| Measurement Method Indication | 1 | 0b0: Use the midamble for CQI measurements 0b1: Use pilots in OL region with MaxMt streams for CQI measurements |
| } |  |  |
| If (LSB #1 in MFM_bitmap == 1){ |  | - MFM 1 |
| MaxMt | 2 |  |
| Measurement Method Indication | 1 | 0b0: Reserved 0b1: Use pilots in OL region with MaxMt streams for CQI measurements |
| } |  |  |
| If (LSB #2 in MFM_bitmap == 1){ |  | - MFM 2 |
| MaxMt | 1~2 |  |
| Num_best_subbands | 2 | 0b00: report all subbands 0b01: 1 best subband 0b10: min{6, YSB} best subbands 0b11: min{12, YSB} best subbands 1≤ Num_best_subbands ≤ YSB |
| Measurement Method Indication | 1 | 0b0: Use the midamble for CQI measurements 0b1: Use pilots in OL region with MaxMt streams for CQI measurements |
| } |  |  |
| If (LSB #3 in MFM_bitmap == 1){ |  | - MFM 3 |
| MaxMt | 1~2 |  |
| Num_best_subbands | 2 | 0b00: report all subbands 0b01: 1 best subband 0b10: min{6, YSB} best subbands 0b11: min{12, YSB} best subbands 1≤ Num_best_subbands ≤ YSB |
| If (q == 0){ |  |  |
| Codebook_coordination | 1 | 0b0: base mode with codebook coordination disabled 0b1: base mode with codebook coordination enabled |
| } |  |  |
| If (Nt == 4){ |  |  |
| Codebook_subset | 1 | 0b0: report PMI from the base codebook or transformed base codebook 0b1: report PMI from codebook subset or transformed codebook subset |
| } |  |  |
| } |  |  |
| If (LSB #4 in MFM_bitmap == 1){ |  | - MFM 4 |
| MaxMt | 1~2 |  |
| If (q == 0){ |  |  |
| Codebook_coordination | 1 | 0b0: base mode with codebook coordination disabled 0b1: base mode with codebook coordination enabled |
| } |  |  |
| If (Nt == 4){ |  |  |
| Codebook_subset | 1 | 0b0: report PMI from the base codebook 0b1: report PMI from the codebook subset |
| } |  |  |
| } |  |  |
| If (LSB #5 in MFM_bitmap == 1){ |  | - MFM 5 |
| MaxMt | 1~2 |  |
| Num_best_subbands | 2 | 0b00: report all subbands 0b01: 1 best subband 0b10: min{6, YSB} best subbands 0b11: min{12, YSB} best subbands 1≤ Num_best_subbands ≤ YSB |
| Measurement Method Indication | 1 | 0b0: Use the midamble for CQI measurements 0b1: Use pilots in OL region with MaxMt streams for CQI measurements |
| } |  |  |
| If (LSB #6 in MFM_bitmap == 1){ |  | - MFM 6 |
| MaxMt | 1~2 |  |
| Num_best_subbands | 2 | 0b00: report all subbands 0b01: 1 best subband 0b10: min{6, YSB} best subbands 0b11: min{12, YSB} best subbands 1≤ Num_best_subbands ≤ YSB |
| If (q == 0){ |  |  |
| Codebook_coordination | 1 | 0b0: base mode with codebook coordination disabled 0b1: base mode with codebook coordination enabled |
| } |  |  |
| If (Nt == 4){ |  |  |
| Codebook_subset | 1 | 0b0: report PMI from the base codebook or transformed base codebook 0b1: report PMI from codebook subset or transformed codebook subset |
| } |  |  |
| } |  |  |
| If (LSB #7 in MFM_bitmap == 1){ |  | - MFM 7 |
| MaxMt | 1~2 |  |
| If (q == 0){ |  |  |
| Codebook_coordination | 1 | 0b0: base mode with codebook coordination disabled 0b1: base mode with codebook coordination enabled |
| } |  |  |
| If (Nt == 4){ |  |  |
| Codebook_subset | 1 | 0b0: report PMI from the base codebook 0b1: report PMI from the codebook subset |
| } |  |  |
| } |  |  |
| } Else { |  | - Multi-BS MIMO feedback request |
| Period (p) | 3 | Transmit feedback header every 4×p frames. - The first report shall start at the next frame. |
| TRU | 2 | - Target RU indicating which RUs or which type of RU to work on for feedback 0b00: Latest best subbands reported for single BS MIMO 0b01: Whole bandwidth 0b10: FFR partition 0 0b11: boosted FFR partition |
| ICT | 2 | 0b00: PMI restriction for single-BS precoding; 0b01: PMI recommendation for single-BS precoding; 0b10: CL-MD for multi-BS precoding; 0b11: Co-MIMO for multi-BS precoding; |
| If (Nt == 4){ |  |  |
| Codebook_subset | 1 | 0b0: report PMI from the base codebook 0b1: report PMI from the base codebook subset |
| } |  |  |
| N_multiBS_reports | 3 | N_multiBS_reports indicates the number of reports. |
| If (ICT = 0b11) { |  |  |
| MaxUser | 2 | Maximum number of users supported in Co-MIMO in the same resource. 0b00: 2 users 0b01: 3 users 0b10: 4 users 0b11: reserved |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| Padding | variable | Padding to reach byte boundary |
| } |  |  |

* A 16 bit CRC is generated based on the contents of the Feedback Polling A-MAP IE. The CRC is masked by the Station ID

Parameters in the Feedback Polling A-MAP IE should be interpreted as for the Feedback Allocation A-MAP IE.

MaxMt: The possible values of MaxMt follow the rules of the Feedback Allocation A-MAP IE.

ACID: HARQ channel identifier. If q=0 or p=0, a single ACID is reserved. Otherwise, two ACIDs are reserved. The second ACID is obtained as ACID+1 modulo 15. When two feedback periods are allocated, the AMS shall send packets at the long period feedback reports with the first ACID, and the AMS shall send packets at the short period feedback reports with the second ACID.

For uplink retransmissions of MIMO feedback, the ABS shall allocate an uplink resource with the UL Basic Assignment A-MAP IE and the ACID corresponding to the packet to retransmit. Retransmissions shall obey the following rules:

- If the retransmission process for the previous HARQ burst is not finished before a new HARQ burst with the same ACID is transmitted, the retransmission process for the previous HARQ burst is terminated and the new HARQ burst overrides it.

- If the retransmission process for the previous HARQ burst reporting short-period feedback is not finished before a new HARQ burst reporting long-period feedback is transmitted, the retransmission process for the previous HARQ burst reporting short-period feedback is terminated and the new HARQ burst overrides it.

- If the retransmission process for the previous HARQ burst reporting long-period feedback is not finished before a new HARQ burst reporting short-period feedback is transmitted, the retransmission process for the previous HARQ burst reporting short-period feedback is carried on, but the short-period feedback content of the retransmitted packet shall be discarded by the ABS and the new HARQ burst reporting short-period feedback overrides it.

Polling_deallocation_bitmap: An AMS supports a maximum of 3 distinct concurrent feedback allocations, including one or several MFMs, the transmit correlation matrix, and multiBS MIMO feedback. Ordering of concurrent allocations in the Polling_deallocation_bitmap shall follow the order MFM i, MFM j (j>i), MFM k (k>j), transmit correlation matrix, multiBS MIMO feedback, as shown in Table X.

Table X - Polling_deallocation_bitmap

| Polling_deallocation_bitmap bit #2 (MSB) | Polling_deallocation_bitmap bit #1 | Polling_deallocation_bitmap bit #0 (LSB) |
| --- | --- | --- |
| MFM i | MFM j (j>i) | MFM k (k>j) |
| MFM i | MFM j (j>i) | Transmit correlation matrix |
| MFM i | MFM j (j>i) | multiBS MIMO feedback |
| MFM i | Transmit correlation matrix | multiBS MIMO feedback |
| MFM i | MFM j (j>i) | n.a. |
| MFM i | Transmit correlation matrix | n.a. |
| MFM i | multiBS MIMO feedback | n.a. |
| Transmit correlation matrix | multiBS MIMO feedback | n.a. |
| MFM i | n.a. | n.a. |
| Transmit correlation matrix | n.a. | n.a. |
| multiBS MIMO feedback | n.a. | n.a. |

Period: The setting of short and long periods p and q when Polling_sub_type == 0b0 and MIMO_feedback_IE_type == 0b0 shall obey the rules below.

For MFM 0 or 1: When p > 0, q = 0, the wideband CQI and wideband STC rate shall be transmitted using short feedback period. p ≥ 0 and q > 0 shall not be set at the same time with MFM 0 or 1.

For MFM 2 or 5: When p > 0, q = 0, the Best_subbands_index, subband CQI and Stream index shall be transmitted using short feedback period. p ≥ 0 and q > 0 shall not be set at the same time with MFM 2 or 5.

For MFM 3: when p > 0 and q = 0, the Best_subbands_index, STC_Rate, subband CQI and subband PMI from the base codebook shall be transmitted using short period. When p = 0 and q > 0, only the correlation matrix shall be transmitted using long feedback period. When p > 0 and q > 0, the Best_subbands_index, STC_Rate, subband CQI and subband PMI from the transformed codebook shall be transmitted using short period, while the correlation matrix shall be transmitted using long period.

For MFM 6: when p > 0 and q > 0, the Best_subbands_index, STC_Rate, subband CQI and subband PMI from the transformed codebook shall be transmitted using short period, while the correlation matrix shall be transmitted using long period. When p = 0 and q > 0, only the correlation matrix shall be transmitted using long feedback period. The ABS shall not set p > 0 and q = 0 simultaneously unless it has previously allocated feedback for the correlation matrix, in which case the Best_subbands_index, STC_Rate, subband CQI and subband PMI from the transformed codebook shall be transmitted using short period.

For MFM 4: when p > 0 and q = 0, the wideband CQI, STC rate and PMI from the base codebook shall be transmitted using short feedback period. When p > 0 and q > 0, the wideband CQI and STC rate shall be transmitted using short feedback period, and the correlation matrix shall be transmitted using long feedback period. When p = 0 and q > 0, only the correlation matrix shall be transmitted using long feedback period.

For MFM 7: when p > 0 and q = 0, the wideband CQI and PMI from the base codebook shall be transmitted using short feedback period. When p > 0 and q > 0, the wideband CQI shall be transmitted using short feedback period, and the correlation matrix shall be transmitted using long feedback period. When p = 0 and q > 0, only the correlation matrix shall be transmitted using long feedback period.

-------------------------------  End of Text Proposal  ---------------------------------------------------